APPROVED

ЦИВР.00530-01 34 01 ЛУ

VOICE COMMUNICATION AND CONTROL SYSTEM

«VCCS Topaz»

VCDT Operator's Manual

ЦИВР.00530-01 34 01

Total sheets 77

2017

CONTENTS

Abbreviations and Definitions	4

1 Abstract	5

2 Interface Organization	5

2.1 Function Toolbar	6

2.2 Volume control	7

2.3 Active Field Area	8

3 Telephone Communication Functions	9

3.1 Direct Access Function	9

3.1.1 Simplex Communication	9

3.2 Function «Duplex»	10

3.2.1 Communication of «Half-duplex» type	11

3.3 Function «General Cancel (CANCEL)»	12

3.4 Function «General Pick-Up»	13

3.5 Function «Connection Hold»	14

3.6 Function «Workstation Listening»	16

3.7 Function «Priority Call»	17

3.8 Function «Group (Circuit) Call»	18

3.9 «Indirect communication»	19

3.9.1 Incoming indirect calls	21

3.9.2 Call history	24

3.9.3 Phonebook	27

3.10 Function «Call Transfer»	30

3.11 Function «Conference»	32

3.12 Function of radio and telephone communication unification	34

3.13 Function «Automatic Redial»	35

3.14 Function «Calling a Group of Subscribers Before an Incoming Call is Answered»	36

3.15 Function «Call Interception»	37

3.16 Function «Handset Lifting»	38

3.17 Function «Emergency Volume»	40

3.18 Function «Quick handset»	41

3.19 Function «Quick headset»	42

3.20 Function «SPLIT»	43

4 Radio Communication Functions	44

4.1 Function «Basic Radio Set Control»	44

4.1.1 Best signal selection (BSS)	47

4.1.2 Radio channel standby mode	49

4.2 Function «Selective Call (SELCAL)»	50

4.3 Function «Retransmission»	52

4.4 Radio Parameters	54

5 Special Functions	57

5.1 Selection of Speaking Device and Volume Control	57

5.2 Acoustic Signals	63

5.3 Terminal Locking (Unlocking)	65

5.4 Short-Term Recording	66

6 Service Functions	69

6.1 Tools	69

6.1.1 Log	69

6.2 Generator	70

7 Emergency Alert System (EAS)	71

7.1 EAS purpose	71

7.2 Air traffic controllers terminal with the right to alert in an emergency (TA)	72

7.3 Reception and recording of alert signals reception acknowledgements (RF)	73

7.4 Air traffic controllers terminal without the right to generate an emergency alert signal (TNA)	76

Revision Record Sheet	77

Abbreviations and Definitions

| VCCS | Voice communication and control system |
| --- | --- |
| VCDT | Voice communication digital terminal |
| IMS | Inspection and management system |
| IME | Inspection and management equipment |
| TMCT | Technical monitoring and control terminal |
| PSTN | Public switched telephone network |
| LSC | Loudspeaker communication |
| DA | Direct access |
| SD | Speaking device |
| WS | Workstation |
| DTMF | Dual-Tone Multi-Frequency analog signal, used to dial a telephone number |
| HF | High frequency waves |
| VHF | Very high frequency waves |
| ACP | Area call panel |
| Rx | Reception |
| Tx | Transmission |
| RS | Radio set |
| BSS | Best signal selection |
| A/C | Aircraft |
| SELCAL | Selective calling system |
| STR | Short-term recording |
| PR | Permanent recording |
| PTT | Push-to-talk |
| ENS | Emergency notification system |
| TA | Air traffic controllers terminal with rights to alert to emergency situation |
| TNA | Air traffic controllers terminal without rights to generate emergency alert signal |
| RF | Reception and recording of alert signals reception acknowledgements |

1 Abstract

This User manual contains information regarding procedure for personnel operation with VCDT of system VCCS «Topaz». The document contains the description of user interface functions, which provide radio communication, telephone communication and use of special types of maintenance.

This Manual also contains short instructions, which allow familiarizing oneself and starting to operate VCDT of VCCS «Topaz» quickly. Summary instructions do not contain detailed description of all functional and system capabilities and describes only basic radio and telephony communication functions.

User Manual provides description of functional capabilities and operation procedure: description of user interface functions use.

2 Interface Organization

1

Description of procedure for operation with VCDT is given for standard VCDT configuration. The view of panels and displayed information are determined by the configuration of a given VCDT. Number, size and location of active field pages, number and location of buttons on the pages, color of buttons and inscription may be adjusted to the specific user requirements. All button inscriptions in this section are considered as an example.

Overall view of program window is shown in Figure 1.

2

3

Figure 1 – Main program window

1 – Function toolbar; 2 – Volume control; 3 – Active field area

2.1 Function Toolbar

The toolbar consists of Main and Additional functional toolbars. Toolbar appearance is set up in IME (see item 5.5 «Functions», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

The main toolbar is opened on VCDT when the latter is activated. An example of Main toolbar appearance is shown in Figure 2.

Figure 2 – Overall view of Main toolbar

An example of Additional toolbar appearance is shown in Figure 3.

Figure 3 – Overall view of Additional toolbar

– Main toolbar is active;

– Additional toolbar is active.

2.2 Volume control

Radio/LSC volume control is located on the right – hand side of the program screen, see Figure 4.

The type of communication is selected by pressing the button located above the volume control:

– communication type «RADIO» is selected;

– communication type «LSC» is selected.

The position of volume control is saved in case of change of communication type and program restart.

The range of adjustment is from - 6 dB to 3 dB with increment of 0.1. The value of selected volume level is added to the value, which was set individually to this particular button, see item 5.1 «Selection of speaking device and volume control» of this Manual.

Figure 4 – Overall Radio/LSC volume control

The volume control becomes red in dB gain area. To change the volume, touch the scale with cursor in any point of control bar. The control position will be set on the scale mark in the closest position to the touched point.

It is allowed to control volume by moving the control marker.

2.3 Active Field Area

The active field area consists of Telephone communication panel and Radio communication panel.

Telephone communication panel may consist of one or several pages with button of direct access to the subscribers. The subscriber name is indicated on each button, see Figure 5.

Figure 5 – Telephone communication panel

Radio communication panel may consist of one or several pages with radio communication buttons. The view of radio communication panel is shown in Figure 6.

Figure 6 – Radio communication panel

3 Telephone Communication Functions

3.1 Direct Access Function

Direct access subscriber call and communication with him are established by using direct access (DA) buttons. The direct access buttons are located in VCDT active field area and may be additionally grouped into pages with tabs.

The procedures for subscriber call and communication are divided into three types: simplex, duplex and half-duplex.

3.1.1 Simplex Communication

This call type automatically connects the sound channel when a direct access button is pressed. Voice is transmitted to the called subscriber as long as the button is held down. The call is displayed on any workstation equipped with direct access button, intended for this purpose.

During outgoing call in simplex mode, only microphone is switched on (to transmit voice information), while the speaker unit is off.

During incoming call in simplex mode, only speaker unit is switched on (to listen to voice information), while the microphone is off.

Thus, simplex communication sessions are one-way.

In case the subscriber call buttons are configured for operation in the simplex mode, a called subscriber hears a caller voice without any actions on his part. In this case the subscriber call buttons operate in non-latching mode.

– incoming voice call is displayed on any workstation equipped with a button configured for it, for example «DA1 SIMPLEX».

– to communicate with subscriber «DA1 SIMPLEX» press the button and, holding it down, transmit voice information.

– release the button to complete the communication session.

3.2 Function «Duplex»

Standard (default) call with two-way path through-connection for both outgoing call and incoming call processing. It is most relevant for communication with public telephone network (PSTN).

Both subscribers can carry out voice communication in both directions at the same time.

In this mode, as a rule, subscriber call buttons operate in latching mode:

– incoming/outgoing call is displayed at any workstation, provided with a button, configured for it, for example «DA2».

– to make/answer a call, press button «DA2», configured for this mode. Press «DA2» again to terminate a call/answer.

– incoming call indication. Button «DA2» and page name «LSC main» flash yellow.

– missed call indication. Icon «Handset» and button «DA2» are violet.

In case the call button is set up for non-latching operation, the process of a call establishing is similar to mode «Simplex», see item 3.1.1 «Simplex Communication» of this Manual.

3.2.1 Communication of «Half-duplex» type

It is a modification of duplex communication. After the connection is established, the subscriber direct access button is switched to mode «Reception – Transmission» – when the button in not pressed, the playback device is connected to the sound channel, the microphone is disconnected, when it is pressed – the microphone is connected and the playback device is disconnected.

Button «CANCEL» on the function toolbar is used to terminate the connection.

– the subscriber button is configured for communication type «Half-duplex».

– communication is established, button is not pressed, microphone listening mode is off.

– communication is established, button is pressed, microphone is switched on, playback device is disconnected.

3.3 Function «General Cancel (CANCEL)»

The button «CANCEL» is used to terminate any active connection (including established and requested connections) and also for dropping the incoming call without answering it.

Any unanswered incoming call is dropped automatically if an answer isn’t given within a certain period of time (the corresponding waiting time is set by the Inspection and Management System.)

Function «General Cancel» terminates all ground connections in sequence, starting from the earliest connection (i.e. the last-coming one) up to the eldest connection (i.e. the first-coming one).

– no active connections are available.

– the active ground connections are available. Single pressing on the button terminates the earliest connection.

3.4 Function «General Pick-Up»

The button «General PICK-UP» is used to pick-up any incoming call (direct or indirect one). Incoming calls are picked-up on on-going basis. By single pressing the button, the eldest call is picked up.

If there are held-on indirect calls, the button «General PICK-UP» is highlighted in green.

3.5 Function «Connection Hold»

«Connection Hold-on» function allows putting all the actual connections on hold. Also, it is possible to switch between two or more put on-hold calls by pressing the button «HOLD».

If the call is put on-hold by any other workstation, the held subscriber will receive the special indication that this mode is activated.

Do the following to put the active connection on-hold:

– green button «DA1» indicates that there is available active connection at the current workstation.

– press button «HOLD» to activate the function.

– the button «DA1» indicates on-hold state. The button «DA1» is highlighted in blue.

– press button «HOLD» or the blue button «DA1» to return to the active connection.

– check if active connection is available at your workstation. Button «DA1» is green.

Switching between the active and held-on connections. Switching between the two connections is performed by putting one call on-hold while picking up another:

– there are two established connection sat the current workstation: connection by «DA2» is active and connection by «DA1» is held-on - button «DA1» is highlighted in blue.

– press button «HOLD» for switching over.

– both connection are switched over to held status. Buttons «DA1» and «DA2» are highlighted in blue.

– press button «HOLD» or button «DA1» for switching over.

– button «DA2» remains on hold and is highlighted in blue. Button «DA1» indicates an active connection and highlighted in green.

– press button «HOLD» for switching over.

– both connections are switched over to held status. Buttons «DA1» and «DA2» are highlighted in blue.

– press button «HOLD» or button «DA2» for switching over.

– connection «DA1» is put on hold, button is highlighted in blue. Button «DA2» indicates an active connection and highlighted in green.

To terminate both of the calls on-hold, press the button «CANCEL» twice.

3.6 Function «Workstation Listening»

This function allows listening to radio and telephone communication of other VCDT. Mutual listening by two subscribers to each other is prohibited. In case the listened subscriber is set up to the simplex call mode, switch off listening function at outgoing call to prevent «self-listening to own message».

There is a security mode, which does not allow to listen to this VCDT (the parameter is set from IME).

– button of listening mode activation.

– listening activation mode is enabled.

– select the workstation to listen to.

– listening is activated. All incoming to and outgoing information from the given workstation is listened. Button «DA1» is highlighted in turquoise. Press the button of listened subscriber, which is highlighted with turquoise, and activate the connection if it is necessary.

– connection to the listened subscriber is established. Button «DA1» is green. Press the green button of established connection with the subscriber, and to return to the function of listening of this subscriber. Button «DA1» is highlighted in turquoise again.

– press button «Listen» and select the listened subscriber to complete the listening function.

– listening is completed.

3.7 Function «Priority Call»

The function is intended to establish communication with VCDT operator, engaged in other communication.

The function is used in case when it is necessary for one VCDT (intruding – I-VCDT) to send urgent information to another VCDT (intruded – ID-VCDT) while ID-VCDT is engaged in conversation (via ground or radio communication).

In order to use this function, I-VCDT shall activate the intrusion mode and indicate the button for direct access to ID-VCDT In this case the button for direct access to ID-VCDT becomes a non-latching simplex call button regardless of its actual configuration. When the direct access button is released, the intruding call is terminated by I-VCDT.

If intrusion to ID-VCD is allowed, it receives the intrusion call (answers it automatically) and switches through the current receiving speaking device to data reception from the incoming call. At that sound from I-VCDT is switched through to 
ID-VCDT only. There is no return sound channel. The subscriber, who is communicating with ID-VCDT operator at the moment, does not hear I-VCDT operator. In case ID-VCDT has a direct access button from I-VCDT, it indicates simplex voice call (with respective color). In this case a call from ID-VCDT can be terminated by pressing this button. This call from ID-VCDT can also be terminated by pressing button «CANCEL», which provides disconnection with the last established connection.

In case the mode of «locking of incoming ground communication during radio transmission» is activated in ID-VCDT, sound from I-VCDT is blocked, but call indication remains.

In case intrusion is prohibited on ID-VCDT, the intrusion call, requested by I-VCDT, will be terminated by ID-VCDT without missed call indication.

In case the intrusion function is carried out onto not engaged in conversation ID-VCDT (freed), the call is still made as an intrusion with the only difference that a receiving intercommunication device is used for alerting from direct access button on I-VCDT. If such a button is not available, default receiving speaking device is used.

– button of intrusion mode activation.

– intrusion activation mode is enabled.

– select a workstation, connection shall be established to.

– an intrusion call is in progress, the button operates in the non-latching mode.

3.8 Function «Group (Circuit) Call»

Group call is a simultaneous call to all internal and/or external parties (subscribers), grouped in the simplex mode beforehand. The group may include up to 16 subscribers. This group is created on IME. The number of groups of a group call at one workstation is not limited.

During conversation speaking devices, which are assigned to button «CIRCUIT», are used for all group subscribers (for SD assignment see item 5.1 «Selection of speaking device and volume control» of this Manual).

3.9 «Indirect communication»

The dial is used to dial indirect numbers.

– button for opening/closing the indirect communication window.

Figure 7 – Window «Indirect communication»

If the digits are pressed with established connection, they are sent to connection as DTMF codes.

– dialing field;

– redial button;

– button «clear dialing field»;

– button for deleting the last symbol in the dialing field;

– button for opening incoming indirect call window;

– button for opening a keypad;

– phonebook opening button. The list of subscribers is formed in IME;

– button for opening call history window;

– output (external gateway) for dialing;

– selection of output (external gateway) for dialing;

– subscriber call button.

3.9.1 Incoming indirect calls

Incoming calls are considered to be indirect in case a direct access button is not set up for the calling subscriber. By default all ground communication calls are also considered to be indirect.

Incoming indirect call window may be opened by pressing the respective button in the Dial or on the Additional toolbar.

– button for opening incoming indirect call window in the Dial.

– button for opening incoming indirect call window on the Additional toolbar. Second pressing of the button closes the window.

– incoming call;

– active connection;

– unanswered call.

SD «Speaker unit» and «Microphone» are used for incoming indirect calls. For SD purpose see item 5.1 «Selection of speaking device and volume control» of this Manual.

Figure 8 – Tab «Active», incoming calls

In case of incoming indirect call, information on the calling subscriber is displayed on the button. Subscriber data is taken from the phonebook if this one exists there.

The information only about the first call is displayed in the «general pick-up» line.

Figure 9 – Tab «Active», active connection

Figure 10 – Subscriber 100 during active conversation, subscriber 132 waits for an answer

Figure 11 – Subscribers 100 and 132 wait for an answer

3.9.2 Call history

The user may view the call history on this tab. The history is limited to last 100 records.

Figure 12 – Tab «History»

The history of each call is presented as follows: call type (answered, missed, declined), call time and calling subscriber address.

– answered call;

– missed call;

– declined call.

– buttons for call list navigation.

– subscriber dial button. When this button is pressed, the dial window will be opened, where the number of called subscriber from call list «History» will be indicated in the dialing field.

– subscriber call.

– called subscriber has answered.

Figure 13 – Call to subscriber 132. Number is selected from call history, 
subscriber call button is pressed

Figure 14 – Called subscriber has answered, subscriber call button is green

3.9.3 Phonebook

When the corresponding button is pressed, the phonebook window will be opened. The list of phonebook subscribers is created in IME (see item 5.6 «Phonebook», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

Figure 15 – List of phonebook subscribers

– button for deleting the last symbol in the filter field;

– button for opening window «Keyboard» to look for phonebook subscriber.

– subscriber dial button. When this button is pressed, the dial window will be opened, where the name of subscriber from the phonebook will be indicated in the dialing field.

To search for a subscriber, open window «Keyboard» by pressing button «ABC» and then enter the required symbol for search in the filter field, see Figure 16.

Figure 16 – Phonebook, window «Keyboard» is opened

Figure 17 – Call to subscriber «Technician». Number is selected from the phonebook, subscriber call button is pressed

3.10 Function «Call Transfer»

This function is required to transfer all regular incoming calls via ground communication lines from the given terminal to the address of VCDT or any other communication network subscriber supporting call transfer.

Call transfer function is activated by pressing the functional button «TRNSFR», the button «DA» (direct access) or by dialing an indirect communication number.

If transferred calls are coming to VCDT, call forwarding (repeated transfer) is possible. Back call transfer (back to the originator) is the only exception. Such calls are dropped.

– if it is necessary to transmit a call from own workstation, press button «TRNSFR» to activate the function. Transfer is deactivated by second pressing of button «TRNSFR».

– the button becomes green-blue.

– select transfer target.

– the view of the button becomes the following – transferring to «DA2».

– transferring from IME is activated.

Active calls transferring

Let’s consider the operation algorithm using the following example. Direct access buttons «DA1» and «DA3» are set up on VCDT (VCDT2) (VCDT1 and VCDT3, respectively). Let’s make a call from VCDT1 to VCDT2, but do not answer it. Thus, an incoming call from VCDT1 to be answered is displayed on VCDT2.

Now to readdress an unanswered incoming call from VCDT2 to VCDT3, use call transferring function.

– press button «TRNSFR» to activate the function. After that select the button of LSC subscriber to be transferred.

– selection of LSC subscriber, who is waiting for an answer. Button «DA1» blinks yellow.

– LSC subscriber is selected for call transfer, button «DA1» is highlighted in dotted line.

– selection of LSC subscriber, to whom the incoming call is transferred. The call is transferred to the selected subscriber, button «TRNSFR» is disabled.

– the transferred call is received by VCDT3 with indication of call transfer from VCDT2 (button «DA2»).

3.11 Function «Conference»

In conference call establishing takes part more than two (up to 16 subscribers) internal or external VCCS subscribers (DID, OREH, MARS, etc.). The number of conference call parties is set by the conference call initiator (he can add new subscribers and exclude the old ones).

Regardless of the type of called/receiving subscriber communication, communication type «duplex» will be set in the conference call mode.

Conference call initiator (the subscriber, who initiates the conference call mode) controls the conference call. He can complete operation using button «CONF» by another button pressing and holding it down for 2 second. Button «CANCEL» may be used to disconnect a subscriber, connected in the conference call mode, who was added first. Disconnection of any party may also be performed by pressing the button of active subscriber.

All other parties may leave conference call without affecting the others. All parties may put a conference call on hold at their workstations, while other parties remain connected.

– create a conference call by pressing the button «CONF» and holding it down for 2 seconds;

– button «CONF» is green, conference call is active;

– graphical icon on the button shows that it is within the conference call parties inclusion mode. The conference call parties inclusion mode switches on as soon as a conference call is activated. Subsequently it is necessary to select the subscribersfor participationin the conference call. Exit from the conference call parties inclusion mode is carried out by a short pressing of the button «CONF»;

– conference member is indicated by green color of the corresponding subscriber button and display of the graphical iconat the left part of the button.

– an invitation to tale part in a conference call looks like a standard call (yellow blinking of calling subscriber button) and availability of icon  at the left part of the button.

Conference call can be carried out with a predetermined number of parties. The set of parties is selected in IME.

Assigned to button «CONF» speaking devices are usedin the conference call mode (for SD assignment see item 5.1 «Selection of speaking device and volume control» of this Manual).

In case the conference call SD are not assigned, SD, selected in IME, will be used, see item 5.1.2 «Speaking devices», «IME Operator's Manual.ЦИВР.00531-01 34 01».

The same SD will be used in case of incoming conference call.

3.12 Function of radio and telephone communication unification

Unification of radio and telephone communication (R+T) is the mode, in which the participants of established telephone connection can listen to incoming voice communication radio signal, at which time telephone communication is not transmitted on the air.

– button for activation/deactivation of mode «R+T»;

– to activate the mode, press and hold down button «R+T»;

– mode «R+T» is activated;

– to deactivate the mode, press and hold down button «R+T»;

– short-term pressing of button «R+T» triggers a graphical icon indicating that the mode of radio buttons selection for LSC transmission is active. Press button «R+T» for a short time again to deactivate the mode;

– buttons «Radio 150.0» and «Radio 144.0» are selected for LSC transmission, they are displayed as icon .

3.13 Function «Automatic Redial»

The automatic redial is return of the call (automatic redialing) in case the called subscriber is busy or the call is unanswered in telephone communication mode.

– button for activation/deactivation of the automatic redial mode;

– automatic redial mode is activated.

– automatic redial mode is displayed on button «LSC» as a respective icon. A subscriber will be automatically recalled in a period of time, set in IME (see item 5.1.3.1 «LSC», field «Pause between recall attempts, seconds»), «IME Operator's Manual. ЦИВР.00531-01 34 01»).

3.14 Function «Calling a Group of Subscribers Before an Incoming Call is Answered»

This function provides calling to group of subscribers before an incoming call is answered in the telephone communication mode. Subscribers call group is generated in IME.

When a button for group calling is pressed, a call comes to the first workstation in the call list. Subscriber's sequence number is displayed on the button.

The call is automatically switched over to another workstation in the group in accordance with the selected sequence in case the subscriber is busy, incoming call is terminated using button «CANCEL» or upon expiry of specified period of time. Subscriber answer timeout is set in IME (see item 5.1.3.1 «LSC», field «Outgoing call answer timeout, seconds»), «IME Operator's Manual. ЦИВР.00531-01 34 01»).

The call will be returned to the first workstation, if it is not answered by the last workstation in the call list. To complete a group call, cancel it from the calling VCDT or answer an incoming call at one of workstation, included in the call group.

3.15 Function «Call Interception»

Operation in the telephone communication mode provides call interception – a function, which allows answering incoming calls, waiting for an answer at another workstation.

– button for activation/deactivation of call interception mode.

Let’s consider the function operation algorithm using the following example. Direct access buttons «DA1» and «DA2» are set up on VCDT (VCDT3) (VCDT1 and VCDT2, respectively). Let’s make a call from VCDT2 to VCDT1, but do not answer it. Thus, an incoming call from VCDT2 to be answered is displayed on VCDT1.

Now use the call interception function to answer a call of another subscriber on VCDT3.

For this purpose activate the function by pressing functional button «INTERCEPT»:

– interception mode is activated.

After that press the direct access button of the VCDT, to which the call to be answered is made, in this case, it is VCDT1 and button «DA1»:

– select a subscriber with an incoming call to be answered.

As a result, VCDT1 switches the incoming call to be answered to VCDT3, but the call   will be performed to the button «DA2», as the initial call originator is VCDT2:

– the intercepted call is displayed as an incoming one from the initial call originator VCDT2 (on button «DA2»).

Answer the call as a usual incoming call:

– the intercepted call is received with indication of call interception from VCDT1 (button «DA1»).

In case there are several incoming calls to be answered on VCDT, which is the interception target, only one call from the waiting line, starting from the oldest one, will be processed during each use of function INTERCEPTION.

3.16 Function «Handset Lifting»

In case SD «Handset» with lifting/replacing detector is available, the algorithm of duplex calls processing, including conference call mode, is changed.

Depending on call type (incoming or outgoing) and status (to be answered or already answered), the «Handset» is automatically assigned as transceiver SD for this call at the time it is «lifted» or if it was already «lifted» before a call from VCDT was made. Besides, all calls, to which the handset was assigned, will be terminated when the handset is replaced.

Function «Handset lifting» processes calls in the following order:

- Incoming calls to be answered. The call is additionally answered.

- Active (answered) incoming calls.

- Outgoing calls to be answered.

- Active outgoing calls.

In case the «Handset» is already «lifted» prior to call origination, all duplex calls are automatically redirected to the «Handset». The incoming calls are answered without air traffic controller’s assistance.

Let’s consider an example of use of SD «Handset» with «lifting» detector.

Incoming calls to be answered

When the handset is lifted (call is answered), the assigned SD are switched over to the lifted handset on the subscriber call button «DA1», which is displayed by icon «Handset». After call termination SD are switched back from the handset to the initial values.

– incoming call to be answered. The handset status is «replaced»;

– incoming call. The handset status is «lifted»;

– incoming call to be answered. The handset status is «lifted»;

– subscriber disconnection. SDs return to the initial values.

Answered incoming calls/active outgoing calls

If the connection has already been established when the handset is lifted, the assigned SDs are switched over to the lifted handset. It is displayed as icon «Handset».

– initial active call, handset status is «replaced». SDs, assigned to the button, are used.

– incoming call. The handset status is «lifted».

Outgoing calls to be answered

– outgoing call to be answered;

– outgoing call to be answered. The handset status is «lifted»;

– subscriber disconnection. SDs return to the initial values.

The call may be terminated as follows:

-	by pressing button «CANCEL»;

-	by pressing subscriber call button «DA1»;

-	by replacing the handset.

3.17 Function «Emergency Volume»

VCDT allows using the emergency volume function.

– button for activation/deactivation of emergency volume mode.

– press the button and hold it down to activate the emergency volume mode.

When button «EMERG VOL» is held down, the volume of all incoming calls (existing and new) is increased in VCDT by a selected value in dB.

By default the value is +4 dB.

It is set for each VCDT separately (see item 5.1.2.4 « Miscellaneous», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

3.18 Function «Quick handset»

The function allows quick selection of a handset for LSC. Radio communication remains working via the microphone and speaker units.

– button for activation/deactivation of the «Quick handset» mode.

 – «Quick handset» mode is enabled. The button operates in latching mode, when activated the button color changes to green. Press the button again to return to the initial state. SDs for LSC become the last set before activation of the function «Quick handset».

3.19 Function «Quick headset»

The function allows quick selection of a headset for all available communication of the current workstation.

– button for activation/deactivation of the «Quick headset» mode.

– «Quick headset» mode is enabled.

The button operates in latching mode, when activated the button color changes to green. Press the button again to return to the initial state. SDs for Radio and LSC become the last set before activation of the function «Quick headset».

3.20 Function «SPLIT»

«SPLIT» - the mode of immediate switching over of the VCDT buttons of each type (LSC, Radio, Telephone communication) to specific speaking devices.

– button for activation/deactivation of «SPLIT» mode.

– «SPLIT» mode is enabled.

By default, SPLIT mode is deactivated. The individual Speaking Devices settings are used during operation, see item 5.1 «Selecting of Speaking Device and Volume Control» of this manual.

While «SPLIT» mode activation, immediate switching over of the VCDT buttons of each type (LSC, Radio, Telephone communication) is carried out to specific speaking devices that are configured on the tab «Speaking devices» - «Speaking devices in SPLIT mode», see «VCDT Operator's Manual. ЦИВР.00531-01 34 01», item 5.1.2.1 «Speaking devices»). When «SPLIT» mode is enabled, individual setting of Speaking Devices is impossible.

While «SPLIT» function deactivation VCDT returns to the default operating mode.

When a stereo headset is connected at an air traffic controllers’ working position, a window is opened to confirm switching over of Speaking Devices buttons to the headset:

Figure 18 – «SPLIT» mode activation

If you select «OK», the «SPLIT» mode is activated. If you select «Cancel», the default operating mode remains. If the headset is not connected, the «SPLIT» mode will not be activated.

4 Radio Communication Functions

4.1 Function «Basic Radio Set Control»

The radio set is controlled using a three-button group: «Rx», «Tx» and PTT button.

Button «Rx» is intended to connect speaking devices to the radio receiver output and visual indication of reception. The button operates in «latching» mode. In the initial state the speaking devices are disconnected from the radio receiver output.

Button «Tx» is intended to connect speaking devices to the radio transmitter input. The button operates in the «latching» mode. In the initial state the speaking devices are disconnected from the radio transmitter input.

– the PTT button is intended to switch over the radio transmitter to «Radiation» mode and is used as visual indicator if radio transmitter operation mode. The button operates in «non-latching» mode. The button reacts to pressing only when button «Tx» is switched on. In the initial state the radio transmitter is set to the standby mode.

– if workstation configuration is correct the activation of PTT button result in changes button color to green. Green button color indicates that the respective radio transmitter is switched over to the «Radiation» mode. The PTT button priority is set up from IME.

– in case the radio transmitter, which is controlled by this button, is used by another VCDT, the button color changes to orange and the button stops to react to pressing. In such a case, pressing the PTT button on the touch panel, or the PTT on the desk microphone holder, or PTT of hand microphone, or PTT of microphone headset triggers sound signal «Incorrect operation» to be given.

– when a voice signal is detected in the reception/transmission channels of radio set, the respective button blinks yellow.

– only reception «Rx» mode is available for this radio set. Check box «Listening only» is selected (see item 5.3.3 «Radio button properties», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

– radio set power supply control. Check box «Radio set power supply control» is selected (see item 5.3.3. «Radio button properties», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

The button is grey – power supply is off.

– press and hold down the button to switch on radio set power supply, at that the button color changes to green.

– the button is green, radio set power supply is switched on.

– press and hold down the button to switch off radio set power supply, at that the button color changes to grey.

Higher listening priority icon

– check box «Reception priority» is selected (see item 5.3.3 «Radio button properties», «IME Operator's Manual.ЦИВР.00531-01 34 01»). A higher listening priority is assigned to one of the selected frequency channels. It is displayed on button «Rx» as a respective icon.

When the function of higher listening priority is activated, the ratio of change of non-priority RS volume may be set up (see item 5.1.2.3 «Radio communication», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

– check box «Complete reception disabling prohibited» is selected (see item 5.3.3 «Radio button properties», «IME Operator's Manual. ЦИВР.00531-01 34 01»). It is prohibited to disconnect the radio channel, used by one operator (air traffic controller) only, who is the only listener of this channel. It is displayed on button «Rx» as a red frame.

4.1.1 Best signal selection (BSS)

Function «BSS» (best signal selection) from the group of radio sets is activated when a pre-configured button is pressed. Check box «BSS function control» is selected (see item 5.3.3 «Radio button properties», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

Button «BSS» is intended for activation, deactivation and visual indication of special operation mode (mode «best signal selection») of radio set group. The button operates in «latching» mode.

In the initial state the button is shown in the released position, light grey.

When the button is pressed, its color changes to green. At the same time all buttons «Rx» of the radio sets, inside BSS group, are switched on. Green button color indicates that all group radio sets are switched on and connected to the workstation speaking devices. At that inscription «BSS» and indication of group number appears on «Rx» buttons of radio sets, added into the group.

Figure 19 – Function of best signal selection for the radio set group is activated

The «Rx» buttons of respective radio sets start to blink yellow upon the detection a carrier frequency by one or several radio sets of the group. At the same time indication (BSS:1), designating that VCDT has determined that the quality of received signal of this radio set is best and that the workstation speaking devices are connected to this very radio set, is displayed on one of buttons «Rx». Visual indication – the button is marked with red frame.

An air traffic controller can activate/deactivate the best signal selection mode at any time and work with each radio set separately.

Check box «Support of BSS group priority increase» – enables/disables BSS function by long pressing of the button. Volume level of non-priority RS is set in decibels (see item 5.1.2.3 «Radio communication», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

If the check box is not selected, BSS is enabled/disabled by long pressing of the button.

– enabling of BSS function by long pressing of the button.

– disabling of BSS function by long pressing of the button.

Priority is switch on/off by short pressing.

– visual indication of priority BSS group.

In case one mechanical PTT is assigned to several BSS groups at the same time and BSS priority is not used, PTT is enabled for all RS in all groups, see Figure 20.

Figure 20

In case the priority mode is activated in one of the groups, PTT is enabled only for RS of this group, see Figure 21.

Figure 21

4.1.2 Radio channel standby mode

The Radio button with channel standby mode is operated just as an ordinary Radio button described in 4.1. In addition, two functional buttons «M» and «S» are available to perform manual switching between two channels. These buttons also display availability status of each channel.

In case the connection with one channel is lost, the system automatically switches to the second available channel. While this operation is performed, all active connections remain active, i.e. neither «RX» nor «TX» are interrupted.

Possible states of the Radio button with channel standby mode:

|  | – initial Radio button state; |
| --- | --- |
|  | – main channel is active, standby channel can be switched to; |
|  | – standby channel is active, main channel can be switched to; |
|  | – main channel is active, standby channel is unavailable and cannot be switched to; |
|  | – standby channel is active, main channel is unavailable and cannot be switched to; |
|  | – both channels are unavailable, Radio button is completely disabled. |

4.2 Function «Selective Call (SELCAL)»

System SELCAL consists of coder SELCAL, installed on the ground, system SELCAL control unit and system SELCAL decoder, installed on the aircraft. Each decoder SELCAL, installed onboard the aircraft, has its unique code. Decoder SELCAL is connected with HF and VHF radio channels of the aircraft. Upon reception of radio signal, the device gives a sound warning about it, and an active radio channel, via which transmission is carried out, is indicated on ACP panel in the crew cockpit.

Thanks to this system the crew does not have to monitor ACP state permanently.

SELCAL function is configured for each Radio button separately from IME, i.e. it is activated/deactivated for this Radio button. Radio button with enabled (but not active) SELCAL function looks as follows:

Enable «Tx» and «Rx» buttons to activate SELCAL function, after that button SELCAL will become green:

Press button «SELCAL» for a short time to open SELCAL code input window, the respective window will open, see Figure 22.

3

4

2

1

Figure 22 – Window of SELCAL code input

1 – List of previously called subscribers (history); 2 – Entered code field;

3 – Button «Call»; 4 – Letter tone input panel

«History» – the list of previously called subscribers. When a code is selected from this list, the code is copied to the entered code field.

«Entered code field» – it is used to display a code, which will be generated.

Button «Call» – button for generation of entered code tone signal in the radio facility. Call may be activated only when the whole code is entered (4 letters), button color shall be green:

Figure 23

Call (sending of code) is made by short-term pressing of button «Call», after that SELCAL window is closed and operation with A/C can be resumed in the standard mode. Operating frequency is also displayed on the button.

When the tone signal code is not entered or is entered incompletely, button «Call» is not active and is grey, i.e. tone signal generation is impossible:

Figure 24

«Entered code field» – 16 tones from A to S are used in SELCAL panel, at that tones I, N, O are not used. Each tone is assigned with its «own frequency».

Rule of SELSAL code entry: the first letter of each pair shall precede the second one in the alphabet and letters shall not be repeated. Upon entry of the1st and the 3rd code symbols, the symbols, located on the panel above the entered one, including the last entered symbol, change their color from green to grey and can not be entered. But there are exceptions – when a grey symbol is held down for more than 5 seconds, the symbol is entered and a sound message of error signal, used on this VCDT, is issued. Upon entry of 2nd and 4th symbols, all symbols of the panel change their color to green.

According to Figure 25, the code is entered (dialed or selected from the list) and button «Call» is active.

Figure 25 – Button «Call» is active

4.3 Function «Retransmission»

Radio channels grouping with possibility to retransmit voice signal from channel to channel in the radio communication mode. Only one such group can be created per VCDT.

Function «Retransmission» has two operation modes:

–	full retransmission. Signals, transmitted by an air traffic controller and received via this radio set receiver, are retransmitted to all radio channels, included in the retransmission group;

–	partial retransmission. During transmission via this radio set, the signal is retransmitted to all radio channels, included in the retransmission group.

– retransmission mode button. The radio set selection mode is activated by short pressing of the button.

– the radio set can be selected for including into the group in the full retransmission mode. Selection is carried out by pressing a respective radio button. The button will look as follows:

– full retransmission mode is selected for radio button. «Rx» and «Tx» are not active.

When the required radio sets are selected, deactivate the selection mode by short pressing of button «FULL RETR».

– full retransmission mode is selected for radio button. «Rx» and «Tx» are active.

– switching over between full and partial retransmission modes is provided by long pressing of the button. By default, the button works in the full retransmission mode.

– the radio set can be selected for including into the group in the partial retransmission mode. Selection is carried out by pressing a respective radio button. The button will look as follows:

– partial retransmission mode is selected for radio button. «Tx» is not active.

When the required radio sets are selected, deactivate the selection mode by short pressing of button «PARTIAL RETR».

– partial retransmission mode is selected for radio button. «Tx» is active.

4.4 Radio Parameters

– button for opening/closing window «Radio parameters».

This window allows remote frequency tuning for the radio set, supporting this function, switching over between main and standby radio sets, if such function is configured on this channel, as well as toggling of squelch mode.

When button «PARAM RADIO» is pressed, the window with the same name will be opened, see Figure 26.

Figure 26 – Window of remote radio set frequency tuning

Select the frequency to be changed. Using digital buttons set the required value and press button «Set», see Figure 27.

Figure 27 – Window of remote radio set frequency tuning, frequency setting

Figure 28 – Window of remote radio set frequency tuning, frequency is set

VCDT transmits this frequency to the radio set. As a result, the value of new frequency is displayed at all workstations with access to this radio channel.

Switching over between main and standby radio sets can be carried out using buttons «Ms» (Master) and «Sb» (Standby) in window «Radio frequency» or directly on Radio button.

Availability of buttons «Ms» and «Sb» on Radio button depends on selection of check box «Ms/Sb switch». The check box is selected in IME (see item 5.3.3 «Radio button properties», «IME Operator's Manual.ЦИВР.00531-01 34 01»).

To toggle the squelch mode, press the respective button:

– button of squelch mode activation;

– squelch mode is activated;

– button of squelch mode deactivation;

– squelch mode is deactivated.

5 Special Functions

5.1 Selection of Speaking Device and Volume Control

Volume can be adjusted separately for each radio set: reception and transmission. Volume can also be adjusted directly on the speaker units.

– button of speaking devices (SD) selection.

Icons are displayed on the button in the following order:

– speaker units;

– handsets;

– headsets;

– right earpiece of stereo headset;

– microphones;

– pedals;

– line recording interfaces;

– speaking devices of unknown type.

The current status of all speaking devices, set up on the terminal, are displayed on the button with icons:

|  | «Green» – speaking device is connected and ready for operation |
| --- | --- |
|  | «Red» – speaking device is unavailable (disconnected or faulty) |
|  | «Grey» – speaking device status is unknown |

– the mode of button selection of speaking devices selection is activated.

The mode can be used for the button of the following type:

–	«LSC»;

–	«Radio»;

–	«Conference call»;

–	«Incoming indirect calls»;

–	«Sound signals»;

–	Dial opening button;

–	Short-term recording (STR).

The setting window will be opened, when one of buttons is pressed.

Figure 29 Selection of speaking device for «LSC» button «DA1»

SD with PTT (one or several) can also be assigned to the selected Radio button. When hardware PTT is used, button «PTT» will be activated from assigned SD.

Figure 30 – Selection of speaking device for «Radio» button

Figure 31 – Selection of speaking device for «Radio» button (BSS)

Figure 32 – Selection of speaking device for button «CONF» (Conference call)

Figure 33 – Selection of speaking device for button «Indirect calls»

Figure 34 – Selection of speaking device for dial opening button

Figure 35 – Selection of speaking device for button «STR»

Figure 36 – Selection of speaking device for button 
«Acoustic signals»

– respective speaking device is not connected to the terminal.

– respective speaking device is connected, access for selection.

– speaking device is selected for respective button («LSC», «Radio», «Conference», «Incoming indirect calls», dial opening button).

Speaking device volume level is set up using the respective buttons:

Button of SD volume decrease

Button of SD volume increase

Button for window «SD selection» closing

Figure 37 – Setting of SD volume level

If check mark «Volume setting is prohibited» is selected, volume increase/decrease buttons become unavailable (see item 5.3.3 « Radio button properties», of «IME operator’s Manual. ЦИВР.00531-01 34 01»»).

5.2 Acoustic Signals

– button for opening/closing the window of sound signals settings. The icon of selected SD will be displayed on the button for opening/closing the window of sound signals settings only if one SD is assigned for all events.

Button for window 
«Sound signals» closing

Button for sound

signals list navigation

Figure 38 – Acoustic signals selection

The list of acoustic signals, which will be played by VCDT at specific events, is located at the left part of window «Acoustic signals». The buttons for selection of speaking device for sound signals playback are located in the right part.

A specific volume level in decibels is set up for each event, at that the speaking device remains common for all events.

Button of sound signal volume decrease

Button of sound signal volume increase

Figure 39 – Setting of acoustic signal volume level

Sound alert can be switched off for some events.

Figure 40 – Sound alert mode is switched off

– sound alert mode is switched on;

– sound alert mode is switched off. The mode is switched on/off by pressing the button.

5.3 Terminal Locking (Unlocking)

– button of terminal locking/unlocking on the toolbar.

– locking is executed by pressing and holding down the button, meanwhile button color changes to red.

– the terminal is locked.

– to unlock the terminal by pressing and holding down the button, at that the button color changes to green – the terminal is unlocked.

5.4 Short-Term Recording

Short-term recording (STR) is test recording and listening to conversations of specific workstation. STR allows recording and playing back the communication session of workstation.

– button STR;

– STR is switched on.

The records are stored as cyclic buffer in the VCDT RAM (storage period is 30 minutes). The list of records is saved until the terminal is rebooted or configuration is refreshed.

When button «STR» is pressed, the window with the same name will be open, see Figure 41.

Figure 41 – Window STR

Each record is presented as follows:

–	type of call;

–	time of call;

–	subscriber name;

–	type of connection (simplex, duplex, half-duplex, reception, transmission).

Type of call:

|  | Outgoing |
| --- | --- |
|  | Incoming |
|  | Incoming indirect |
|  | Outgoing indirect |
|  | Reception via radio |
|  | Transmission via radio |

To switch on self-listening, select the required line and press the conversation record playback button, see Figure 42.

Button for conversation record list navigation

Figure 42 – Conversation record playback

– button «Play conversation record»;

– button «Stop conversation record;

– button for activation/deactivation of STR mode.

The mode of workstation permanent recording (PR) can be set in IME. The recording process is activated automatically when the terminal is started and cannot be deactivated by the user. By default, cyclic records storage time is 10 minutes.

Conversation records can be filtered depending on their type, see Figure 43.

Figure 43 – The list of conversations is filtered by «Outgoing» type

6 Service Functions

6.1 Tools

– button for opening/closing window «Tools».

Button «Log»– opening of protocol window, see Figure 44.

Button «Full screen» – switching over VCDT to full screen mode.

Button «Reset SD » – reset of speaking devices, assigned by the user (see item 5.1 «Selection of speaking device and volume control» of this Manual), to settings, received from IME.

Button «Screen calibration» – start of system program of screen calibration.

Button «Restart» – restart of VCDT interface.

Button «Exit» – termination of VCDT interface operation.

6.1.1 Log

Figure 44 – Window «Log»

6.2 Generator

This function provides continuous transmission of tone signal of given frequency to the channel to check speech path.

Generator activation button is arranged on the toolbar and operates just like the LISTENING function – first click the GENERATOR button, then choose the required «Radio» or «LSC» button, for which the generator shall be activated. By default, frequency 425 Hz is used in this function.

– button for activation/deactivation of generator mode.

– generator operation mode is activated.

– generator is assigned to button «DA1». Sinewave symbol will appear on the button next to SD symbol.

– connection is established, tone signal for speech path check is issued to the channel.

– generator is assigned to «Radio» button. Sinewave symbol will appear on the button next to SD symbol.

– button «Tx» is active. PTT is active, tone signal for speech path check is issued to the channel.

7 Emergency Notification System (ENS)

7.1 ENS purpose

Emergency notification system (ENS) is designed to create an emergency communications network on the basis of the current network of operative communication VCCS «Topaz» of an air traffic controllers shift in order to alert the airport's rescue team about the prerequisite to incident in a circular mode.

ENS allows:

- creating of several notification scheme;

- assigning of roles to each VCDT- transmission, reception or ignoring of alert signals and voice communication signals;

- assigning to one or several VCDT the role of the receiver of acknowledgments that the subscribers have received alert signals, including the initiator of emergency alert for the purpose of controlling;

- sending to each subscriber a special emergency alert signal, the duration of which can be changed by the settings in IME;

- transmitting voice information in a circular mode when the Emergency notification button is pressed;

- delivering audio and voice information to the line output for recording;

- displaying the reception of emergency alert signals by the blinking of indicating panel «ALARM!» on the subscriber's terminal, if technically possible;

- connection of  an external acoustic and/or indicating panel of emergency notification.

ENS setting is executed from IME. The function of the Emergency notification button is selected by means of checkmarks (see item 5.3.5 «Emergency notification button properties», «IME Operator's manual. ЦИВР.00531-01 34 01»). Depending on selected checkbox in ENS, the terminals are divided into the following types:

- air traffic controllers terminal with the right to alert in an emergency (TA). The terminal is equipped with the button «ALARM», by pressing on that the controller sends a voice message on the air;

- Reception and recording of alert signals reception acknowledgements (RF). VCDT (DCP) is equipped by this function for the subscriber responsible for the transmission monitoring of emergency alerts. RF may be equipped both TA and TNA;

- air traffic controllers terminal without the right to generate an emergency alert signal (TNA). The terminal is intended for receiving an emergency alert signal from TA. The terminal becomes a TNA, if it is selected in the notification scheme (see item 9.1 «Notification scheme setup», «IMT Operator's manual. ЦИВР.00531-01 34 01»»).

7.2 Air traffic controllers terminal with the right to alert in an emergency (TA)

Air traffic controllers terminal with the right to alert in an emergency (TA) is equipped with the «ALARM» button of red color. The button is equipped with a progress bar, which protects against accidental pressing. The inscription on the button consists of the name of the button and the name of the notification scheme, in our example it is «ALARM» and   «ALARM 1» respectively. The name of the button is indicated in IME, see item 5.3.5 «Emergency notification button properties» in the instruction manual of IME operator. ЦИВР.00531-01 34 01»». The name of notification scheme is specified in the item 9.1 «Notification scheme setup», in the instruction manual of the operator. ЦИВР.00531-01 34 01»».

The controller sends a voice message on the air, when the button is pressed and held down. In this case, the TA buffers a voice message that is equal to the duration of the audible alarm for a subsequent sequential transmission of the audio and voice messages to the network in a circular mode. Also, the TA transmits a command to other terminals over the network about the transfer of subscribers of emergency alert to the «ALARM» mode (if the subscriber has a technical capability).

– the button of activation/deactivation of emergency alert mode.

– press and hold the button to activate emergency alert mode, meanwhile the button color changes to green.

– emergency alert mode is activated. While the button is being held the recording of voice communication is in progress. Release the button to terminate the recording.

7.3 Reception and recording of alert signals reception acknowledgements (RF)

All subscribers of VCDT (DCP), responsible for controlling of emergency alert signals transmission, are equipped with the function of reception and recording of alert signals reception acknowledgements (RF).

–  the button of emergency alert signal transmission control.

– the button starts blinking yellow, if there are any changes in subscriber state during alert signal reception.

When you press the button the window with a list of subscribers will be opened, from which it is expected to receive an acknowledgment of emergency alert signal. If TNA comes to the mode «ALARM», what means that equipment confirmed the reception ALARM signal, the line with subscriber name appears red see Figure 45.

Figure 45 – The list of subscribers, from which it is expected to receive an acknowledgment of emergency alert signal. TNA is in «ALARM» mode

If a controller confirmed the reception of ALARM signal (by pressing blinking yellow display ALARM or by pressing the button CANCEL), the line with the subscriber name becomes green, see Figure 46.

The control list accommodates time of emergency alert signal reception and time of its confirmation by a subscriber (NOTIFICATION RECEIVE TIME\NOTIFICATION CONFIRM TIME). In brackets is indicated: for the moment of signal reception -the amount of time from the moment of signal formation on TA in sec. and for the moment of confirmation – how many seconds is passed from the moment of signal reception.

Black color of the line with subscriber name means that the subscriber has not received ALARM signal.

Figure 46 – The list of subscribers being confirmed emergency alert signal reception

If a subscriber does not answer the call or has not confirm receipt of a signal for a certain time, the call is automatically canceled by the TA. A line with the name of such a subscriber will be crossed out, see Figure 47. The waiting time for confirmation of a call is configured on TMCT (see item 9.1 «Notification scheme setup», parameter «Call confirmation timeout, in minutes» in the instruction manual of IME operator. ЦИВР.00531-01 34 01»).

Figure 47 – VCDT #2 subscriber confirmation timeout

View the list of subscribers by pressing the control button of emergency alert signal transmission. Lines with subscribers’ names will be of black color.

Figure 48 – The list of subscribers for emergency alert signal transmission control

VCDT\DCP and other subscribers of VCCS «Topaz», not added in emergency alert system, ignore the ALERT signal and do not switch over to the ALARM mode.

7.4 Air traffic controllers terminal without the right to generate an emergency alert signal (TNA)

Air traffic controllers terminal without the right to generate an emergency alert signal (TNA) is intended for alert signal reception from TN. In case of ALARM signal reception TNA comes to the ALARM mode - on the function panel switches on red blinking MESSAGE, see Figure 49. Alert sound signal and following voice alert from TA are played back through the loudspeaker.

Press onto the message (of the touch screen) or the button CANCEL (of the handset) for emergency alert signal reception confirmation. ALARM mode will be dropped.

Figure 49 – TNA in ALARM mode

After dropping of ALARM signal beneath the button of choosing of speaking devices appears the button of repetitive playback of emergency message, see Figure 50. You can listen to this voice message if necessary on TNA by pressing of the given button.

Figure 50 – The button of repetitive playback of emergency alert

– the button of repetitive playback of emergency message.

– press and hold down this button to close it and exit from emergency alert playback mode.

Revision Record Sheet

| Rev. | Sheet (page) numbers |  |  |  | Total sheets (pages) in document | Doc. No. | Registration number of accompanying document and date | Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | revised | replaced | new | cancelled |  |  |  |  |  |