# APPROVED

# ЦИВР.00531-01 34 01 ЛУ

# VOICE COMMUNICATION AND CONTROL SYSTEM

«VCCS Topaz»

# IME Operator's Manual

ЦИВР.00531-01 34 01

# Total sheets 185

2015

CONTENTS

Abbreviations and Definitions	5

1 Abstract	7

2 Getting Started	7

2.1 Interface Organization	9

3 System Settings	11

3.1 IMS Settings	11

3.2 SNMP Setting	12

3.3 External Gateways Setting	13

3.4 Users Setting	14

4 System Objects	17

4.1 Creating a System Object	17

4.2 System Object Types	19

4.3 Replaceable units	21

4.4 Configuring System Object	25

5 Voice Communication Digital Terminal (VCDT)	27

5.1 Main Settings	29

5.1.1 System parameters	30

5.1.2 Settings	30

5.1.2.1 Speaking devices	30

5.1.2.2 LSC	31

5.1.2.3 Radio communication	33

5.1.2.4 Miscellaneous	35

5.2 Connections	37

5.2.1 SIP endpoints	38

5.2.2 SIP routes	41

5.2.3 Radiostations	41

5.3 GUI Editor	42

5.3.1 Panel properties	46

5.3.2 LSC button properties	47

5.3.2.1 Speaking devices	52

5.3.2.2 Circular call setup	52

5.3.3 Radio button properties	53

5.3.3.1 Speaking devices	58

5.3.3.2 Linked radios	59

5.3.4 Radio button properties (Trainer)	60

5.3.4.1 Speaking devices	64

5.3.5 Emergency notification button properties	65

5.4 Voice Events	69

5.5 Functions	74

5.6 Phonebook	78

6 Line Interface Unit Analog (LIU-A)	79

6.1 Main Parameters	80

6.1.1 Controller Module status	80

6.1.2 Global parameters	82

6.1.3 Network parameters	82

6.1.4 Reservation parameters	83

6.1.5 Interfaces	83

6.1.6 LI status	84

6.1.7 Channels	85

6.2 Internal route tables	92

6.3 Signalling Configuration	94

6.3.1 SSC signalling type	94

6.3.2 DTMF signalling type	94

6.3.3 LB signalling type	95

6.3.4 FXO signalling type	96

6.3.5 FXS signalling type	97

6.4 Reservation	98

7 Line Interface Unit Digital (LIU-D)	101

7.1 Setting of Web Interface	104

7.1.1 Page Hardware info	105

7.1.1.1 Section Device information	105

7.1.1.2 Section Software information	106

7.1.1.3 Section Memory information	107

7.1.1.4 Section DSP information	107

7.1.2 Page System	108

7.1.2.1 Section Driver Configuration	108

7.1.2.2 Section DSP setup	109

7.1.2.3 Section Network configuration	110

7.1.2.4 Section Services startup	112

7.1.3 Page Configuration	113

7.1.3.1 Section Main settings	113

7.1.3.2 Section Boards configuration	113

7.1.3.3 Section SIP/PJ	113

7.1.3.4 Section H323 configuration	114

7.1.3.5 Section Trunks configuration	115

7.1.3.6 Section SS7 configuration	116

7.1.3.7 Section АК configuration	118

7.1.4 Page Modems	119

7.1.5 Page Routing	119

7.1.5.1 Section Main settings	119

7.1.5.2 Section Signalling targets	119

7.1.5.3 Section AK targets	122

7.1.5.4 Section Modem targets	123

7.1.5.5 Section Patterns	124

7.1.5.6 Section Connections	128

7.1.5.7 Section Check patterns	128

7.1.5.8 Button «Save changes»	129

7.1.6 Page Statistics	130

7.1.6.1 Section Controller	130

7.1.7 Page Syslog	133

7.1.8 Page Logout	133

7.2 Channel Registration in the System	134

8 Briefing and Training System (BTS)	137

8.1 VCDT Training Configuration	137

8.2 Configuring BTS LIU-A	144

8.2.1 VOIP peers	144

8.2.2 Patterns	147

8.2.3 Files	148

9 Emergency Notification System Setting	149

9.1 Notification scheme setup	149

10 Monitoring System	152

10.1 Monitoring	156

10.2 Setting of Sound Event Alerting	160

10.3 VCDT Statistics	161

10.4 Logins	163

10.5 Emergency notification monitoring	165

10.6 Monitoring parameters	167

11 Remote VCDT Control	169

11.1 Updating Training Configuration	170

11.2 Updating Configuration	171

11.3 Calls Transferring	172

12 Object Maintenance	174

13 DB Backup and Restore	175

13.1 Emergency recovery of a corrupted database	177

13.1.1 Transaction limit exceeding	178

14 User Change	179

15 Power Supply Control	180

Appendix A	181

Setting of Printer Epson LX-350	181

Appendix B	184

Gain Parameters	184

Revision Record Sheet	185

Abbreviations and Definitions

| AGC | Automatic gain control |
| --- | --- |
| ANI | Automatic number identifier |
| ATX | Automatic telephone exchange |
| BSS | Best signal selection |
| BTC LIU-A | Line interface unit analog for briefing and training system |
| BTS | Briefing and training system |
| CM | Controller module |
| CUPS | Common UNIX Printing System |
| DA | Direct access |
| DB | Database |
| DC | Digital communication |
| DCP | Digital communication panel |
| DTMF | Dual-Tone Multi-Frequency analog signal, used to dial telephone number |
| E&M | Analog telephone signalling standard (Ear and Mouth signaling) |
| ECT | Emergency Communication Telephone |
| ENS | Emergency notification system |
| FLASH memory | Special type of non-volatile writable solid-state memory (Flash memory) |
| FXO | Analog interface of telephone exchange subscriber device (Foreign exchange Office) |
| FXS | Voice interface or port, emulating ATX interface extension, used to connect common analog telephone to multiplexer (Foreign Exchange Station) |
| HSP | Hot Standby Protocol |
| IME | Inspection and management equipment |
| IMS | Inspection and management system |
| LB | Local battery |
| LI | Line interface |
| LIA | Analog line interface |
| LID | Digital line interface |
| LIU-A | Line interface unit analog |
| LIU-D | Line interface unit digital |
| LSC | Loudspeaker communication |
| PhL | Physical line |
| PIATX | Private industrial automatic telephone exchange |
| PR | Permanent recording |
| PTT | Push-to-talk |
| PTT | Half-duplex voice communication standard with two-way radio interface and possibility to transmit a signal in one direction at a time (Push-to-talk) |
| RAM | Random access memory |
| RF | Reception and recording of alert signals reception acknowledgements |
| RRSC | Remote radio set control |
| RS | Radio set |
| RSC | Radio set control |
| RTP | Real-time Transport Protocol |
| Rx | Reception |
| SD | Speaking device |
| SELCAL | Selective calling system |
| SIP | Communications protocol, method of initiating and terminating user Internet session, including multimedia exchange (Session Initiation Protocol) |
| SIP signalling | Sting of exchange of special data, containing SIP protocol commands and parameters, determining media data transmission in media channel |
| SIP URI | SIP addressing scheme, used to call a subscriber via SIP |
| SNMP | Internet-standard protocol for devices control in IP networks based on TCP/UDP architectures (Simple Network Management Protocol) |
| SQL | Squelch |
| STR | Short-term recording |
| TA | Air traffic controllers terminal with rights to alert to emergency situation |
| TMCT | Technical monitoring and control terminal |
| TNA | Air traffic controllers terminal without rights to generate emergency alert signal |
| Tx | Transmission |
| UPS | Uninterruptible power supply |
| URI | Uniform Resource Identifier |
| UTC | Universal time coordinated |
| VCCS | Voice communication and control system |
| VCDT | Voice communication digital terminal |
| VFCh | Voice frequency channel |
| WS | Workstation |

1 Abstract

This document contains information regarding procedure for personnel operation with IME (Inspection and Management Equipment) program of system «VCCS Topaz». The document contains description of functions, which provide control of communication terminals, main and standby communication managers (modes of transmitter), system parameters settings, as well as settings for operation in Trainer mode.

IME is intended to monitor condition of communication terminals, active network equipment (switches, gateways) and control them during operation.

Inspection and management system (IMS) equipment is a redundant terminal, based on PC and special software.

2 Getting Started

Three access levels are used to work in IME: Administrator, Engineer and Technician.

Administrator (root) adds, deletes system objects, backs up and recovers DB, adds configuration, reboots the system, looks through the event log. Administrator also has access to configuration of editor settings.

Engineer updates configuration, reboots the system, looks through the event log. Engineer also has access to configuration editor settings.

Technician looks through the event log and has access to system object status data.

Two users are created in basic configuration of IME – Technician and Administrator.

By default IME is loaded with «Technician» access level (see Figure 1).

Figure 1 - IME loading by default

To configure IME, it is necessary to log in as an Administrator (root). For this purpose, select menu item «Change user», see Figure 2.

Figure 2  Access level «Technician»

Enter «root» in field «User name» in the authorization window, there is no password. In the field «Database» drop down list to select a database to work.

Figure 3  User authorization as an Administrator (root)

# When button «OK» is pressed, IME is loaded with enhanced functional capabilities.

To ensure safety, it is recommended to set a password for user «root» in tab «Users», see item 3.4 «Users Setting» of this Manual.

2.1 Interface Organization

1

Overall view of IME program window is shown in Figure 4.

2

6

3

5

4

Figure 4  Main program window

1 – main menu bar. It contains such items as: «DB management», «Change user», «Power supply», «Main system settings», «General». The set of menu items depends on the access level of authorized user.

2 – toolbar:

– user session termination, IME is set to operation mode with access level «Technician»;

– quick access to loading configuration sets in Trainer mode. The button is available for user with «Administrator and «Engineer» access levels;

– switching over between «mouse» operation modes. If the button is pressed, the system structure is panned behind the free space between objects. If the button is not pressed, several objects are selected in the system structure area.

– locking of objects movement in the system structure area;

– deleting of selected object from the system structure area;

– system time changing;

– local time indication mode is selected;

– UTC time indication mode is selected.

– monitoring system status indicator.

3 – active field area. It contains system objects the system consists of. The context menu may be used to update object configuration, enable readdressing, put for maintenance, reboot the system, delete an object, view and change object elements set, set up object parameters. Access to mentioned functions depends on the user access level.

4 – list of system objects by type. All system objects of the system are displayed as a list in tab «System Structure».

5 – system message window. A list of system operation messages is displayed in this window.

6 – status bar. Authorized user access level and path to connected database are indicated in the bar.

3 System Settings

To start setting up IMS, select main menu item «System settings», then «General», as per Figure 5.

Figure 5  Main menu, item «General»

The following tabs will be available in opened window «System settings» of IMS:

- «TMCT settings»;

- «SNMP»;

- «External gateways»;

- «Users»;

- «Emergency Notification».

For Emergency notification system (ENS) settings see item 9 of this Manual.

3.1 IMS Settings

To ensure correct indication of local time on the toolbar, select the required value in field «UTC and local time difference» in hours.

If it is necessary to print out important events, select an appropriate check box and specify the printer, which will be used to print out the events.

Figure 6  System settings, tab «TMCT settings»

If the printer is local, the path to the local printer device must be specified in the «Printer» field, «/dev/usb/lp0» for example.

To support printing through CUPS, you need to specify the name of the CUPS printer, for example, «LX-350». To find the printer name, run the command: «lpstat -p -d».

It is recommended to set the required printer as the «default printer» with the command «lpoptions -d <printer name>».

For printer setting see Appendix A of this Manual.

3.2 SNMP Setting

To inform VCCS monitoring system on the condition of separate VCCS system components, SNMP protocol is used.

Figure 7  System settings, tab «SNMP»

Addresses of SNMP agent and event listeners are common for all system VCDT. These parameters are used in VCDT server.

«SNMP agent address» – local address of observation object is indicated as shown in Figure 7.

«SNMP receivers address list» – one or several addresses are indicated, that receive events from SNMP agents. Comma is used as a separator when several addresses are indicated.

«Syslog address» – in case duplicating of BTC services operation logging is required, its address and Syslog server port are entered in this field.

3.3 External Gateways Setting

The list of external gateways is set up in this tab. The list is used in VCDT «Indirect communication», for selection of output for number dialing see item 3.9 of «VCDT Operator's Manual. ЦИВР.00531-01 34 01».

Figure 8  System settings, tab «External gateways»

Field «Title» – the name of gateway, shown on the button’s dropdown menu.

Field «Gateway dial string» – address of output to the gateway, it is indicated in SIP URI format. Macro "%num%" during dial to gateway is changed to a selected user number.

# URI – uniform recourse identifier.

# URI = <SCHEMA>:[<ADDR>@]<HOST>[:PORT]

where:

SCHEMA is addressing scheme (SIP or TEL);

ADDR is address;

HOSTis IP address or DNS name;

PORT isIP port (5060).

# For example: 	sip:bloggs@212.123.1.213

sip:10@192.168.1.10

3.4 Users Setting

Tab «Users», available for Administrator only, is intended to form a list of users in IMS.

Figure 9  System settings, tab «Users»

# To add a new user, press button «Add».

Figure 10  Selection of user to be added

Figure 11  Tab «Users», adding of new user

# In the opened window specify new user name and password.

To change the user data, select a required line and press button «Change». Toolbar «User configuration» will be opened in the right part of the tab.

Figure 12  Tab «Users», changing of user «Engineer» parameters

Upon making the required changes, press button «Save changes».

Figure 13  Tab «Users», access level «Technician»

Each user has additional rights. When user «Technician» is selected, check box «VCDT reboot allowed» becomes available.

In case check box «VCDT reboot allowed» is active, when a system object is selected, reboot of VCDT graphic user interface, VCDT system service or controller module becomes available (see item 11«Remote VCDT control» of this Manual).

Figure 14  Additional rights of user «Technician»

4 System Objects

4.1 Creating a System Object

A system object is a piece of equipment, which requires monitoring.

Figure 15  System object creation

To create a new system object, open the context menu with right mouse button and select the required type of system object (system object types are set in DB), see Figure 15.

Upon selection of required element the following window appears:

The window of new object adding is similar for all types of system objects.

Figure 16  Adding a new object

# System object address is indicated in field «IP address».

A newly created object will appear as an icon in the System structure, see Figure 17.

Figure 17  Created object icon

4.2 System Object Types

VCDT – voice communication digital terminal – is an «operator-system» interface in a part of entry of commands to establish radio and telephone communication connections, enabling of additional functions and communication. It allows to establish voice communication connection, control the connection by enabling additional functions, carry out voice communication, provides visual and audio indication of connection status and stages of its establishing.

DCP – digital communication panel – is a version of VCDT in different equipment configuration.

TMCT – technical monitoring and control terminal – is intended to monitor condition of communication terminals, active network equipment (switches, gateways) and control them during operation.

Gateway LI multiplexer (LIU-A) – multi-purpose line interface unit analog LIU-A – is intended:

- to control radio sets, connected to it via physical lines or standard voice frequency channels, upon commands transmitted depending on the type of DC or AC signals control line, and to transmit/receive LF signal to/from the radio set;

–	 to interface VCCS with loudspeaker equipment of air traffic controller communication via 2- and 4-wire physical lines or standard voice frequency channels with DC or AC signaling, or without signaling depending on the line type and type of interacting LSC equipment, and to transmit/receive LF signal to/from communication line. It may act both as individual equipment, intended for communication of only one VCCS subscriber, provided with VCDT, and common equipment, intended for communication of several VCCS subscribers, provided with VCDT.

Gateway LI multiplexer (LIU-A BTS) - universal analog unit of BLI-A line interfaces, which is intended for use as a part of the Briefing and Training system (BTS). Unlike BLI-A it does not require the presence of real equipment such as radio sets and simulates the work with this equipment.

Gateway E1 multiplexer (LIU-D) – multi-purpose line interface unit digital. LIU-D is a special-purpose multichannel telephone communication VoIP gateway with standard telephone interfaces. It is intended to interface «VCCS Topaz», in which VoIP protocols are used for voice data transmission, with conventional telephone communication networks and telephone systems via standard telephone interfaces.

Gateway LI multiplexer (ADDPAC) – multifunction VoIP gateway with simultaneous support of both analog voice interfaces and digital ones (E1/T1).

# Ethernet switch – manageable industrial Ethernet switch with redundancy function.

UPS – uninterruptible power supply – is intended for power supply in case of short-term switching-off of the main power supply source, as well as for protection against existing interference in the mains, maintaining allowable parameters for the main source mains.

# Inverter is continuous DC power supply source, based on the rectifier modules.

SHDSL modem is a modem, providing symmetrical duplex data transmission via a pair of copper conductors. It is intended to solve a last mile problem, connect remote segments of local Ethernet network and for connection o Internet.

Monitoring server – is a server of continuous monitoring of condition of communication terminals, active network equipment (switches, gateways) and storage of collected data.

SIP-phone is a hardware or software telephone, which allows to make phone calls using VoIP technologies.

Emergency notification terminal (ENT) is a specialized terminal used in the Airport's Emergency notification System (ENS) and made in the fixed telephone format. The notification is carried out by transferring of the ENS call to the given terminal, when receiving it, an audio signal is sent to the speaker of the terminal with the simultaneous switch on of the indicating panel.

Emergency communication unit (ECU) is a centralized switch for the emergency communication system intended to provide the operation of the air traffic controller and technical personnel with radio sets and telephone communication facilities in case of complete or partial failure of the main VCCS communication system.

TP_LINK print server is a network device that allows using the connected office equipment (printer, scanner) by all users from a single local network.

4.3 Replaceable units

Each units (object) is created with a certain set of elements. Each replaceable units (system object) type has its list of possible elements (controller module, headset, speaker, handset, etc.).

Figure 18  Set of elements of «VCDT» system object

The list of elements, making up an object, may be changed. To edit the list of elements, open window «Object`s replaceable units» – open the context menu with the right mouse button and select item «Replaceable units».

Figure 19  System object menu

Window «Object`s replaceable units» consists of buttons, located in the upper part of the window, and a list of elements, see Figure 20.

– add an element to configuration;

– delete an element from configuration.

Figure 20  Object`s replaceable units: VCDT

The list of elements has the following fields: «Unit type», «Title», «Location» and «Status». Field «Unit type» is editable. By default, this field indicates system names of elements, which if necessary can be changed. Open the window with a mouse double click to edit the name of element, see Figure 21.

Figure 21  Editing element name

To add an element to configuration, press button «Add unit» and select a required name from the drop-down list, see Figure 22.

Figure 22  List of elements available for adding (VCDT)

# Peculiarities of «Audio» type elements setting

When setting up object elements of «Audio(in)», «Audio(out)», «Audio(in/out)» types, take into account that the following prefixes to the audio device names are provided to ensure correct indication of speaking devices, connected to the set-up ports:

«~audio-head~» – headset;

«~audio-hand~» – handset;

«~audio-speaker~» – speaker;

«~audio-mic~» – microphone.

«~audio-rec~CODEC» – external recording (used for system object of DCP type).

Thus, if name «~audio-head~Logitech» is assigned to an object element of «Audio(in/out)» type, it means that a headset is connected to port «Logitech».

Figure 23  Adding an element to configuration with position indication

Element numbering starts with «0» value. In case of «–1» value the system assigns a position number to the added element automatically.

When several object elements of «Audio(in)», «Audio(out)», «Audio(in/out)» types are added, start numbering with 0 up to the required number of elements –1. For example, if there are five elements, the numbering will be as follows: 0, 1, 2, 3, 4.

In case external recording is required for a system object of DCP type, start numbering of elements list from «Audio(out)~audio-rec~CODEC». For external recording two ~audio-rec~CODEC elements with appropriate numbers 0 and 1shall be available, see Figure 24.

When editing composition of system object of DCP type, take into account the following peculiarity: by default software automatic gain control (AGC) is switched off on receiving devices from sound cards (separate microphones or handsets with headsets). To switch on AGC, it is required to add a special attribute in the device name, for example: ~audio-mic~:AGC:PCH. In this case microphone AGC will be switched on, see Figure 24.

Figure 24  Object`s replaceable units: DCP

4.4 Configuring System Object

Each system object has its set of set-up parameters. Configuration editor is intended to set up system objects of VCDT and DCP type. For description of Editor operation see chapter 5 of this Manual.

Common Configuration editor is used to set up system objects of gateway E1 multiplexer, gateway LI multiplexer, Ethernet switch, inverter, UPS, modem, sip-phone type.

Let us take Ethernet switch Advantech EKI-7656С configuring as an example. Open the context menu with right mouse button and select item «Configuration».

Figure 25  Ethernet switch, context menu

The configuration editor consists of system object basic parameters and web browser, which is used for access to web interface for system object set-up, see Figure 26.

# System object basic parameters

Field «Type» – type of object in «VCCS Topaz». It is not editable.

Field «System ID» – identifier, given by the system automatically. It is not editable.

Field «Title» – object text description, given when it was created. If necessary, it may be changed.

Field «IP address» – object IP address, if necessary, it may be changed.

# Web browser

The following buttons are located in the upper part of web browser window:

– return to the previous page;

– go to the next page;

– refresh the page;

– open links in external browser.

System object

basic parameters

Web browser

Figure 26  Configuration editor, Ethernet Switch Advantech EKI-7656С

5 Voice Communication Digital Terminal (VCDT)

Configuration editor is used to set up the parameters of system object of VCDT type.

To open the Editor, open the context menu with right mouse button and select item «Configuration».

Figure 27  VCDT system object menu

Figure 28  Overall view of Configuration editor

Configuration editor consist of the following tabs:

–	«Main Settings»;

–	«Connections»;

–	«GUI Editor»;

–	«Functions»;

–	«Voice Events»;

–	«Phonebook».

Each tab has its own set of parameters and buttons, whose description is given below. Overall view of the Editor is shown in Figure 28.

- 5.1 Main Settings

The tab is used to setup System parameters, speaking devices (by default, in SPLIT  mode), LSC parameters, Radio communication parameters.

– button for saving changes.

– settings have been changed, saving is required.

Figure 29  Configuration editor, tab «Main settings»

5.1.1 System parameters

Figure 30  Tab «Main Settings», System parameters

Field «System ID» – system object identifier, given by the system automatically.

# Field «IP address» – system object IP address.

# Field «Title» – system object name, indicated in the system structure.

5.1.2 Settings

5.1.2.1 Speaking devices

Speaking devices setting by default and for operation in SPLIT mode is performed on the tab «Speaking devices». SPLIT - the mode of immediate switching over of the VCDT buttons of each type (LSC, Radio, Telephone communication) to specific speaking devices.

Figure 31  Tab «Main Settings», Speaking devices

Default devices

«Main default devices» – respective speaking devices will be used in case the devices, assigned to specific buttons, are not available.

«Conference default devices» – in case conference call SD's are not assigned, common default devices will be used in VCDT.

«Emergency Notification devices» – certain speaking devices can be assigned for emergency notification alert:

«Speaker» – a device for TNA type terminal, which will be used to playback voice messages, received from TA;

«Microphone» – a device which, will be used to record voice message on TA.

Speaking devices in SPLIT mode

Speaking devices are assigned for each type of buttons (LSC, Radio, Telephone communication) during operation in SPLIT mode.

Check box «Manual speaker devices assignment is allowed» - setting makes the function of speaking devices buttons reassigning available for VCDT operator on GUI buttons.

5.1.2.2 LSC

Figure 32  Tab «Main Settings», LSC setting

Field «Default ANI» – if ANI is not assigned to a specific button, the value of this field is used. Line format: SIP URI.

«SIP endpoint for outgoing indirect calls» – name of connection, which will be used for outgoing indirect calls (see item 5.2.1 «SIP endpoints» of this Manual).

«Outgoing call answer timeout, seconds» – upon expiry of specified time an unanswered outgoing call will be cancelled.

«Pause between failed call redial attempts, in seconds» – time, upon the expiry of which a recall will be initiated, is set (used during automatic redial).

«Maximum simultaneous calls count» – maximum number of active calls is set («1» by default). In case the specified value is exceeded, «Busy» response will be given to the incoming call. It does not affect listening, priority call and simplex modes. Calls, put on hold on this VCDT, are not considered active either.

«Maximum call duration, in seconds» – upon expiry of specified time active connections will be terminated. In case value «0» is set, active connections will never be terminated automatically.

«Unproductive call control, in seconds» – upon expiry of specified time the connection is automatically terminated in case there is no voice signal in the line. In case value «0» is set, active connections will not be terminated automatically when there is no voice signal.

Check box «`Click Accepted` signal playback when connection established» –the signal is given only if the connection is initiated by pressing a screen button.

Check box «Indicate missed incoming duplex calls» – unanswered incoming duplex calls will be displayed on the appropriate button in violet:

Check box «Indicate successful incoming simplex calls as missed calls» – completed incoming simplex calls will be displayed as missed incoming duplex calls (appropriate violet button).

«Missed call indication timeout, in seconds» – upon expiry of specified time indication of missed call on the button (violet) will be reset. In case value 0 is set, indication is reset with button «CANCEL» only. Indication is also reset by incoming call by using this button.

Check box «Handle handset`s PTT button as CANCEL button» – pressing of handset PTT will be equal to pressing screen button «CANCEL».

Check box «Circular call: use PRIORITY (INTRUSION) call if called party is busy» – when the check box is selected, conference calls will be made in mode Intrusion.

5.1.2.2.1 Automatic incoming call transfer

«Incoming call answer timeout, in seconds» – upon expiry of specified time an unanswered incoming call will be cancelled.

«Transfer address if not answered» – in case of reply absence from called subscriber, an address, to which the call will be transferred, is indicated. Line format: SIP URI.

«Transfer address if busy» – in case the subscriber is busy over LSC, or during transmission via RS, an address, to which the call will be transferred, is indicated. Line format: SIP URI. At that check box «Indicate VCDT as busy on active TX» shall be selected, see Figure 33.

5.1.2.3 Radio communication

Figure 33  Tab «Main Settings»,Radio communication setting

# Active reception – button Rx is green.

Check box «Indicate VCDT as busy on active RX» – display on other terminals, that this VCDT is engaged in radio communication (button DA on other terminals is orange).

Active transmission – buttons Tx and PTT are green.

Check box «Indicate VCDT as busy on active TX» – display on other terminals, that this VCDT is engaged in radio communication (button DA on other terminals is orange). In case the check box is selected, all incoming calls are displayed, at that sound and automatic answer function are switched off.

Check box «Block LSC`s incoming calls autoanswer on active TX» – incoming simplex calls and calls via button DA with automatic answer will be displayed as usual incoming calls (blink yellow).

Check box «BlockLSC`s incoming calls autoanswer on active RX» – incoming simplex calls and calls via button DA with automatic answer will be displayed as usual incoming calls (blink yellow).

Check box «Block RX on active TX» – switching off of speaking devices from buttons Rx of all radio sets during transmission via any RS.

Check box «Use microphone from the same device which PTT was used to activate TX» – if the check box is selected, the SD, used as a microphone for button Tx, will be changed promptly depending on the pressed mechanical PTT. In case transmission has been activated with PTT «Desk microphone», speak into the desk microphone, with PTT «Headset» – speak into the headset microphone.

At that SD, set for transmission, is ignored (see «VCDT Operator's Manual. ЦИВР.00531-01 34 01», item 5.1 «Selection of speaking device and volume control» or item 5.3.3 «Radio button properties» of this Manual).

In case the check box is not selected, the device, set in Radio button properties, is always used as a microphone regardless of which mechanical PTT was used to activate transmission.

This option is used only when several PTT's are assigned to Radio button at the same time.

Check box «`Click Accepted` signal playback when using RX, TX and PTT on-screen buttons» – the signal is given only if the connection is initiated by pressing screen buttons Rx, Tx and PTT.

«VCDT default priority» – the value of priority is selected from the drop-down list (from 0 to 15, 0 – maximum, 15 – minimum). The parameter is used when the users do not have their own set-up value of PTT priority to establish communication via Radio button (see description of field «PTT priority»).

«Default bearing address» – the terminal will send messages on frequency and status of used radio set PTT to the indicated address. The address of external bearing messages reception system is specified in the following format:

<IPaddress>:<Port>:<ID>, ID parameter (direction finding identifier) is not mandatory.

The parameter will be used only for those Radio buttons, whose own bearing address is not set up.

«Voice detection indication, in seconds» – time, during which buttons Rx and Tx will continue to display availability of voice (yellow blinking) after detector switching off.

«RX priority: sound volume muting for other radios, in dB» – the specified value will be added to the current volume level of non-priority RS at the moment voice appears in RS, whose parameter «Reception priority» is active. By default the value is equal to «–4».

«Indicate stuck PTT if active for specified time, in seconds» – if PTT is help more than the specified time, the warning sound will be played back along with the red border on the PTT button. If set to «0», this function will be turned off.

Check box «BSS group priority elevation allowed» – if the check box is selected, BSS function is enabled/disabled by long pressing of the button. Priority is switch on/off by short pressing.

If the check box is not selected, BSS is enabled/disabled by long pressing of the button.

«BSS group priority: sound volume muting for other groups, in dB» – the indicated value will be added to the current level of reception volume in non-priority BSS groups. By default the value is equal to «–4».

Parameters Session Start Threshold, Silence Threshold, Session End Threshold, PTT Release Delay, Voiced Threshold, Session Recalc Hysteresis, Activity Lost Hysteresis belong to fine BSS functions adjustment.

5.1.2.4 Miscellaneous

Figure 34  Tab «Main Settings», Miscellaneous

Check box «Listening from other VCDTs is allowed» – other terminals may listen to this VCDT.

Check box «Incoming priority (intrusion) calls are allowed» – intrusion from other terminals is authorized on this VCDT.

«Emergency Volume, dB» – when button «Emerg volume» is held down, volume of all incoming calls in VCDT is increased by the value indicated in the field. By default, the value is +4 dB.

Check box «Recorder in permanent mode» – if the check box is selected, recording process is activated automatically when the terminal is switched on and cannot be switched off by the user. By default, cyclic records storage time is 10 minutes.

«Hide secondary windows on inactivity» – additional function windows (for example, «Dial» or «Incoming calls») will be automatically closed if they are not active for a specified period of time. Time is specified in seconds. In case value «0» is specified, automatic closing is disabled.

5.1.2.4.1 External recording interfaces

External recording interfaces setting. Recording line or type of recording – RTP is selected from the drop-down list. Audio streaming is sent in the RTP protocol using G.711 A-law codec. When RTP is selected, field «RTP stream receivers» appears, see Figure 35. Address is specified in one of two formats:

# <Main IP>:<PORT>

or

<Main IP>:<PORT>,<Backup IP>:<PORT>

Figure 35  External recording interfaces (VCDT)

In case a system object of DCP type includes «Audio(out)~audio-rec~CODEC» element type (see item 4.3 «Replaceable units» of this Manual), respective interface names will be displayed in the drop-down list, see Figure 36.

Figure 36  External recording interfaces (DCP)

- 5.2 Connections

This tab is used to set up such parameters as SIP endpoints, SIP routes and availability of Radiostations.

Figure 37  Configuration editor, tab «Connections»

5.2.1 SIP endpoints

To connect to other terminals and subscribers, VCDT configuration implies one or several SIP endpoints. Today VCDT settings with several SIP connections are not used.

By default, SIP connection «Default SIP Endpoint» is created for VCDT, see Figure 38.

Figure 38  Sip endpoints «Default SIP Endpoint»

SIP endpoints has the following parameters: IP address, Port, Scheme, Media ip address, Media ports range, and limit number of simultaneous calls.

Field «IP address» – local network address of signalling point. For example, «192.168.1.32:5060». By default the following value is used: «0.0.0.0:5060».

Field «Scheme» – scheme of addressing during outgoing calls. The possible values are: sip or tel. «Sip» is used by default.

Field «Media ip address» – local address, from which RTP sessions shall be created. If the parameter in not determined, a local address, using which signalling exchange is carried out, will be used.

Field «Media ports range» – the range of ports, via which RTP protocol exchange is carried out. The default value is «8000-8999».

Field «Simultaneous call limit» – the parameter indicates the number of outgoing registrations. If the parameter is not determined, the value is taken to be 0.

The above SIP connection parameters may be edited, if necessary.

To get information on the status of other terminals, subscription mechanism is used.

For example, to get information on the status of terminal with IP address, it is necessary to add a subscription by pressing button «Add subscription».

Figure 39  Configuration editor, tab «Connections», adding a subscription

Field «Subscription address» – IP address of the terminal or name of the host, whose status information is required, is indicated.

Field «Subscription name» – the name of subscription packet, usually it is «presence».

Field «Keep-alive interval» – subscription acknowledgement time, set in seconds. The minimum value is 30 seconds, the maximum value is 3600 seconds.

Drop-down menu button

If it is necessary to add subscriptions for all VCDT's at once or for all LIU-As, press the drop-down menu button:

Figure 40  Configuration editor, tab «Connections», adding subscriptions

If it is necessary to add subscriptions for given VCDT's, select item «Add subscriptions to selected VCDTs».

Window «VCDT selector» will be opened. Using checkboxes select the required VCDT, see Figure 41.

Figure 41  Configuration editor, tab «Connections», adding subscriptions

for selected VCDT

To register a terminal SIP endpoint with a gateway, add a registration record by pressing button «Add registration».

«Registration» is a subscription, which informs the registration record that the incoming calls to the indicated address («login») shall be sent to this SIP connection. The number of registration records is unlimited.

Field «Registrar address» – IP address or SIP registrar host name is indicated.

Field «Login» – name of registrar account.

Field «Password – account password.

Field «Keep-alive internal» – registration acknowledgement time, set in seconds. The minimum value is 30 seconds (it is recommended to set 60 seconds, minimum), the maximum value is 3600 seconds.

Figure 42  Configuration editor, tab «Connections», adding registration

# Used buttons

– add endpoint.

– add registration.

– add subscription;

– add subscriptions for all system VCDTs;

– add subscriptions for selected system VCDTs;

– add subscriptions for all LIU-As.

– delete SIP endpoint;

– delete registration;

– delete subscription.

5.2.2 SIP routes

Figure 43  Configuration editor, tab «Connections Editor», SIР routes

Fields «Title» and «Dial mask» – information is taken from the main settings, tab «External gateways», see item 3.3 «External gateways setting» of this Manual.

ANI and availability are set for each type of gateway. ANI is indicated in SIP URI format. In case check box «Availability to VCDT» is selected, this route is used in the dial of edited VCDT.

Check box «Yes/No» is selected by double mouse click.

5.2.3 Radiostations

Field «Radiostation» consists of the radio set channels, which are set up in the system objects of LIU-A, BTC LIU-A, LIU-D types.

Check box «Availability to VCDT» determines if this radio set is available for radio buttons setting in the Interface editor. Availability status «Yes/No» is changed by double mouse click.

Figure 44  Configuration editor, tab «Connections Editor», Radiostations

- 5.3 GUI Editor

Tab «GUI Editor» is intended to configure VCDT operating interface (LSC buttons, Radio and Radio (Trainer) buttons).

Figure 45  Configuration editor, tab «GUI Editor»

Overall view of GUI Editor is shown in Figure 45.The following buttons are located in the upper part of the Editor window:

– open VCDT configuration;

– open previous version of current configuration;

– open other VCDT configuration.

– save changes to interface;

– save configuration under a new name.

– settings have been changed, saving required.

– add a new panel with tabs.

– align buttons by grid.

– lock movement and change of button size.

– open the previous version of the configuration;

– open the next version of the configuration.

If the interface is set up for the first time, press button «Add panel». Page with LSC button will appear, button position and size are set by default.

Figure 46  Tab «GUI Editor», initial VCDT setting

When working with VCDT operating interface configuration, the configuration can be saved under a new name. For this purpose press the drop-down menu button and select item «Save configuration with a new name», see Figure 47.

Drop-down menu button

Figure 47  Tab «GUI Editor», drop-down menu button

In window «VCDT configuration saving» indicate new configuration parameters: configuration title and description, see Figure 48.

Figure 48  Tab «GUI Editor», window of configuration saving under new name

To open VCDT interface configuration, press button «Open configuration».

Figure 49  Selecting VCDT interface configuration

If it is necessary to open the previous version of current configuration, press the drop-down menu button and select menu item «Open previous configuration version», see Figure 50.

Drop-down menu button

Figure 50  Tab «GUI Editor», drop-down menu button

# In the opened window select VCDT name from the list, see Figure 51.

Figure 51  Selecting previous version of current VCDT configuration

If it is necessary to open other VCDT configuration, press the drop-down menu button and select menu item «Open other`s VCDT configuration». In the opened window select VCDT name from the list, see Figure 52.

Figure 52  Selecting other VCDT configuration

5.3.1 Panel properties

This tab indicates name and size of the page (toolbar), accommodating LSC buttons or Radio buttons, grouped by certain criteria (name).

Toolbar

Figure 53  Configuration editor, tab «GUI Editor», page parameters setting

5.3.2 LSC button properties

# LSC button properties are set in the same-name tab, see Figure 54.

Specify the button name. Set button text font and style by pressing button «A+». If it is necessary to reset the font settings to default, select the respective check box, see Figure 54.

Figure 54  Configuration editor, «GUI Editor», LSC button properties setting

Button size and position can also be set. Button size may be set manually or selected from the list, see Figure 55.

Figure 55  Selecting LSC button size

The functions of button size and position changing, button copying and deleting are available in the context menu. When the copying function is selected, the button will be copied with all its parameters.

Figure 56  Mode of LSC button size and position change

To add a new button select the respective menu item «Add LSC button»:

Figure 57  Configuration editor, tab «GUI Editor», adding LSC button

«Channel equipment» – when the button is pressed, device selection window will be opened. The list of SIP endpoints is taken from the tab «Connections», see item 5.2.1 «SIP endpoints» of this Manual.

Figure 58  Tab «GUI Editor», button LSC properties, selecting channel equipment

Figure 59  Address of subscriber to be called and ANI

«Outgoing calls address» – the address of subscriber to be called is indicated in the field. Line format: SIP URI.

«ANI» is indicated in SIP URI format. If this field is empty, value from field «Default ANI» is taken (see item 5.1.2.2 «LSC» of this Manual).

If it is necessary to use LSC button to call a group of subscribers before the incoming call is answered, indicate subscribers addresses through «,» in «Outgoing calls address», see Figure 60.

Figure 60  Address of subscribers to be called

When LSC button is pressed, the subscribers will be called in accordance with the set order. The time interval, after which the call is automatically switched over to the next subscriber, is set in field «Outgoing call answer timeout, seconds» (see item 5.1.2.2 «LSC» of this Manual).

Figure 61  Tab «GUI Editor», LSC button properties, selecting button operation mode

Button operation mode is set using check boxes.

«Duplex» – standard (default) call with two-way path connection for both outgoing call and incoming call processing. In this mode, as a rule, subscriber call buttons operate in latching mode. In case the call button is set for non-latching operation, calling process is similar to «Simplex» mode.

«Simplex» – this mode uses voice instead of call signal to inform the called subscriber. The call is displayed on any workstation equipped with direct access DA keyboard, intended for this purpose.

During outgoing call in simplex mode, only microphone is switched on (to transmit voice information), while the speaker unit is off.

During incoming call in simplex mode, only speaker unit is switched on (to listen to voice information), while the microphone is off. Thus, simplex communication sessions are one-way.

«Halfduplex» – it is used for LSC buttons, during communication with which the speaking device is switch on only when the button is pressed.

«No fixation» – if the check box is selected, the connection remains active until the button is held down.

«Autoanswer» – if the check box is selected, when a direct incoming call is made to this button, it is answered without user assistance.

«Circular in conference mode» – if the check box and respective (subscribers) direct access buttons in tab «Circular call setup» (see below) are selected, the conference call will be made in the conference mode with predetermined number of subscribers.

«Emergency call mode» – if the check box is selected, the color of LSC button text changes to red, and in case the connection is successively established, an alert signal will be issued to the called subscriber. It works for both usual simplex, half-duplex and duplex LSC buttons and conference ones.

«Disable volume adjustment» - activating of check box in the window «Audio device selection» makes volume adjustment of speaking devices unavailable (see item 5.1 «Selection of speaking device and volume control», of «VCDT operator’s Manual.ЦИВР.00531-01 34 01).

«Parallel channel» - with the set flag, when the button with an already active call (the button is «green» or blinking «yellow») receive second incoming call, this second call will be ignored. If the flag is not set, the «second» calls are rejected with the busy code or transferred to the address specified in the «Transfer address if busy» field (see item 5.1.2.2.1 «Automatic incoming call transfer» of this Manual).

5.3.2.1 Speaking devices

# This tab is intended to select speaking devices to be used.

Figure 62  Tab «GUI Editor», selecting speaking devices for LSC button

Speaker unit and microphone are selected from the drop-down list. In case «Default device» is selected in the field, the speaking device type will be taken from tab «Main settings», «Main default devices», see item 5.1.2.1 «Speaking devices» of this Manual.

5.3.2.2 Circular call setup

Figure 63  Tab «GUI Editor», Circular call setup

Select those LSC buttons in the list, which will be used for circular call.

Circular (group call) – simultaneous call to all internal and/or external parties (subscribers), grouped in the simplex mode beforehand. The group may include up to 16 subscribers.

5.3.3 Radio button properties

Setting of Radio button is similar to LSC setting: indicate button name, text font and style, position and size, select a radio set, its frequency from the list (the list of radio sets is taken from tab «Connections», see item 5.2.3 «Radiostations» of this Manual).

When setting Radio button, width of PTT button may be set by indicating the required value in field «PTT button width». When value «–1» is set, the width is calculated automatically.

Figure 64  Configuration editor, tab «GUI Editor», Radio button properties setting

«Reserve Radiostation» - Radio button will operate in channel reservation mode if reserve radio set is selected. Special configuration of LIU-A is required for this mode to work properly.

If  Radio button has both channels configured, the two additional buttons Mand Sappear on the button.

To reset the reserve channel selection, the pull down menu item «Main Radiostation only mode» must be used.

Check box «Main/Reserve channel switch» – if the check box is cleared, the two additional M and S buttons in channel reservation mode will not be displayed, and the VCDT operator will not be able to switch channels manually.

Nevertheless, in the event of a failure of the main channel, the RX button will display the indicator of operation on the reserve channel:

Check box «Set this frequency on start» – if the check box is selected, frequency indicated in field «Frequency», is automatically set when VCDT is activated.

Check box «Operational frequency change allowed» – if the check box is selected, this RS is displayed in the radio set list in VCDT in window «Radio parameters» and is available for remote frequency tuning.

«Group ID» – identifier of buttons Radio group is specified, it is used during group operations with Radio buttons, for example, best signal selection (BSS).

«Standard» – if the check box is selected, Radio button operates in the standard mode.

«RX only» – if the check box is selected, only reception mode Rx becomes available for this radio set:

«Power control switch» – if the check box is selected, the radio set button is set to the power supply control mode:

«BSS function switch» – if the check box is selected, the best signal selection function is activated for radio sets group. Radio button will look as follows:

Figure 65  Setting Radio button properties, selecting button 
«BSS function switch» mode

«PTT button without fixation» – if the check box is selected, PTT of this radio set is active only when button PTT is held down.

Figure 66  Setting button Radio properties, selecting bearing message sending mode

Check box «SELCAL enabled» – if the check box is selected, Radio button can open toolbar «SELCAL».

Select the required value of Radio set radiation class from the drop-down list for the time of SELCAL operation, if necessary, see Figure 66. When calling in mode SELCAL, value, indicated in field «Normal mode class» is used.

Check box «Master/Stanby radio set switch» – if the check box is selected, selector switch of master and standby radio sets is displayed in Radio button.

Check box «Prohibit turning RX off» – if the check box is selected, disabling of radio channel, used by one air traffic controllers only, is prohibited. The «RX» button is displayed with a red frame:

Check box «Prohibit turning RX off (locally)» – mutually exclusive with «Prohibit turning RX off» option. When you enable the «Prohibit turning RX off (locally)» option, the «Prohibit turning RX off» option is automatically disabled and vice versa.

If the «Prohibit turning RX off (locally)» check box is set, then the «RX» button is highlighted with a red frame after the first activation and further «RX» deactivation is impossible.

Check box «RX priority» – if the check box is selected, higher listening priority is assigned to one of the frequency radio channels. A respective icon appears on the button:

Higher listening priority icon

Check box «Exclude from Recorder» – if the check box is selected, communication on this button is not included in short-term recording.

Check box «Disable volume adjustment» - activating of check box in the window «Audio device selection» makes volume adjustment of speaking devices unavailable (see item 5.1 «Selection of speaking device and volume control», of «VCDT operator’s Manual.ЦИВР.00531-01 34 01).

Check box «Exclude from Main Volume Control» – if the check box is selected, this button is not affected by general volume control.

Check box «Ignore SPLIT mode» – setting of check box means that speaking devices of the current button will not reassign during activation/deactivation of SPLIT mode (see item 5.1.2.1 «Speaking devices» of this Manual).

«Send radio bearing» – if the check box is selected, the terminal sends messages on used radio set frequency and PTT status to the indicated address (see Figure 66). The address of external bearing messages reception system is specified in the following format:

<IPaddress>:<Port>:<ID>, ID parameter (direction finding identifier) is not mandatory.

5.3.3.1 Speaking devices

This tab is intended to select speaking devices to be used. Speaker unit and microphone are selected from the drop-down list. In case «Default device» is indicated in the field, the speaking device type will be taken from tab «Main settings», «Main default devices», see item 5.1.2.1 «Speaking devices» of this Manual.

Figure 67  Tab «GUI Editor», selecting speaking devices for Radio button

The indicated type of speaking device is displayed on the button as a respective icon (pedal), see Figure 67. Several selected speaking devices are displayed as icon "" (handset, pedal and microphone), see Figure 68.

Figure 68  Tab «GUI Editor», selecting several speaking devices for Radio button

Field «PTT priority» – priority access to the shared radio channels. The priority value is selected from the drop-down list: from 0 to 15, 0 is maximum value, 15 is minimum one. If «VCDT default priority» is indicated in the field, the parameter is taken from tab «Radio communication», see item 5.1.2.3 of this Manual.

At that provided is the procedure for conflict resolution in case of PTT signal activation simultaneously by two or more operators with the same priority rights.

5.3.3.2 Linked radios

Figure 69  Tab «GUI Editor», Linked radios

When any of buttons Rx or Tx of selected radio set is activated, all buttons Rx and Tx of the interfaced radio sets are deactivated automatically.

5.3.4 Radio button properties (Trainer)

Trainer Radio buttons emulates all functions of Radio button, working using the same protocol as LSC buttons. Buttons Rx and Tx are locked when pressed and have indication during sound transmission or reception (VAD). Button PTT is pressed without latching provided that button Тx is already pressed, communication is carried out in the simplex mode.

To add a button, select item «Add Radiostation button (Trainer)» from the drop-down menu, see Figure 70.

Figure 70  Adding Trainer Radio button

Trainer Radio button with default parameters will appear on the page, see Figure 71.

Figure 71  Trainer Radio button, default parameters

Trainer Radio button setting: indicate button name, text font and style, position and size, select a radio set from the list (in this case, SIP connections), set its frequency manually.

When setting the button, width of PTT button may be set by indicating the required value in field «PTT button width». When value «–1» is set, the width is calculated automatically.

Figure 72  Example of Trainer Radio button properties

«Outgoing calls address» – like LSC buttons, the address of subscriber to be called is indicated in the field. Line format: SIP URI.

«ANI» is indicated in SIP URI format. If this field is empty, value from field «Default ANI», tab «LSC» is taken (see item 5.1.2.2 «LSC» of this Manual).

Check box «Exclude from Recorder» – if the check box is selected, communication on this button is not included in short-term recording.

Check box «Disable volume adjustment» - activating of check box in the window «Audio device selection» makes volume adjustment of speaking devices unavailable (see item 5.1 «Selection of speaking device and volume control», of «VCDT operator’s Manual.ЦИВР.00531-01 34 01).

Check box «Exclude from Main Volume Control» – if the check box is selected, this button is not affected by general volume control.

Check box «Ignore SPLIT mode» – setting of check box means that speaking devices of the current button will not reassign during activation/deactivation of SPLIT mode (see item 5.1.2.1 «Speaking devices» of this Manual).

«Send radio bearing» – if the check box is selected, the terminal sends messages on used radio set frequency and PTT status to the indicated address (see Figure 72). The address of external bearing messages reception system is specified in the following format:

<IP address>: <Port>:<ID>, ID parameter (direction finding identifier) is not mandatory.

If the address is specified in full format <IPaddress>:<Port>:<ID>, it completely replaces the address, indicated in the settings of Trainer Radio button, in the autonomous operation mode in VCDT equivalence table (see item 8.1 «VCDT Training Configuration» of this Manual).

If the address is specified in format <IPaddress>:<Port>, the button address is analysed to calculate its ID. Thus, the address in Trainer Radio button can be specified using two variants only – <IPaddress>:<Port>:<ID>or <ID>.

If field «Send radio bearing» is empty, the value of field «Default bearing address» (see item 5.1.2.3 «Radio communication» of this Manual) is used in the standard VCDT operation mode and in integrated Complex mode.

Figure 73  Trainer Radio button, Speaking devices

5.3.4.1 Speaking devices

This tab is intended to select speaking devices to be used. Speaker unit and microphone are selected from the drop-down list. In case «Default device» is indicated in the field, the speaking device type will be taken from tab «Main settings», «Default devices» (see item 5.1.2.1 «Speaking devices» of this Manual).

The indicated type of speaking device is displayed on the button as a respective icon, see Figure 73. Several selected speaking devices are displayed as icon «» (handset, pedal and microphone), see Figure 74.

Figure 74  Selecting several speaking devices for Trainer Radio button

Field «PTT priority» – priority access to the shared radio channels. The priority value is selected from the drop-down list: from 0 to 15, 0 is maximum value, 15 is minimum one. If «VCDT default priority» is indicated in the field, the parameter is taken from tab «Radio» (see item 5.1.2.3 «Radio communication» of this Manual).

At that provided is the procedure for conflict resolution in case of PTT signal activation simultaneously by two or more operators with the same priorities.

5.3.5 Emergency notification button properties

To create emergency notification button, use «Add Alarm button» menu item, see Figure 75.

Figure 75  Creating emergency notification button

New emergency button will be created on the current panel («LSC» page in our example) with the default parameters, see Figure 76.

Figure 76  Emergency notification button with the default parameters

Emergency notification mode

The functionality of LSC button is specified by means of check marks selected on ENS, see Figure 77. In TA mode, button is used to record voice message and start emergency notification procedure using the one of the preconfigured notification schemes, see item 7.2 «Air traffic controllers terminal with the right to alert in an emergency (TA)», «VCDT Operator's Manual. ЦИВР.00531-01 34 01».

In RF mode, button is used as an emergency notification events collector. It collects all of the notification data from subscribers (the moment when a subscriber received emergency signal, the moment when he starts listening to the voice message and when he has finished listening) and presents it in a popup window, see item 7.3 «Reception and recording of alert signals reception acknowledgements (RF)», «VCDT Operator's Manual. ЦИВР.00531-01 34 01».

Figure 77  Emergency notification mode selection

«Notification scheme» – Notification scheme from the «Emergency Notification» system settings, see 9.1 «Notification scheme setup».

In FP mode, an additional address filter becomes available, see Figure 78.

By default, this list contains all of the subscribers from the selected notification scheme. The corresponding checkbox must be cleared to exclude a subscriber from notification control.

Figure 78  RF notification mode address list filter

5.4 Voice Events

This tab is used to set up voice events configuration for VCDT. A sound file is assigned for each event.

Voice events are assigned for newly created VCDT by default. The overall view of the tab is shown in Figure 79.

The left part accommodates the list of voice events names, the right part – respective sound file. For files, assigned by default, only file description is displayed, file name is not displayed.

Description of file for voice events, assigned by default

Name of voice event configuration

List of voice event names

Figure 79  Configuration editor, tab «Voice Events»

– listen to voice event;

– stop listening;

– open voice file selection window.

During operation the initial configuration of sound events may be changed. One or several sound files from the list may be replaced.

For this purpose proceed as follows:

– open window «Voice file selector» by pressing button ;

– upload a new file to the system by pressing button  (see Figure 80) and selecting a new voice file name (see Figure 81). If it is necessary to provide description of the selected voice file, by default it will contain file name, see Figure 82;

– confirm file selection by pressing «OK»;

– new uploaded voice file will appear in the list, see Figure 83.

New file uploading

Figure 80  Tab «Voice Events», uploading a new sound file

Figure 81  Tab «Voice Events», selecting a new sound file

Figure 82  Field for file description

By default file

description contains

file name

File name

Figure 83  Tab «Voice Events», new sound file is uploaded

Button «Save changes» allows to save changes under the existing voice configuration name or under a new name by pressing the drop-down menu button and selecting item «Save configuration with a new name».

– save changes of voice configuration;

– save voice configuration under a new name.

– settings have been changed, saving is required.

Drop-down menu button

– open sound events configuration.

When button «Open configuration» is pressed, the window with the list of existing saved VCDT sound configurations is opened, see Figure 84. Confirm the selection by pressing «OK» button.

Figure 84  Selecting voice configuration

5.5 Functions

- This tab is used to set up Main and Additional VCDT Functional Toolbars. The main toolbar is opened on VCDT when it is activated.

The left part of the tab accommodates the list of available functions, the right part – configurable VCDT functional toolbars.

Availability of functions depends on the check boxes selection. Functions, selected in the list, are displayed on VCDT toolbar (main and additional). Functions, highlighted in grey color, are permanently displayed in VCDT.

Figure 85  Configuration editor, «Functions»

The number of functions of each toolbar is limited by 8 buttons. The toolbars are set up by dragging functional buttons to empty segments.

– termination of each current connection (existing or being established) as well as termination of unanswered incoming call.

– answering to any incoming call (direct or indirect).

– putting all current connections on hold.

– automatic redial in case the called subscriber is busy or the call is unanswered in telephone communication mode.

– listening to radio and telephone communication of other VCDT.

– establishment of communication with VCDT operator, engaged in other communication.

– answering to incoming calls to be answered on other workstation.

– redirection of all regular ground communication incoming calls from the terminal to other address: VCDT or other network subscriber, supporting readdressing.

– conference communication.

– radio channels grouping with possibility to retransmit voice signal from channel to channel in the radio communication mode.

– combining of radio and telephone communication (R+T). In this mode the participants of established telephone connection can listen to incoming voice communication radio signal, at that telephone communication is not transmitted on the air.

– when the button is held down, volume of all incoming calls is increased to the indicated dB value. By default, the value is +4 dB. It is set for each VCDT separately, see item 5.1.3.3 «Miscellaneous», field «Emergency Volume».

– remote frequency tuning for the radio set, supporting this function, switching over between main and standby radio set channels, activation of squelch mode.

– operative RS squelch mode switching.

– SPLIT. Immediate switching over of the VCDT buttons of each type (LSC, Radio, Telephone communication) to specific speaking devices.

– «Quick handset». The function allows quick selection of a handset for LSC. Radio communication remains working via the microphone and speaker units.

– «Quick headset». The function allows quick selection of a headset for all available communication of the current workstation.

– operative «Incoming call» signal mute.

– indirect communication.

– frequency generator. This function provides continuous transmission of tone signal of given frequency to the channel to check speech path.

– short-term recording (STR). Test record and listening to conversations of separate workstation.

– audio signals setting.

– terminal locking.

– tools. When this functional button is selected, the window with the following buttons is opened in VCDT:

Button «Log» – opening of protocol window.

Button «Full screen» – switching over VCDT to full screen mode.

Button «Reset SD» – reset of speaking devices, assigned by the user, to settings, received from IME.

Button «Screen calibration» – launching of system program of screen calibration.

Button «Restart» – restart of VCDT interface.

Button «Exit» – termination of VCDT interface operation.

Buttons availability depends on the check boxes selected.

5.6 Phonebook

This tab is intended to set up the phonebook, whose list of numbers is used in VCDT, see item 3.9 «Indirect communication», «VCDT Operator's Manual. ЦИВР.00531-01 34 01».

Each note contains sequence number, indicated name, dial line, remark to the record. The sequence number is given automatically by the system, but other fields are editable. Field «Description» is limited to 255 symbols.

Dial line format: SIP URI, or in case a shot number is indicated, dial shall be carried out using sip route, selected in the Dial (see item 5.2.2 «SIP routes» of this Manual).

Figure 86  Configuration editor, «Phonebook»

To add a new note to the phonebook, press a button with required name, see Figure 87. In the opened line specify the required information.

To select several notes to be deleted from the phonebook, hold down key «Ctrl» and select the required lines with left mouse button, after that press the delete button.

Figure 87  Configuration editor, «Phonebook», adding a new note

– add a new note to the phonebook.

– delete the selected notes from the phonebook.

6 Line Interface Unit Analog (LIU-A)

This item contains description of parameters, required to set up Line interface unit analog (LIU-A).

The upper part of the window accommodates the following fields and check boxes:

field «System ID» – system object identifier, given by the system automatically. The field is not editable;

field «Title» – system object name, indicated in the system structure. If necessary, it may be changed;

field «IP address» – system object IP address. If necessary, it may be changed;

check box «Current» – check box selection allows to view current LIU-A configuration;

check box «New» – check box selection allows to edit current LIU-A configuration. If necessary to save new configuration, press button «Apply configuration».

– check of availability of LIU-A at the indicated IP address and configuration query.

Figure 88  LIU-A parameters

- save new configuration LIU-A.

See description of operation with tabs «Main parameters», «Internal route tables», «Signalling configuration» and «Reservation» below.

6.1 Main Parameters

LIU-A parameters are displayed in this tab as a tree structure.

Figure 89  LIU-A parameters, tab «Main parameters»

6.1.1 Controller Module status

When branch «Controller Module status» is opened, the current status is displayed. Non-editable fields are given for information only.

Operational status:

0XX – ready and operational (green);

000 – OK (code 000);

1XX – ready but has limitations (yellow);

100 – working on single power source (code100);

2XX – ready but almost reached limits (orange);

3XX – unserviceable, but monitoring is available (red);

300 – does not work (code 300);

4XX – unserviceable and monitoring is unavailable (dark red).

Configuration version – the configuration version number is displayed.

Software build – the version and time of the software build are displayed.

FPGA software version – the version of the FPGA software build is displayed.

Bootloader version – the bootloader version is displayed.

Controller Module ID – if the values of identifiers of all connected LI are identical, their value is set in this identifier, otherwise the value is –1.

Operating mode – the type of operation mode is displayed, set in the «Global parameters», see item 6.1.2 of this manual.

Controller Module number – if the redundancy mode is set up in the gateway and two CM's are installed, this parameter determines CM number in the gateway. It may be equal to 0 or 1. CM numbering is similar to LI.

When branch «Ethernet status» is opened, the list of available interfaces and their status will be displayed:

1 – up;

2 – down;

3 – testing;

4 – unknown;

5 – dormant;

6 – not present;

7 – lower layer down.

6.1.2 Global parameters

Operating mode:0 – standalone;

1 – slave;

2 – master.

Configuration mode:

0 – linked to LIU-A ID;

1 – static configuration.

Configuration sync:

0 – off;

1 – on.

6.1.3 Network parameters

Network address – IP address of LIU-A.

Network address mask – subnet mask.

Network address external gateway – default IP address of router.

Broadcast address – address for broadcast packet transmission in the network.

Bonding mode:

-1 – disable bonding;

0  – round robin balance;

1  – active-backup mode;

2  – XOR policy balance;

3  – broadcast policy;

4  – 802.3ad dynamic link aggregation;

5  – adaptive transmit load balancing;

6  – adaptive load balancing.

Syslog receiver – IP address of server syslog.

SNMP trap receiver – IP address of SNMP agent, to which TRAP messages will be sent to UDP filter:

use module to filter redundant UDP traffic:

0 – off;

1 – on.

UDP filter: network node cache size – maximum size of peer history.

UDP filter: IP addresses cache size – maximum size of IP packet identifiers history.

UDP filter: IP packets TTL – «time to live» of IP packet in history, set in ms.

UDP filter: network node TTL – «time to live» of peer in history, set in ms.

NTP servers – list of IP addresses for NTP servers. Mark «;» is used as separator.

Configuration sync IP address white list – indicated is the list of IP addresses, mark «;» is used as separator. It is allowed to indicate a range of addresses, the first and the last addresses are separated with «-».

For example: 192.168.12.1;192.168.12.101-192.168.12.160. LIU-A will synchronize configuration with this server list. The list is used if parameter «Configuration sync» is activated. If the list is empty or does not contain any correct element, all available networks will be used as synchronization servers.

Сonfiguration sync address black list – indicated is the list of IP addresses is indicated, mark «;» is used as separator. It is allowed to indicate a range of addresses, the first and the last addresses are separated with «-». For example: 192.168.12.20-192.168.12.30.

All addresses from this list will be ignored during configuration synchronization. The list is used if parameter «Configuration synchronization» is activated.

TOPAZ servers – the list of TOPAZ IP addresses, bearing messages will be sent to. Mark «;» is used as separator.

6.1.4 Reservation parameters

Transport – list of address information of the transport protocol for the reservation protocol. The entries in the list are separated by a «;» (semicolon). Acceptable recording formats are the following:

- UDP:IP_MULTICAST:PORT, where UDP is used as transport protocol, IP_MULTICAST – address of the multicast group, PORT – port for reception and transmission;

- CAN:ID, where CANopen is used as network protocol. Transport protocol has not chosen yet.

6.1.5 Interfaces

0 – sequence number of interface in LIU-A. It assumes a value from 0 to <number of interfaces>–1.

Line interface type:

0 – interface is disabled;

1 – Radio;

2 – LSC;

3 – Digital communication.

Line interface pattern – binary pattern, characterizes interface board operation. The pattern file is selected from the respective window, which is opened by double click, see Figure 90.

Figure 90  Pattern file selection window

6.1.6 LI status

When branch «Line Interface status» is opened, the current LI status is displayed. Non-editable fields are given for information only.

Interface index number. It assumes a value from 0 to <number of interfaces>-1.

Line Interface ID. The gateway receives the identifier value from the line interface while its connection. The range of assumed values is from -1 to 255. In case of error or absence of LI, value -1 is selected.

Digital software version – software version in major.minor format.

Analog software version – software version in major.minor format.

Digital channels capacity – number of channels supporting LID.

Analog channels capacity – number of channels supporting LIA.

Interface status:

0XX – ready and operating (green);

000 – OK (code 000);

1XX – ready, limited functionality (yellow);

104 – ANALOG_INTERFACE_HARDWARE_ERROR. The ALI_HW_STATE package with the hardware error information will be received from the LIU-A;

105 – DIGITAL_INTERFACE_HARDWARE_ERROR. The

DLI_HW_STATE package with the hardware error information will be

received from the LIU-D;

2XX – ready but almost reached limits (orange);

3XX – unserviceable, but monitoring available (red);

300 –not working (code 300);

4XX – unserviceable and monitoring is unavailable (dark red);

400 – connection lost (code 400).

Boot loader version – in major.minor format.

FCPGA software version – in major.minor format.

# CPU load – indicated in percent.

Power channel 1 fuse alarm:

0 – normal;

1 – alarm.

Power channel 2 fuse alarm:

0 – normal;

1 – alarm.

Channel 1 node 1 {PTT} alarm:

0 – normal;

1 – alarm.

Channel 2 node 1 {LSC1} alarm:

0 – normal;

1 – alarm.

Channel 1 node 2 {pattern} alarm:

0 – normal;

1 – alarm.

Channel 2 node 2 {LSC2} alarm:

0 – normal;

1 – alarm.

6.1.7 Channels

Sequence number of channel in the interface number X. It assumes a value from 0 to 7.

Answer mode for incoming call from network – the parameter is used for SIP signalling:

0 – ON_CONNECTION. Answer all incoming connections at once;

1 – ON_CALLING. Answer only if LI is operating and after sending the call to the line;

2 – ON_CALL. Answer if LI is operating and upon reception of acknowledgement that the call is answered in the line.

Answer mode for outgoing call to network – the parameter is used for SIP signalling. Describes when 'CALL' is sent to a line.

0 – ON_CONNECTION. Immediately after signalling session

setup;

1 – ON_CALLING. Immediately after remote party indicates 'ringing';

2 – ON_CALL. Immediately after media session setup.

Channel activity – the parameter value determines channel activity in the interface with number X, channel is active or interface is off:

0 – channel disabled;

1 – channel is active.

Common: channel type:

- – duplex;

- – simplex;

- – half-duplex.

Common: hand if missing voice frames on receiver – parameter of SIP signalling:

0 – function disabled;

>0 – time in ms, if during the selected time there is no voice frames (packets) reception, CM terminates the call.

Common: logic type – LSC type, which corresponds to this channel of interface number X:

0 – no defined;

1 – 2-wire line. It is usual duplex channel with one reception line and one transmission line.

2 – 4-wire line. It is duplex channel, which consists of two 20-wire channels, but only reception is used in the minor channel, and only transmission in the major one.

Common: receiver delay – artificial delay during reception from the line, it is indicated in milliseconds, 1024, max.

Common: recorder mixer:

0 – off;

1 – on.

Common: signalling detector signal level – signal level for signalling detector in dB. It may assume a value within the range of –300...0. The parameter is related to SELCAL, DTMF signalling for Radio and LSC SSC.

Common: signalling level to voice level ratio – signal level for signalling detector in dB. It may assume a value within the range of –300...0. The parameter is related to SELCAL, DTMF signalling for Radio and LSC SSC.

Common: transmitter delay – artificial delay during transmission to the line, it is indicated in milliseconds, 1024, max.

Digital Comm: signalling type:

0 – off;

# 1 – ATS QSIG.

Gain: Automatic Gain Control for transmitter – the parameter value determines operation of automatic gain control for transmission to the line:

0 – off;

# 1 – on.

Gain: Automatic Gain Control for receiver – the parameter value determines operation of automatic gain control for reception from the line:

0 – off;

# 1 – on.

Gain: adjusting resistor RC – the parameter determines gain of preset resistor RC on this sub-channel. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

Gain: adjusting resistor RR – the parameter determines gain of preset resistor RR on this channel. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

Gain: reception – the parameter determines gain during reception on this channel. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

Gain: recorder – the parameter determines gain of recording output on this channel. Gain is used for mixed reception and transmission data. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

Gain: recorder additional – the parameter determines gain of recording output of reception only on this channel. Gain is used for auxiliary recording output, to which reception data are sent. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

Gain: self-monitoring – the parameter determines gain during self-listening on this channel. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used. Self-listening is relevant for 4-wire line only (Radio and 4-wire LSC). Negative value indicates that the parameter is not used.

Gain: transmission – the parameter determines gain during transmission on this channel. The range of possible values is from 0 to 255 (for values see Appendix B of this Manual). Negative value indicates that the parameter is not used.

LSC: call type – type of LSC call:

0 – no;

1 – DC, direct current control;

2 – tone;

3 – voice;

4 – FXO;

5 – FXS;

6 – MB call;

7 – IVA tone frequency;

8 – DPU 60V;

9 – DPU А;

10 – DPU 60V with voice control;

11 – DC control;

12 – E&M with voice control.

LSC: channel signalling subtype: It takes the following values:

none - default value. This value shall be used if the channel

signaling subtype is not defined;

seize - this value is valid only for FXO, LB telephone signaling.

Pick up the phone to call.

tone - the value is valid only for FXO telephone signaling. Pick up

the phone to call and use pulse mode to dial the number;

pulse - the value is valid only for FXO telephone signaling. Pick up

the phone to call and use pulse mode to dial the number.

LSC: echo canceller – the parameter value determines echo canceller operation:

0 – off;

1 – on.

Radio: BSS:

0 – off;

1 – on.

Radio: PTT over SQL priority – relative priority of PTT signal and SQL signal:

1 – PTT > SQL;

2 – PTT <SQL;

3 – PTT = SQL.

In case PTT priority is lower than SQL priority, during transmission (sending of PTT signal to LI) the receiver is not switched off, transmission is prohibited. In case PTT priority equal to SQL priority, during transmission (sending of PTT signal to LI) the receiver is not switched off, transmission and reception are carried out at the same time. In case PTT priority is higher than SQL priority, during transmission (sending of PTT signal to LI) the receiver is switched off, only transmission is available.

Radio: channel ID – channel ID from radiostation’s setup.

Radio: channel frequency – radio set channel frequency (in MHz), it is transmitted to the line board when LIU-A is activated.

Radio: channel signalling subtype:

0 – default. Preferred value if no channel subtype defined;

1 – Radiostation channel in gateway mode. Radio interface type channels only. This value corresponds to the default value for channels with Radio interface type;

2 – Radiostation channel in terminal mode. Radio interface type channels only. SQL signal used as PTT, and PTT used as SQL.

Radio: emission class of radio station – the parameter is used to transmit voice and depends on the radio set type. It is transmitted to the line board when LIU-A is activated. In case the value is not configured, the value is considered to be taken from the radio set. Allowable values:

N0N – not defined;

J3E – sideband amplitude telephony with suppressed carrier;

НЗЕ – sideband amplitude telephony with full carrier;

R3E – sideband amplitude telephony with reduced carrier;

F1B – single-line frequency teletype;

G1B – single-line phase teletype;

A1B – amplitude teletype.

Radio: radiostation call type:

0 – not defined;

1 – SELCAL.

Radio: radiostation power on – energizing radio set at gateway activation:

0 – do not power on;

1 – power on.

Radio: radiostation type – type of radio set, corresponding to this channel of interface number X:

0 – channel do not support radiostation control;

1 – FAZAN-19;

2 – Polet;

3 – PP500 PIRS;

4 – FAZAN;

5 – SmartCross.

Radio: connection type^

0 – common point;

1 – dry switch relay;

65535 – unknown type.

Radio: receiver – receiver function status at activation:

0 – off;

1 – on.

Radio: remote control – radio set control (RSC) at activation:

– analog_local: local radio set control. LIA is responsible for control);

– digital_remote_tone_1020hz: remote radio set control using 
1020 Hz tone frequency. LID is responsible for control;

– digital_remote_tp112: remote radio set control «PTT plus», version 1.1.2. LID is responsible for control;

– digital_remote_tone_2600hz: remote radio set control using 2600 Hz tone frequency. LID is responsible for control;

– digital_local_tone_1020hz: local radio set control using 1020 Hz tone frequency. LID is responsible for control;

– digital_local_tone_2175hz: local radio set control using 2175 Hz tone frequency. LID is responsible for control;

–  digital_local_tone_2600hz: local radio set control using 2600 Hz tone frequency. LID is responsible for control;

–  digital_local_tone_2700hz: local radio set control using 2700 Hz tone frequency. LID is responsible for control.

# Tone control implies signal («PTT») control using tone frequency.

Radio: remote control mode – remote radio set control operation mode:

0 – controlled by local side;

1 – controlled by remote side.

Radio: remote endpoint radio channel number. Radiostation (interface) number on the remote endpoint. Valid only if LI is configured as local manager end. Interface number accept values from 0 to 7, and limited to channel capacity of LIA and LID.

Interface number accept a values from 0 to number of count –1.

Radio: remote endpoint radio interface number. Radiostation (interface) number on the remote endpoint. Valid only if LI is configured as local manager end. Interface number accept values from 0 to interface count -1.

Radio: reservation – radio set redundancy mode:

0 – off;

1 – on.

Radio: runtime channel switch: radio set channel switching over during gateway operation using special commands in signalling:

0 – prohibited;

1 – allowed.

Radio: self-monitoring – status of self-listening function at activation:

0 – off;

1 – on.

Radio: squelch operation mode – it is transmitted to the line board when LIU-A is activated. It is transmitted to the line board when LIU-A is activated. In case the value is not configured, the value is considered to be taken from the radio set.

0 – defined;

1 – off;

2 – on.

Radio: voice detection threshold – SNR threshold, determining availability of voice signal in the channel, indicated in decibels from–10dB to +10dB.

URI: reception – establishing of reception via a channel in the interface number X – data, received from the network or other channel, are switched to this channel. Allowable values:

# SIP:number

# RTP:port

RADIO:interface:channel

TABLE

For example, RTP:30000

URI: transmission – establishing of transmission from the channel in interface with number X – data received from the channels are transmitted to the network or other channel.

Allowable values:

# SIP:number[@ip_address[:port]]

# SIP:conf

# SIP:reg

RTP:ip_address:port

RADIO:interface:channel,[RTP…]

# TABLE

- SIP. If SIP is used, the address may look as follows, for example: SIP:1001@192.168.12.201:5000. In case it is necessary to establish a conference call with several subscribers, the list of receivers addresses shall be indicated separated by commas: SIP:1001@192.168.12.201:5000, SIP:2001@192.168.12.202 The maximum number of receivers is 4.

In case it is not necessary to indicate precise addresses of receivers, configuration SIP:conf or SIP:reg may be used. SIP:conf – establishing a conference call with registered peers; SIP:reg – call to all registered peers and establishing a session with the first one to answer.

- RTP. For RTP protocol both multicast and unicast of the address are allowed. For example, RTP:224.1.1.1:30000 or RTP:192.168.1.2:30002. As it is allowed to transmit to several receivers at once, their addresses shall be indicated separated by commas: RTP:192.168.1.2:30002,RTP:224.1.1.1:30000. The maximum number of receivers is 4.

- RADIO. If this channel belongs to radio interface, it operates in RRSC «remote end» mode, one of the channels, both voice and signalling will be switched to, shall be indicated as a value. For example, RADIO:3:0 means that all data, received by configured radio interface, will be switched to 3 interface and 0 channel. Unlike the previous configuration types, in case of RRSC only one receiver is allowed.

- TABLE. In case channel signalling implies its own addressing, it will be switched to URI from the table for this interface. The table search is run using two keys – channel number and Caller ID of channel signalling, see item 6.2 «Internal route tables» of this Manual. In case URI contains a list of values, the types of channels of all records shall be the same.

URI: transmission subscription – if SIP signalling is clearly indicated in URI, the gateway may subscribe to these URI, and if they are engaged, it may transmit engagement signal to the line provided that channel signalling supports it.

# 1 – on;

0 – off.

VAD: hand on receiver silence– the parameter is used for LSC/DC signalling:

0 – function disabled;

>0 – time in ms, if during the selected time for reception voice is

not detected, CM terminates the call.

VAD: hand on transmitter silence – the parameter is used for LSC/DC signalling:

0 – function disabled;

>0 – time in ms, if during the selected time for transmission voice

is not detected, CM terminates the call.

VAD: minimum threshold of exceeding signal/noise ratio – the minimum excess of voice over the noise in dB detected by the voice detector.

VAD: minimum voice duration in milliseconds – the minimum voice duration in milliseconds detected by the voice detector.

VAD: voice silence detection pause: duration of silence in milliseconds to detect no voice in channel.

6.2 Internal route tables

In case channel signalling implies its own addressing, that is the value of such channel parameters as «Transmission URI» and «Reception URI» in tab «Main parameters» is of value of TABLE, than switching is carried out to URI from the internal routing tables.

The table search is run using two keys – channel number and «Caller ID» of channel signalling. In case «Destination URI» contains a list of values, the types of channels of all records shall be the same.

Figure 91  «LIU-A parameters», tab «Internal route tables»

Parameter «Active» determines whether this record will be processed during search.

– recording is active;

– recording is not active.

# The value of «Caller ID» depends on the type of channel signalling.

# The allowable values of «Destination URI»: SIP:номер[@ip_адрес[:порт]]

# If SIP is used, the address may look as follows, for example: SIP:1001@192.168.12.201

The internal routing tables may be used differently. For example, if parameter «Transmission URI» for a given channel is configured as «TABLE», then depending on URI from table «Transmission» the call was made to, such «Caller ID» will be transmitted on channel signalling.

And if parameter «Reception URI» for a given channel is configured as «TABLE», then depending on URI «Caller ID» was received via channel signalling from, URI is selected from table «Reception» for redirecting.

The size of the table is fixed and equals to 32 records.

6.3 Signalling Configuration

This tab is used to set up the parameters of SSC, DTMF, LB, FXO, FXS signalling.

Each table line describes signalling settings for a separate channel of respective interface.

6.3.1 SSC signalling type

Figure 92  SSC signalling parameters

«Channel» – sequence number of channel in the interface number X, signalling settings belong to. It assumes a value from 0 to 7.

«Call signal duration» – rated duration of call signal in milliseconds. Value 0 corresponds to default value (600 ms) in accordance with specifications for SSC. Allowable value is from 0 to 65535 ms. LID setting.

«Detectable signal duration» – rated duration of detected signal in ms. Value 0 corresponds to default value (600 ms) in accordance with specifications for SSC. Allowable value is from 0 to 65535 ms. LID setting.

6.3.2 DTMF signalling type

Figure 93  DTMF signalling parameters

«Channel» – sequence number of channel in the interface number X, signalling settings belong to. It assumes a value from 0 to 7.

«DTMF duration» – duration of DTMF signal in milliseconds. Allowable value is from 0 to 65535 ms. LID setting.

«Pause duration» – duration of pause between two successive DTMF signals in milliseconds. Allowable value is from 0 to 65535 ms. LID setting.

«DTMF detector» – DTMF signalling detector: on/off, selected from the drop-down list, see Figure 94. LID setting.

Figure 94  Activation/deactivation of DTMF detector

6.3.3 LB signalling type

Figure 95  LB signalling parameters

«Channel» – sequence number of channel in the interface number X, signalling settings belong to. It assumes a value from 0 to 7.

«Call format number» – inductor call format number (frequency/form/power). Allowable value is from 0 to 255. LIA setting.

«Minimum call duration» – minimum duration of inductor call in milliseconds. Allowable value is from 50 to 1000 ms. LIA setting.

«Timeout of a call absence» – timeout of inductor call absence in milliseconds. Allowable value is from 1000 to 65535 ms. LIA setting.

«First call duration» – duration of first inductor call in milliseconds. The first inductor call is made immediately after beginning of session between LIU-A and subscriber. Allowable value is from 0 to 65535 ms. MC setting.

«Subsequent call duration» – duration of next inductor call in milliseconds. The next inductor call is made during established session between LIU-A and subscriber. Allowable value is from 0 to 65535 ms. MC setting.

6.3.4 FXO signalling type

Figure 96  FXO signalling parameters

«Channel» – sequence number of channel in the interface number X, signalling settings belong to. It assumes a value from 0 to 7.

«Pause duration» – duration of pause between dial pulses in milliseconds. Allowable value is from 30 to 255 ms. LIA setting.

«Pulse duration» – duration of dial pulses in milliseconds. Allowable value is from 30 to 255 ms. LIA setting.

«Pause between dialing digits» – indicated in milliseconds. Allowable value is from 200 to 2000 ms. LIA setting.

«FXO port reset duration» – indicated in milliseconds. Allowable value is from 200 to 2000 ms. LIA setting.

«FXO port seizure duration» – indicated in milliseconds. Allowable value is from 200 to 2000 ms. LIA setting.

«Minimum call duration» – minimum duration of inductor call is indicated in milliseconds. Allowable value is from 50 to 1000 ms. LIA setting.

«Timeout of a call absence» – inductor call absence is indicated in milliseconds. Allowable value is from 1000 to 65535 ms. LIA setting.

6.3.5 FXS signalling type

Figure 97  FXS signalling parameters

«Channel» – sequence number of channel in the interface number X, signalling settings belong to. It assumes a value from 0 to 7.

«Min. pause duration» – minimum duration of pause between dial pulses in milliseconds. Allowable value is from 30 to 255 ms. LIA setting.

«Min. pulse duration» – minimum duration of dial pulses in milliseconds. Allowable value is from 30 to 255 ms. LIA setting.

«Min. inter-digit pause» – minimum inter digit pause, indicated in milliseconds. Allowable value is from 100 to 1000 ms. LIA setting.

«Min. ATS port reset duration» – indicated in milliseconds. Allowable value is from 200 to 2000 ms. LIA setting.

«Min. ATS port setup duration»  –  indicated in milliseconds. Allowable value is from 200 to 2000 ms. LIA setting.

«Call sequence number» – number of inductor call sequence (template). Allowable value is from 0 to 255. LIA setting.

«Call format number» – inductor call format number (frequency/form/power). Allowable value is from 0 to 255. LIA setting.

«Call detector» - on/off, selected from the drop-down list. LIA setting.

6.4 Reservation

Tab «Standby» is intended to reserve and display of the current state of LIU-A resources.

Figure 98 – Setting of standby mode parameters of LIU-A resources

The current state of LIU-A resources is displayed in the upper part of the tab on the «Status» panel. The configuration of standby parameters is performed in the bottom part of the tab on the "Configuration" panel. When the setting is completed press button «Apply configuration».

«Sequential record number» – assumes the value from 0 to 63.

«Mode» – record type is selected from the drop-down list:

off – record is not used;

Standby – record gives reservation of resource on the current LIU-A;

SmartCross – record is used in SmartCross configuration table.

«Host identifier» – unique host identifier, assumes the value from 0 to0xFFFFFFFF.If «Standby» record type is used, 4 bits of LIU-A IP-address may act as host identifier. If «SmartCross» record type is used, host identifier can be any number.

«Service» – LIU-A resource selected for reservation. It is selected from drop-down list:

The list of available LIU-A resources:

LIU-A. Gateway (MCU);

LIU-A. Line interface 1;

LIU-A. Line interface1 channel 1;

LIU-A. Line interface1 channel 2;

LIU-A. Line interface2;

LIU-A. Line interface2 channel 1;

LIU-A. Line interface2 channel 2;

LIU-A. Line interface3;

LIU-A. Line interface3 channel 1;

LIU-A. Line interface 3 channel 2;

LIU-A. Line interface4;

LIU-A. Line interface4 channel 1;

LIU-A. Line interface4 channel 2;

ECU. Gateway (MCU);

ECU. Line interface1 (LI-RC1);

ECU. Line interface1 channel1;

ECU. Line interface1 channel 2;

ECU. Line interface2 (LI-RC2);

ECU. Line interface2 channel 1;

ECU. Line interface2 channel 2;

ECU. Line interface3 (LI-RC3);

ECU. Line interface3 channel 1;

ECU. Line interface3 channel 2;

ECU. Line interface4 (LI-RC4);

ECU. Line interface4 channel 1;

ECU. Line interface4 channel 2;

ECU. Line interface5 (LI-RC5);

ECU. Line interface5 channel 1;

ECU. Line interface5 channel 2;

ECU. Line interface6 (LI-RC6);

ECU. Line interface6 channel 1;

ECU. Line interface6 channel 2;

ECU. Line interface7 (LI-RC7);

ECU. Line interface7 channel 1;

ECU. Line interface7 channel 2;

ECU. Line interface8 (LI-RC8);

ECU. Line interface8 channel 1;

ECU. Line interface8 channel 2.

«Group number» – reservation group, to which the resource belongs. It assumes the value from 0 to 255.

«Priority» – static priority, assumes the value from 0 to 255.

«Request time» – time interval in ms, which is used by host to send messages.  It assumes the value from 0 to 65535.

«Timeout» – time interval in ms, during that host waits for a message. It assumes the value from 0 to 65535.

7 Line Interface Unit Digital (LIU-D)

This item contains information on parameters, required to set up Line interface unit digital (LIU-D).

During connection, supervisor password entry window will be opened (user root). To confirm the password, press key «Enter».

After the first successful login the entered password is saved on this TMCT and all subsequent authorizations will be carried out automatically.

Figure 99  Configuration editor, LIU-D, password entry

In case wrong password is entered, the following screen is displayed for 15 seconds (time countdown is shown in the lower right corner):

Figure 100  Screen in case of wrong password entry

After that the wrong password is reset and the initial screen is displayed. When the correct password is entered, a window with tabs appears, see Figure 101.

Figure 101  Configuration editor, LIU-D

Tab «WWW» (web interface) is user to configure IL software, controller software and router software using single interface.

Tab «Radio stations» is used to register channels in the system, for more detailed information see item 7.2 «Channel Registration in the System» of this Manual.

7.1 Setting of Web Interface

At any given time the page with settings, selected by the used, is displayed in the main window:

Hardware info – information on available LIU-D resources, software component versions and RAM use.

System – IL software configuring.

Configurations – controller software configuring.

Modems – modem pool configuring.

Routing – router software configuring.

Statistics – LIU-D use statistics.

Syslog – viewing of last information (last 500 records), registered locally in the system log Syslog.

Logout – disconnection from http server.

The tab pages are divided into logically isolated parts (sections), separated by horizontal separator with section name. A section may contain one or several windows (or contain nothing at all) and command buttons. The section, located at the bottom of the page (with a thin solid line without name as a divider) belongs to the page as a whole (page Hardware info, which is completely informative, does not have such a section).

Figure 102  LIU-D, tabs

To create windows icon  – add a window – may be arranged on the horizontal separator. One or several icons  – add a child window and icon  – close the window may be arranged on the window name toolbar. If there are several icons , when the cursor is pointed at the icon, a pop-up help with instructions on the type of window which will be added if mouse left button is pressed, appears.

The main elements inside the window are text fields (single or multi-line), drop-down lists, commands buttons and markers (check boxes). These elements do not have any specific features.

An additional element is options list field with priority and activity indicator. The field has a vertical scroll bar and horizontal separator between the list of active and disabled options. The active options are located in the upper part of the field above the separator and are shown in bold type. The uppermost active option has the highest priority. To activate (or disable) an option, as well as to change the priority, use additional controls – arrows:

– up arrow (activation or increasing of the priority)

and

– down arrow (disabling or decreasing of the priority).

The option of moving in the list is selected with the cursor, the selected option is highlighted with blue background. When an arrow is pressed, the selected option is moved by one position in the list.

Remark: in case during configuration prior to switching over to another page new values are not saved, they will be lost, as while the page selection, the values are generated again based on the current configuration file values and system utilities output. The values are saved by pressing button «OK», located in the lower part of the window.

7.1.1 Page Hardware info

# It is an information page and displays summary data on LIU-D and available resources.

Information, provided on page Hardware info, is intended mostly for manufacturer's support services. Data on LIU-D equipment components, available resources, key software module versions, RAM use are displayed on the page. The overall view of the page is shown in Figure 103.

- 7.1.1.1 Section Device information

Section Device information contains information on the main LIU-D components. They include: telecommunication central processing unit (CPU), random access (RAM) and non-volatile (FLASH) memory, framer (FRAMER), digital signal processors (DSP). Data for column Description are taken from virtual file system /proc ОС Linux:

CPU	– file /proc/cpuinfo

RAM	– file /proc/meminfo

FLASH	– file /proc/mtd

DSP	– file /proc/dspmap

FRAMER	– file /proc/driver/hdlc/peb22554

Figure 103  Overall view of page Hardware info

- 7.1.1.2 Section Software information

Section Software information contains information on key software components of telecommunication central processing unit and DSP. They include: OS Linux (OS), OS driver voice (RTP), OS daemon server (HAL SERVER), signal processors software (G.711, G.729, G.723, MODEM).

Data for column Version re taken from the following sources:

OS	– file /proc/version

RTP	– file /proc/voice/version

G.711	– file /proc/voice/version

G.723	– file /proc/voice/version

G.729	– file /proc/voice/version

MODEM	– file /proc/modem/version

HAL SERVER	– daemon launch result usr/local/sbin/server with key –version

- 7.1.1.3 Section Memory information

Section Memory information contains standard information on RAM use in Linux OS board. It includes: total memory volume in the system (Total), memory not used by the system (Free), used reserved memory (Active), unused reserve memory (Inactive), cached memory (Cached).

Data for column Value are taken from file /proc/meminfo.

- 7.1.1.4 Section DSP information

Section DSP information contains information on software, uploaded to DSP.

In accordance with the design, DSP resources are divided between two hardware banks (Bank #1 and Bank #2). Information on the installed DSP modules and availability of uploaded software in DSP is displayed for each bank.

Column DSP contains sequence number of signal processor as a device, column Codec – type (purpose) of uploaded software (in case of «----»  software is not uploaded). For non-available (not installed) DSP column Codec is empty.

# Column Bus# contains the number of bus, on which DSP is located.

# Column Free t/s provides information on the number of free time slots in each bank bus.

7.1.2 Page System

Page System is intended to configure board software and control gateway software loading process (file – /usr/local/etc/interlink.sh).

- 7.1.2.1 Section Driver Configuration

# Window Drivers

Window Drivers contains a complete list of drivers, which may be installed in the system. The main part of the drivers is enabled depending on the device specific purpose.

Figure 104 – Section Drivers Configuration

Brief description of the drivers and recommendations on configuring are given below:

amodem	– modem driver;

e1	– framer driver;

g711	– driver + DSP code of codec G.711;

g711mcc	– driver G.711 for mcc controller;

g723	– driver + DSP code of codec G.723.1;

g729	– driver + DSP code of codec G.729ab;

hal3x	– service driver, shall be always enabled;

hdlc	– service driver, shall be disabled;

kvoice	– service driver, shall be always enabled;

lapb	– service driver, shall be disabled;

mcc	– service driver, shall be always enabled;

mccnet	– service driver, shall be disabled;

mdmcodec-v205	– modems DSP code;

mixer	– driver + DSP code of conference communication and DTMF decoder;

scc	– service driver, shall be disabled;

sppp	– service driver, shall be disabled;

wdt	– service driver, shall be disabled.

# Window mcc

Window mcc is used to distribute the resources of mcc controller of telecommunication processor. All-in-all there are 128 channels of mcc controller, which shall be distributed between framer (double value from field SS7 channels), voice channels pool (Voice channels) and pool of HDLC controller channels (HDLC channels).

The number of channels in field SS7 channels shall correspond to the total number of signal channels ОКС-7/EDSS1 on the board (including standby ones).

Voice channels pool is reserved for codec G711mcc driver.

Pool of HDLC controller channels is intended to be used in the systems, including digital and analog subscriber line circuit boards. Two channels shall be reserved for each digital line circuit, one channel – for analog circuit line.

A typical example of mcc channels reservation is given below:

SS7 channels	=	4

Voice channels	=	60

HDLC channels	=	60

# Window g711mcc

Window g711mcc is used to assign reserved channels of mcc controller voice pool to the driver of codec g711mcc (field mcc. Voice channel). The value in this field shall be less or equal to the number of channels in field Voice channel of window mcc.

- 7.1.2.2 Section DSP setup

The section is intended to configure the resources, provided by DSP.

Figure 105 – Section DSP setup

# Resources window

The maximum number of channels (Channel capacity), which may be served using the installed DSP software (DSP program), is indicated in the information window of maximum amount of resources. For example:

# DSP program Channel capacity

modem	032

g711	000

g723	003

g729	055

mixer	064

# Warning! Resources, available at the moment of startup, are indicated in the window.

In accordance with the design, DSP resources are divided between two hardware banks (Bank #1 and Bank #2).

# Column DSP contain sequence number of signal processor as a device.

Column Codec contains a drop-down list of DSP software available for loading (when      «----» is selected, software is not loaded to DSP). For unavailable (not installed) DSP, the element of drop-down list in column Codec is locked.

# Column Bus# contains the number of bus, on which DSP is located.

Column Free t/s provides information on the number of free time slots in each bank bus. If during attempted configuration (distribution of software between DSP) the limit of time slots is exceeded, a warning, highlighted in color, will appear in the column: negative number – number of time slots over the limit.

Warning! When installing DSP software, make sure you activated the appropriate driver in window Drivers.

- 7.1.2.3 Section Network configuration

This section is intended for network settings.

Figure 106 – Section Network configuration

Window network.config

Fields description intended for main network settings configuring:

DHCP – use of DHCP protocol for IP– address receiving. From drop-down list «yes/no» may be chosen. DHCP protocol – network protocol, that allows devices to receive IP-адрес automatically and other necessary for operation in TCP/IP networks parameters.

IPADDR – static IP-address is indicated. It is used if the value DHCP «no» is set.

NETMASK – sub network mask is specified. It is used If the value DHCP «no» is set.

GATEWAY – gateway IP-address is specified by default. It is used if the value DHCP «no» is set.

BONDING – set checkmark if necessary to unite two physical network interfaces into one logical in order to provide network fail-safety.

BONDING_DEVS – the list of network interfaces united into one logical (bond0). Network interfaces are divided by space bar. The default value is «eth0 eth1», which is not recommended to change without coordination with manufacturer.

BONDING_OPTS – the list of parameters of bonding interface. The default value is «mode=3 miimon=100», which is not recommended to change without coordination with manufacturer.

Window system.cong

Additional settings of network filter are applied in case of network interfaces unification:

IPV4_FORWARD–redirectionofthe fourth version of IP-protocol packet:

1 – on;

0 – off.

KLOG_LEVEL – logging level of kernel message.

DDD_UDP_ENABLE – filter activation of duplicated packets:

no – filter is on;

yes – filter is off.

DDD_UDP_IFNAME – interface name, where packet filtration will be carried out. Default value is «bond0».

DDD_UDP_PEER_TTL – lifetime of UDP/ICMP connection in history (packet number).

DDD_UDP_PACKET_TTL – lifetime of ID packet in packet history of specific connection (packet number).

DDD_UDP_CACHE_MAX – connection history size (number of connections).

DDD_UDP_HISTORY_MAX – history size of identical packets.

WindowHOSTNAME

HOSTNAME – in this field text host name is specified.

- 7.1.2.4 Section Services startup

The section is intended to control startup of software, determining product operability.

Figure 107 – Section Services startup

Check box Start server process at hardware startup in LIU-D equipment shall always be selected.

Check box Start controller process at hardware startup is not selected when the board is controlled by external controller software.

Check box Start router process at hardware startup is not selected when external product control interface is used. The check boxes are used during equipment reboot.

Check box Start autoconfig process at hardware startup activation means that automatic save and server configuration back up service will be started.

Check box Reboot hardware indicates, that LIU-D software will be rebooted after the changes are saved.

Warning! In the current software version the changes come into effect only after software reboot.

Save changes reboots software. During reboot the following animated window appears on the screen:

Figure 108 – Rebooting

Upon completion of rebooting, it is necessary to repeat login procedure, described above, to log in.

- 7.1.3 Page Configuration

The page is intended to generate control data of controller software and set up the parameters of controller software interaction with server software (file – /usr/local/etc/controller.conf).

- 7.1.3.1 Section Main settings

Figure 109 – Section Main settings

The settings of this section may be changed only based on manufacturer's recommendations.

- 7.1.3.2 Section Boards configuration

Figure 110 – Section Boards configuration

Symbolic name of the board is indicated in field «Board name», its IP address – in field «Board IP».

- 7.1.3.3 Section SIP/PJ

LIU-D acts as UA in the network based on SIP protocol. SIP protocol v.2 is implemented in LIU-D.

T.38 protocol (up to 5 channels T.38) is supported. When voice data is transmitted, codecs G.711, G.729, G.723, based on DSP, may be used. It is not recommended to use codecs G711mcc.

Figure 111 – Section SIP/PJ

- 7.1.3.4 Section H323 configuration

LIU-D acts as a gateway without multi-endpoint connection controller (terminal Type=60) in the network based on H.323 protocol stack. The product is compatible with the most widespread versions of H.323 protocol stack (v2-v4).

To minimize time of connection establishing, Fast Connect (Fast Start) procedure may be used. Encapsulation of H.245 messages (H245 tunneling) is not used at the moment.

T.38 protocol (up to 5 channels T.38) is supported.

When voice data is transmitted, codecs G.711, G.729, G.723, based on DSP, may be used. It is not recommended to use codecs G711mcc.

Window Terminal # allows to specify personal settings for specific network point.

Figure 112 – Section H323 configuration

- 7.1.3.5 Section Trunks configuration

E1 interface configuring consists of referencing of the interface to the board, assigning a unique identifier to each interface within the board, assigning a unique identifier to each interface within the article, selection of signalling system and assigning a group identifier, based on which call routing in the product is carried out.

The interfaces are configured after the gateway board the interfaces belong to is configured.

Synchronization source is selected depending on the product activation circuit. In case the circuit does not imply use of E1 interfaces or partner ATX is not a part of network (private or public network) and can not act as a synchronization source, the built-in synchronization source (mode «free run») is used. Otherwise, synchronization sequence is taken from the E1interface reception path, selected during configuring. The interface is selected using its unique (within the product) identifier.

Figure 113 – Section Trunks configuration

EDSS1 configuring consists of selection of network side, that means the role of LIU-D for ATX (or other device), connected to this E1 interface – a part of the network or terminal unit.

Field «Synchronizer» – selection of E1 interface (Trunk sequence) to extract a synchronization signal from E1 reception path in case an external synchronization source is used. It is filled in with parameter Trunk ID value.

Field «Trunk ID» – unique interface identifier within the product (controller). In theory, it assumes a value from 1 to N×4, where N is the number of Interlink boards.

Field «Board» – Interlink board identifier (parameter slot1, slot2, …) within the controller.

Field «Digital path» – E1 interface identifier within each Interlink (Board).

Field «Trunk logic» – the type of signalling used on E1 trunk is selected from the drop-down list:

cas – channel-associated signalling using protocols r 1.5, r2, e_and_m (parameter Cas type);

isdn – signalling protocol DSS-1 of digital network ISDN;

ss7 – protocol «common signalling channel No. 7». If this signalling type is selected, go to section SS7 Configuration to set up the respective parameters, see item 7.1.3.6 «Section SS7 configuration» of this Manual.

Field «Group ID» – interface identifier to search for route (field group) of router service, as well as control point of CC RINO server SIP channels. The field is available when signalling isdn is selected (field «Trunk logic»).

Field «Network side» – network termination side. Values: network or user, selected from drop-down list.

Field «Timeslots» – voice channel timeslots of E1 interface. The number of timeslots, available to the user, is from 0 to 31, depending on signalling, most often it is 30 (slot 0 is reserved for service information, slot 16 is recommended but not mandatory for service information). Therefore, slots 1 to 31 may be used for data and voice transmission. Format for cas and isdn: 1..15,17..31.

Field «Links» – selection of link for ss7 trunk.

Field «Hdlc type» – for ECT (Emergency Communication Telephone) the value is «alarm_ggs».

Field «Hdlc timeslot» – for ECT the value is«31».

Field «Hdlc address» – for ECT the value is«0x1111».

- 7.1.3.6 Section SS7 configuration

SS7 configuring consists in assigning an address of signalling point (OPC code), DPC code (code), configuring link sets, setting signalling links and ISUP sub-system parameters.

Signalling point address (OPC code) is provided by appropriate state authority. Number and configuration of link sets is determined by scheme and conditions of connection to the operator network. Marks of signalling links SLC and codes of DPC destinations are determined by the connecting network operator.

ISUP parameters are selected as agreed with the operator. Channel identification code CIC determines the number of speaking channel between two sets (five lest-significant bits indicate the timeslot, the other ones – E1 number). Timeslots, used as signalling channels, are excluded from speaking channels.

In case signalling type ss7 is selected in field «Trunk logic» (see item 7.1.3.5 «Section Trunks configuration» of this Manual), set up the parameters if this signalling.

Figure 114 – Section SS7 configuration

To create a linkset, press button «Add linkset».

Linkset – link identifier, it is assigned automatically when link adding button is pressed (Add linkset).

Field «DPC» – destination point signalling code in the network.

Link#N – description of link N:

Field «Timeslot» – channel timeslot of E1 interface signalling;

Field «SLC» – link code (from 1 to 16).

Field «Trace ID» – integer-value link identifier (value from 0 to 65535).

ISUP – description of ss7 user part parameters:

Field «CIC Base» – conditional identifier of initial voice channel timeslots (usually starts with 0).

Field «Timeslots» – voice channel timeslots of E1 interface. For a stream with link in the first channel timeslot: 2..31.

Field «Group» – interface identifier to search for route of router service, as well as control point of CC RINO server SIP channels.

- 7.1.3.7 Section АК configuration

This section is intended for multi-operated interface boards configuring.

Figure 115 – Section АК configuration

Field «Slot» – the number of multi-operated interface board mounting position

Field «Board» – see item 7.1.3.5 of this Manual.

Field «Group» – see item 7.1.3.5 of this Manual.

Field «E&m_v5-channels» – the comma-separated list of channels, which are used as E&M (from 4 to 7).Channels that are not in this list will be used as FXO. Channels from 0 to 3 are always used as FXS.

Field «4wire-channels» – the comma-separated list of 4-wire channels. Channels that are not in this list will be used as 2-wire channels. Channels from 0 to 3 are always used as 2-wire channels.

Check mark «Save changes and try to apply setting without restarting controller» – in addition to saving changes made on this page, the controller service will try to apply new configuration without restarting and disconnecting currently established connections and sessions.

Check mark «Save changes and restart controller» – the controller service will restart with a new configuration if this option is checked. This is a recommended method when there are many changes in configuration was made.

Figure 116

7.1.4 Page Modems

# Page Modems is intended to configure fax modems support.

Figure 117 – Page Modems

7.1.5 Page Routing

The page is intended to generate route data of router software and set up the parameters of router software interaction with server controller (file – /usr/local/etc/router.conf).

- 7.1.5.1 Section Main settings

Figure 118 – Section Main settings

Parameter Address contains IP address and port for connection to controller software.

Parameter Remote CDR receivers contains IP address and port for connection to autonomous billing software.

Parameter Session timeout, seconds determines timeout of route information for router software.

- 7.1.5.2 Section Signalling targets

The section is intended to generate outgoing call parameters (address data of called subscriber, information on calling subscriber, codec selection preferences) and indicate the route in accordance with the group number. It is recommended to determine all routes prior to operation with section Patterns of this page.

Field name shall contain route name. To facilitate configuration, it shall be significant, for example:

# SIP>ISDN/SS7

Field called generates the address line from the address data, received by the gate. The address line is generated using regular expressions, indicated in the field. The field may contain several regular expressions each expression is given in «/xxx//» construction. If there are several constructions, they shall be separated by at least one space.

For example (see Figure 119) – address data, received from SIP-UA, looks as follows:

«sip:+74843971106@192.168.1.79», it is necessary to receive local 5-digit number, that is transform the sequence to the form of «71106».

For conversion, a line of the following form is entered: «/sip:\+748439// /@.*$//».

The first sequence «/sip:\+748439//» removes «"sip:+748439» from the address data, the second one – «"/@.*$//» removes «@192.168.1.79»,

where: «$» is end of line, «.*» is any number of any symbols, including symbol 0.

Figure 119 – Section Signalling targets

# ISDN/SS7>SIP

For example, (see Figure 119) – information on called subscriber, received from partner ATX, looks as follows:

«4843930000», it is necessary to receive SIP URI with calling subscriber number and gateway credentials, i.e. transform the sequence to the form of «4843930000@192.168.1.53».

For conversion, a line of the following form is entered:

«/(^.+$)/\1@192.168.1.53/».

The first sequence «/(^.+$)/» saves the whole number, received from the partner ATX with temporary variable «1»; the second one «\1@192.168.1.53/» generates the end of line from the content of temporary variable «1» and line «@192.168.1.53»: «4843930000@192.168.1.53»;

where: «^» is the beginning of line, «.+» is one or several symbols.

Field caller generates information on the calling subscriber. Information is generated using regular expressions, similarly to filled called.

# Example:SIP>ISDN/SS7 (Figure 119)

Information on calling subscriber, received from SIP-UA, looks as follows:

«sip:192.168.1.53», it is necessary to change/generate ANI, i.e. transform the sequence to the form of, for example, «0123456789».

For conversion, a line of the following form is entered:

«/(sip.+$)/0123456789/»

# Example:ISDN/SS7>SIP (Figure 119)

Information on calling subscriber, received from ATX, looks as follows:

4843971106, it is necessary to transform it to the following form: 4843930000@192.168.1.44.

For conversion, a line of the following form is entered:

/(^.+$)/\1@192.168.1.44/, but if ANI is not received from ATX (empty), the coincidence construction fails to operate and the router applies a default value: sip:192.168.1.79 (gateway address).

Field codec set of window #xx is not used in the current software version.

Field type is intended to set target protocol parameters.

Example:

type isdn

isdn.cg.typeofnumber – Calling Party Number (CGNUM) TypeOfNumber

isdn.cg.plan – CGNUM NumeringPlanIdentification

isdn.cg.screeningind – CGNUM ScreeningIndicator

isdn.cd.typeofnumber – Called Party Number (CDNUM) TypeOfNumber

isdn.cd.plan – CDNUM NumeringPlanIdentification

Field group displays parameter Group ID (of page Configuration) of target interface, i.e. identifies it. It shall not be left empty.

- 7.1.5.3 Section AK targets

Section AK targets is intended to configure analog subscriber line circuit boards.

Field name determines «telephone» number of AK.

Field slot determines the number of AK board slot.

Field channel determines the number of channel on AK board.

Field caller id determines the number of calling AK (AIN).

Field «called» is not used.

Logic of router operation with АК is different from channel operation using signaling, consequently in this case there is the point to consider it together with section Patterns (it will be discussed below).

Incoming call initially goes though appropriate (by board parameters: slot, channel) АКtargets, where it is «attached» by virtual AIN (caller id). An appropriate pattern is searched next by parameters «called» and «caller id», and if successful, destination target will be determined.

Example: FXS>АТА (Figure 120).

Incoming call goes to the FXS channel (slot=8, channel=0), with dialed number 5, AK target #1(to_fxs) is determined and parameter «caller id=3» is added to the call. Pattern#1 (called=^5.+, caller=3) is selected next, and is sent to signalling target #1 «to_alarmphone» (АТА). This target setting has its own format – inline (^.*)/trunk:04@01\1/ fragment «trunk:04@01» determines the trunk (4) and timeslot in it (01), by which the call will broadcast on АТА.

Example: FXO>FXS (Figure 120).

Incoming call (inductor) goes to the FXO channel (slot=8, channel=4), AK target #2 (to_fxо) is determined and parameter «caller id=9»is added to the call,pattern #4 (called=^.*, caller=9) is selected next, and is sent to AK target #1 «to_fxs».

Figure 120 – Section AK targets

- 7.1.5.4 Section Modem targets

Section Modem targets is intended to configure modem terminations of article LIU-D.

- 7.1.5.5 Section Patterns

Figure 121 – Section Patterns

The section is intended to select a path for incoming call routing. The selection is made by comparing received gateway address sequence with some sample (template, pattern). Comparison with pattern is carried out successively (i.e. starting with Pattern #1, then with Pattern #2 and so on until match, or until the patterns are over). If no suitable pattern is found for an address sequence, the incoming call will be terminated with a message on the absence of route. Comparison is carried out until the first match, see Figure 122.

Figure 122 – Section Patterns

Field called of window Pattern #xx determines the pattern for comparison with received address data. The pattern is a regular expression.

ExamplePattern#1 (see Figure 121):

It is necessary to extract a local number from the address data, received from SIP network, and check it for compliance with PIATX numbers range, connected directly to the gateway. Suppose, address data, received from SIP network, looks as follows: «sip:+74843971142@obninsk.ru». It is necessary to check the local number for compliance with number range of «70000-79999», selected by PIATX.

# Field called will have value «^sip:\+7484397+».

Field caller of window Pattern #xx analyses information on the calling subscriber, operating similarly to field called.

Example (Figure 121): value «^sip:192.168.1+» checks information on calling subscribers using sip protocol for compliance of IP address with initial values 192.168.1, for example 192.168.2.35 will not pass this filter.

Value «^sip:192.168.1+» checks information on calling subscribers using SIP protocol for compliance of IP address with initial values 192.168.1, for example 192.168.2.35 will not pass this filter.

Remark: value «^$» indicates an empty line, and the pattern will match ANI only in case the letter is absent.

For Pattern#2 (Figure 121) it is important that the first digits of a number and ANI, received from PIATX, coincide with the pattern mark.

Field group of window Pattern #xx determines the identifier of interface, via which the call is received, it may be empty.

Field targets of window Pattern #xx determines the route in case the address data match the pattern. All available routes, configured before, are shown as a drop-down list.

# Filed type of window Pattern #xx

reject – rejection with code 21 is generated

native – route is allowed

called – rejection with code 3 is generated

# Field routing timeout of window Pattern #xx determines the router timeout.

# Field answer timeout of window Pattern #xx determines the answer timeout.

# Field dial timeout of window Pattern #xx determines the dial timeout.

An example of setting an incoming call routing over several routes at once or in succession.

Several signalling targets are shown in Figure 123, in particular we were interested in targets with names sip131, sip132 and sip133.

Figure 123 – Signalling targets: sip131, sip132, sip133

Setting of incoming call routing over three routes at once with answer timeout of 10 seconds (answer timeout = 10), until the first subscriber to answer or upon expiry of answering interval is shown in Figure 124.

Figure 124 – Setting of incoming call routing over three routes at once

In this example three routes are determined if field targets, this pattern will be used in case the address data match the pattern (field called). If necessary, the answer interval may be changed for different routes.

Now let's consider an example of incoming call routing over three routes, but with successive use see Figure 125.

Figure 125 – Setting of incoming call routing over three routes with successive use

Route sip131 (answer timeout = 10, dial timeout = 0), will be used first, dial without timeout, answer timeout is 10 seconds.

After that route sip132 with dial timeout of 10 seconds and answer timeout of 10 seconds will be used, upon expiry of timeouts, route sip133 is used.

20 second timeout is set for route sip133 with answer timeout of 10 seconds, upon expiry of which the call is terminated if there is no answer.

# If there is an answer over one of the routes, the next routes are not used.

The timeout may be changed optionally, the essential point is that if the dial routes are to be used successively, it is necessary to observe a specific formula: dial timeout of the next route shall be equal to the time of answer timeout + dial timeout of the previous route.

- 7.1.5.6 Section Connections

The section is intended to set up terminals connection to the radio sets. The parameters, selected in the section, are displayed in tab «Radio stations», see item 7.2 «Channel Registration in the System» of this Manual.

Figure 126 – Section Connections

# Field type – type of Radio set, selected from the drop-down list.

Field radio.rx.addr – IP address of the receiver.

# Field radio.rx.port – port.

Field radio.tx.addr – IP address of the transmitter.

# Field radio.tx.port – port.

Field radio.freq.no – frequency in MHz.

Field radio.freq.chan – channel number in Radio set.

Values, indicated in fields cas.trunk.no and cas.trunk.timeslot, are taken from section «E1 Trunks configuration» (Trunk ID and Timeslots, respectively), see item 7.1.3.5 «Section Trunks configuration» of this Manual.

- 7.1.5.7 Section Check patterns

The section is intended to verify that the routing settings are correct. Depending on the input data enter the required values in the fields «called», «caller» or «group».

Figure 127 – Section Check patterns

When you click on the «Check» button, the «Result» window will appear with information according to the specified parameters.

- 7.1.5.8 Button «Save changes»

Figure 128 – Button «Save changes»

Button «Save changes» saves all the changes made on this page and instructs the router service to read the updated configuration file.

7.1.6 Page Statistics

Page Statistics contains data on the current distribution of LIU-D resources and network side statistics with channels (telephone network) switching.

Note: if necessary, statistics of data transmission network is gathered using utility tcp dump. To analyze the gathered data, programs Hammer Call Analyzer and Wire Shark may be used.

- 7.1.6.1 Section Controller

Controller version R0754 – the line contains controller software version, the first letter {Test Release} indicates the type of software: T – test, R – release software version.

Controller started: 10/08/2015 00:00:53 uptime 20 days 00:00 – date and time of controller software start, number of days, hours and minutes from the moment of last loading.

# Statistics created: 30/08/2015 00:00:54 – time of statistics page creation.

Figure 129 – Section Controller

# Table Board

Table Board contains information on LIU-D boards. Parameters: «Board name», «IP», «State», «Board version», «Driver version», «Restarts», «Connection time», «References».

«Board name» – board name.

«IP» – IP address and IL software control port on the board.

«State» – board state at the time of scanning.

«Board version» – board version.

«Driver version» – driver version.

«Restrats» – board restarts counter from the time of controller software start.

«Connection time» – time of continuous interaction of IL software and controller software.

# Table System resources

Parameters: «Board name», «G.723 RTP (u/a)», «G.729 RTP (u/a)», «MCC-T (u/a)», «MCC-B (u/a)», «MCC-D (u/a)», «A-Modem (u/a)», «D-Modem (u/a)», «T.38 (u/a)», «Mixer (u/a)».

«Board name» – board name

«G.723 RTP (u/a)» – number of codecs G.723 (used/available).

«G.729 RTP (u/a)» – number of codecs G.729 (used/available).

«MCC-T (u/a)» – number of voice channels G.711mcc (used/available).

«MCC-B (u/a)» – number of voice channels assigned for ISDN-BRI set (used/available).

«MCC-D (u/a)» – number of channels assigned for boards control and for signalling links of ISDN-PRI (used/available).

«A-Modem (u/a)» – number of analog modems (used/available).

«D-Modem (u/a)» – number of digital modems (used/available).

«T.38 (u/a)» – number of channels T.38 (used/available).

«Mixer (u/a)» – number of channels G.711 (used/available).

# Table Е1 trunks

Parameters: «ID», «Board name», «Digital path», «Status», «Clock source», «Slip alarm», «Basic frame alarm», «Carrier alarm», «AIS alarm», «RAI alarm», «Watch».

«ID» – system-unique trunk identifier.

«Board name» – name of board the trunk belongs to.

«Digital path» – board-unique trunk identifier.

«Status» – {SYNC, CL}

«Clock source» – {*, -} – attribute, indicating whether a trunk is a synchronization sequence source.

«Slip alarm» – slip error.

«Basic frame alarm» – alignment error.

«Carrier alarm» – carrier error.

«AIS alarm» – indication of input signal loss.

«RAI alarm» – remote alarm indication.

«Watch» – indication whether the trunk status as potential synchronization sequence source during backup is monitored.

Figure 130 – Table E1 trunks

# Table ISDN brief statistics

Parameters: «trunk», «group», «side», «status», «restarts», «timeslots (u/a)», «calls», «unsuccessful calls», «T303», «T304», «T308», «T310», «T313».

«trunk» – system-unique trunk identifier.

«group» – group identifier.

«side» – network side.

«status» – {Multiple frame established, Awaiting establishment}.

«restarts» – number of restarts.

«timeslots (u/a)» – number of timeslots: used (u)/ available(a).

«calls» – total number of calls.

«T303» – value of timer T303 operation counter. Timer purpose: interval between transmission of SETUP and reception of ALERT, CONNECT, CALL_PROCEEDING, SETUP_ACKNOWLEDGE or RELEASE_COMPLETE.

Value of T303 = 4 s.

«T304» – value of timer T304 operation counter. Timer purpose: interval between reception of SETUP_ACKNOWLEDGE and transmission of message INFORMATION (during transmission of address data in the overlapping mode) and reception of message ALERT, CONNECT or CALL_PROCEEDING.

Value of T304 = 15 s.

«T308» – value of timer T308 operation counter. Timer purpose: interval between transmission if message RELEASE and reception of message RELEASE_COMPLETE or RELEASE.

Value of T308 = 4 s.

«T310» – value of timer T310 operation counter. Timer purpose: interval between reception of message CALL_PROCEEDING and reception of one of messages ALERT, CONNECT, PROGRESS or DISCONNECT.

Value of T310 > 40 s.

«T313» – value of timer T313 operation counter. Timer purpose: time interval between transmission of message CONNECT and reception of message CONNECT_ACKNOWLEDGE.

Value of T313 = 4 s.

Figure 131 – Table ISDN brief statistics

# Table ISDN detailed statistics

group=0 trunk=1 – the line indicates the number of trunk and trunk group identity.

Parameters: «id», «timeslot», «call state», «node state», «isdn state», «call reference», «E1 timeslot», «call waiting», «calls».

Figure 132 – Table ISDN detailed statistics

# Table SS7 detailed statistics

If there are no SS7 trunks in the system, only headers are given instead of tables SS7 detailed statistics and ISUP detailed statistics.

# Table ISUP detailed statistics

Parameters: «id», «timeslot», «call state», «node state», «ss7 state», «E1 timeslot».

Figure 133 – Table ISUP detailed statistics

7.1.7 Page Syslog

Page Syslog is intended to be used by the manufacturer's support service. By default both local logging-in and logging in external host are disabled. Used configuration of system Syslog is given in directory /etc in file syslog.conf.

When local logging-in is disabled, page Syslog looks as follows:

Attention! Enabling of local logging-in in syslog (logging-in in file var/log/messages) without consent of support service may lead to RAM shortage and, thus, degradation of device performance.

The state of Syslog at the moment of browser connection to http server is displayed on the page. The page is not refreshed automatically. To refresh the data manually, button «Refresh» is used.

When local logging-in is enabled, page Syslog looks as follows:

Figure 134 – Page Syslog

7.1.8 Page Logout

When tab «Logout» is selected, the user is disconnected from http server.

7.2 Channel Registration in the System

After web interface is set up, go to tab «Radio stations» to register the channels in the system. The overall view of the tab is shown in Figure 135.

The upper part of the tab accommodates button «Check parameters», when it is being pressed, a request is made to LIU-D to receive an updated list of radio sets.

– reception of updated list of radio sets.

Figure 135 – Tab «Radio stations», overall view

The list of radio stations (channels) with specified parameters is displayed in the tab. The parameters are taken from section «Section Connections», see item 7.1.5.6 of this Manual.

The channel is registered in the system under its name. Indicate the channel name in field «System channel name», for example, PEER.251.1, see Figure 136.

Figure 136 – Tab «Radio stations», entering channel name

After quitting the field, the status of channel PEER.251.1 is automatically changed to «Registered in system», see Figure 137.

Figure 137 – Tab «Radio stations», channel PEER.251.1 is registered in the system

Channel status:

|  | Channel is not registered in the system |
| --- | --- |
|  | Channel is registered in the system |

An alternative method of channel registering is to open the context menu with right mouse button and select item «Register channel», see Figure 138.

Figure 138 – Tab «Radio stations», registering a channel

If a user attempts to register a channel without name, the name will be assigned to the channel automatically in the following format: RADIO.HOSTID.LICHANNEL, see Figure 139.

The field with automatically assigned name is editable.

Figure 139 – Registering a channel with automatically assigned name RADIO.10.1

To delete channel registration from the system, select a required line, open the context menu and select item «Unregister channel»).

Figure 140 – Deleting channel registration from the system

The registered channels are displayed in the list of radio sets, see Figure 141. In this example, these are channels with names PEER.251.1 and RADIO.10.1.

List «Radiostation» consists of the radio set channels, which are set up in the system objects of LIU-A, BTC LIU-A, LIU-D type, see item 5.2.3 «Radiostations» of this Manual.

Figure 141

8 Briefing and Training System (BTS)

Instructors carry out air traffic controllers (dispatchers) training in the standalone and complex mode.

In the standalone training mode, dispatcher's WP 1 and instructor's WP 1 are connected to each other only. Connection is provided only between these two WS's using button Radio (Trainer).

In the complex mode, at least two dispatchers and two instructors carry out operations. Dispatchers communicate via LSC communication with each other, and not with the instructor (as in the standalone mode). In this case the instructor acts as a pilot and external LSC subscribers, at that the instructors hear communications of their dispatchers.

Each dispatcher works at his workstation (WS) with its unique configuration. Training configuration is exactly the same as WS of each dispatcher. Instructor's task is to simulate standard and abnormal situations, which may take place during dispatcher’s work.

8.1 VCDT Training Configuration

According to the type of WS, all VCDT are divided into «Instructor» and «Dispatcher». WS type is set at the stage of system object creation (see item 4.1 «Creating a System Object» of this Manual).

Figure 142 – Creating a new system object

The text description of VCDT name must contain keyword «Instr» or «Disp», which determines the type of WS – «Instructor» or «Dispatcher».

Operation mode selection and updating training workstation’s configurations are carried out in the window, which is opened by selecting menu item «VCDT Training configuration», see Figure 143.

Figure 143 – Menu item «VCDT Training configuration»

When the menu item is selected, the window, whose overall view is shown in Figure 144, will be opened.

Buttons for selecting configuration updating modes – «Standalone» or «Complex» are located in the upper part of the window. Button for access to the page of setting with different parameters, facilitating VCDT selection and configuration for their updating, is located in the right part.

Figure 144 – Overall view of window «VCDT Trainer configuration»

The number of pairs, which is required for operation, is indicated in field «Instructor – Dispatcher pairs».

In the standalone mode the minimum number of set pairs is 1, in the complex mode – 2. The maximum number of pairs in both modes is 9.

VCDT, to which configuration shall be set, is selected from drop-down list «Instructor», the selection is made from all available system VCDT of indicated type. WS configuration for respective VCDT is selected from drop-down list «Configuration», the selection is made from all available system configurations, see Figure 145.

VCDT, to which configuration shall be set, is selected from drop-down list «Dispatcher», the selection is made from all available system VCDT of indicated type. WS configuration for respective VCDT is selected from drop-down list «Configuration», the selection is made from all available system configurations, see Figure 145.

If there are two configurations with the same name, for example, «Standalone Instr AC» and «Standalone Disp AC», when one of them is selected for this WS, the second one is selected automatically.

Figure 145 – Selecting configuration

To upload the selected configurations to the selected VCDT, press button «Upload configurations». Detailed information on the progress of updating will be displayed in the protocol window, see Figure 146.

Figure 146 – Uploading current configuration

The selected VCDT and their configurations can be saved as a named set to be used again. Press button «Save set», select item «Save as». Specify the name in the title entry field and press «OK»:

Figure 147 – Saving configuration

To load a previously saved set of configurations to the window, press button «Open set» and select the required set by its name.

Figure 148 – Opening configurations set

After that, the list of VCDT and their configurations is loaded in the window. To apply the selected set, press button «Upload configurations».

Alternative method of configurations set uploading is to use menu item «Load Training Configuration Set», see Figure 149.

Figure 149 – Alternative method of configurations set opening

Select a respective mode and configurations set name. Window «VCDT Training Configuration» with the selected set will be opened. To apply it, press button «Upload configurations».

# The saved set may be deleted by holding down key «Shift» while it is selected.

To save a configurations set under other existing name, press button «Save set» and select an existing name (in this example, Set1).

Figure 150 – Saving another configurations set under existing name

In case of positive answer, another configurations set will be saved under name Set1.

Figure 151 – Saving another configurations set under name Set1

VCDT equivalence table may be used to facilitate selection of configurations of «Instructor» and «Dispatcher» WS.

To open the equivalence table, press the button located in the upper right part of the window, see Figure 152.

Select Instructor's VCDT and respective Dispatcher's VCDT from the drop-down list.

For the standalone mode the address of external bearing messages reception system is specified in the following format:

<IPaddress>:<Port>:<ID>, ID parameter (direction finding identifier)is not mandatory.

In case the address is specified in the Equivalence table in format <IPaddress>:<Port>:<ID>, it completely replaces the address, indicated in the settings of button Radio (Trainer), see item 5.3.4 «Radio button properties (Trainer)» of this Manual.

If the address is specified in format <IPaddress>:<Port>, the button address is analyzed to calculate its ID.

Thus, the address in Radio (Trainer) button can be specified using two variants only – <IPaddress>:<Port>:<ID>or <ID>.

For the complex mode, the value, specified in field «Default bearing address» is used (see item 5.1.2.3 «Radio communication» of this Manual).

Figure 152 – VCDT equivalence table

For example, when Instructor 1 VCDT «Instructor 1» is selected on the standalone mode page, «Dispatcher 1» is selected automatically in the Dispatcher VCDT field in accordance with the Table, see Figure 153.

Figure 153 – Automatic equivalent selection

8.2 Configuring BTS LIU-A

Protocol of configuring and monitoring of LIU-A (Line interface unit analog) and BTS «Topaz» (Briefing and training system) LIU-A.

This item contains the description of tabs «VOIP peers», «Patterns» and «Files». Description of parameters of tabs «Main Parameters» and «Signalling Configuration» is given in items 6.1 and 6.3 of this Manual.

Figure 154 – BTS LIU-A parameters, tab «VOIP peers»

8.2.1 VOIP peers

The parameter value determines VoIP peer activity. In case the peer is on, it will be taken into account during analysis of switching pattern table (tab «Patterns»).

# 1 – on;

0 – off.

Sequence number of VoIP peer. It assumes a value from 0 to 255.

# RURI

URI for establishing incoming call to LIU-A using one of VoIP protocols.

SIP:number | RTP:port

For example: RTP:30000 or SIP:240

- SIP:number.This value corresponds to simplex call via SIP. Signalling control using the subscription mechanism SUBSCRIBE/NOTIFY. Number in the SIP signalling can be both a usual number for SIP (SIP:300) and contain set expressions.

- RTP:port. This value corresponds to simplex call via extended RTP protocol. The port number is also used as SIP number to transmit signalling using the subscription mechanism SUBSCRIBE/NOTIFY.

If LIU-A IP address is 192.168.12.240 and the value of one of LIU-A VoIP peers in field RURI is SIP:240, to call this peer using SIP protocol, called URI on the calling side (when it is called) shall be SIP:240@192.168.12.240.

If LIU-A IP address is 192.168.12.240 and the value of one of LIU-A VoIP peers in field RURI is RTP:30000, to call this peer using extended RTP protocol, called URI on the calling side (when it is called) shall be RTP:192.168.12.240:30000.

Further call switching is carried out in accordance with the switching pattern table (tab «Patterns»).

# TURI

URI for establishing outgoing call from LIU-A using one of VoIP protocols. Allowable values:

# SIP:number[@ip_address[:port]] | RTP:ip_address:,[RTP…]

- SIP. If SIP is used, the address may look as follows, for example: SIP:1001@192.168.12.201:5000. In case it is necessary to establish a conference call with several subscribers, the list of receivers addresses shall be indicated separated by commas: SIP:1001@192.168.12.201:5000, SIP:2001@192.168.12.202. The maximum number of receivers is 4. Number in the SIP signalling can be both a usual number for SIP and contain set expressions.

- RTP. For RTP protocol both multicast and unicast of the address are allowed. For example: RTP:224.1.1.1:30000 or RTP:192.168.1.2:30002. As it is allowed to transmit to several receivers at once, their addresses shall be indicated separated by commas: RTP:192.168.1.2:30002,RTP:224.1.1.1:30000. The maximum number of receivers is 4.

In case URI contains a list of values, the types of channels of all records shall be the same.

# Control over subscriptions

If SIP signalling is clearly indicated in TURI, LIU-A may subscribe to these URI, and if they are engaged, it may transmit engagement signal to the calling channel provided that calling channel signalling supports it.

# 1 – switch on subscriptions;

0 – switch off subscriptions.

# Signalling subtype:

0 – default. Default value. In case the subtype of

signalling is not determined for the channel, this value shall be used;

# 1 – radio_gateway. The value is relevant for channels with

RTP signalling type. RTP channel operates in the gateway mode. This

value also corresponds to the default value for channels

with RTP signalling type;

# 2 – radio_workstation. The value is relevant for channels with

RTP signalling type. RTP channel operates in the workstation

mode. Signal SQL is used as PTT, while PTT – as SQL;

# 3 – rtp_gateway. The value is relevant for channels with

RTP signalling type. RTP operates in rtp_gateway mode and cab be

switched to another RTP channel in rtp_gateway mode. At that

reception will be switched to transmission, and transmission to reception.

# Name of VCES channel

The name of VCES channel is indicated. The list of these channels will be indicated in field «Radiostation» (see item 5.2.3 «Radiostations» of this Manual).

# Set expressions formation principles

The set expressions for incoming calls may have the following values:

- «T» (one letter) – designates any number of digits.

- «T..T» (N letters) – designates that the number of digits shall be equal to N.

For example: SIP:1TT. All three-digit numbers, starting with 1, correspond to this pattern.

The set expressions for outgoing calls may have the following values:

(CALLED:L:C) – returns last L symbols of field «Called», if the length of «Called» 1 is less than L, the number is supplemented with L-1 symbols C from the left.

(CALLED:L) – returns (CALLED:L:0).

(CALLED) – returns field «Called» as it is.

{CALLED:S:L} – returns first L symbols of field «Called» starting with symbol number S. Symbols numbering starts with 0.

{CALLED:S} – returns {CALLED:S:L}, where L is the length of field «Called».

{CALLED} – returns field «Called» as it is.

The number may contain one pattern only.

For example, if the value of RURI is «SIP:1(CALLED:3)@192.168.12.61», and Called is equal to «12», after substitution it will be «SIP:1012@192.168.12.61».

8.2.2 Patterns

The switching pattern table is used to route the incoming calls. If an incoming call is received in the channel, there is the first active record for this channel in the pattern table, its fields «Called» and «Caller» coincide with the values of this incoming call. If such a record is found, the call is switched to the outgoing channel.

Fields «Called» and «Caller» are filled in differently for different channels during incoming call. In general case, field «Called» indicates the number of party being called without any additional signalling data. In general case, field «Caller» indicates the number of caller without any additional signalling data.

The parameter value determines the record activity. In case the record is active, it will be taken into account during analysis of switching pattern table:

1 – on;

0 – off.

Sequence number of record. It assumes a value from 0 to 255.

Figure 155 – BTS LIU-A parameters, tab «Patterns»

# Channel incoming

The name of the channel an incoming call is made to. The following values are allowable:

«VOIP:N» – this value corresponds to the peer number N from VoIP peers table;

«CHANNEL:I:C» – this value corresponds to the channel number C of interface number I;

«FILE:N» – this value corresponds to a sound file number N from sound file table;

«» – an empty channel name will be ignored.

Called incoming

# If this field is not empty, it will be compared to field «Called» of incoming call.

# Caller incoming

# If this field is not empty, it will be compared to field «Caller» of incoming call.

# Channel outgoing

The name of the channel an incoming call will be switched to. The following values are allowable:

«VOIP:N» – this value corresponds to the peer number N from VoIP peers table;

«CHANNEL:I:C» – this value corresponds to the channel number C of interface number I;

«FILE:N» – this value corresponds to a sound file number N from sound file table;

«» – an empty channel name indicates that the call will be terminated.

# Called outgoing

# If this field is not empty, field «Called» of outgoing call will be substituted with this value.

# Caller outgoing

# If this field is not empty, field «Caller» of outgoing call will be substituted with this value.

8.2.3 Files

Figure 156 – BTS LIU-A parameters, tab «Files»

The parameter value determines the sound file activity. If the file is on, it may be used in the pattern table.

1 – on;

0 – off.

# Sequence number of sound file. It assumes a value from 0 to 255.

# Filename

Path to the file relative to the directory, where media files are stored.

9 Emergency Notification System Setting

The Emergency Notification System (ENS) is intended to create emergency communication network based on the used operational communication network of VCCS to warn the airport rescue team on the prerequisite for flight accident.

9.1 Notification scheme setup

Emergency notification schemes are configured on «Emergency Notification» tab in «System parameters» window, Figure 157. «System parameters» window can be opened by selecting main menu items «System settings»\«General».

Notification scheme list

Notification scheme parameters оповещения»

Figure 157 – «Emergency Notification» setup page

Setup page contains the list of notification scheme sand the configuration panel for the selected scheme. The main list management buttons are:

– create and add new notification scheme to the list;

– completely remove selected scheme from system.

Scheme parameters

«Title» – scheme title.

«Signal duration, ms» – alarm signal playback duration. This alarm signal is played to a subscriber before voice message.

At the moment, «Beeper» voice event is used as an alarm signal (see 5.4 «Voice Events»). If the «Beeper» audio file is longer than this value, it’s playback will be interrupted. If the «Beeper» audio file is shorter than this value, it’s playback will be looped until «Signal duration» time runs out. If this value is set to «-1», then «Beeper» audio file will be played one time entirely from start to end.

Notification subscribers are selected in «Subscriber list». This list contains all VCDT, DCP and SIP-phones. External subscribers can be added manually.

– check all subscribers in list;

– clear all check marks.

– add an external subscriber to the notification list;

– remove an external subscriber from the notification list.

To add external subscriber to the list, click «Add» button and enter its distinctive title and an address in the relevant edit fields. Subscriber’s address has a SIP URI pattern.

To edit external subscriber’s data, select it from subscribers list and change the edit fields. Changes are saved immediately.

Figure 158  «Emergency Notification» setup page, external subscriber edit

Additional subscriber parameters

«Call confirmation timeout, in minutes» – if the call is not answered or confirmed by a subscriber in the specified time, it will be cancelled by the originating side.

Value is set minutes. Default value is 0, i.e. no timeout and the calls are never cancelled automatically.

10 Monitoring System

The monitoring system (of equipment condition) is a set of procedures, which allows to get information on the current technical condition of the equipment (measurements of object status, including their switching on/off, change of configuration) based on the results of measurements of selected parameters and monitoring of equipment operation.

Graphical display 
 of system objects

Information line

System messages

Figure 159  Monitoring system, system messages

The monitoring system status indicator is located on the toolbar. Possible state are:

# or

Clicking on the indicator will open an information window showing the current  status of the database:

Figure 160  Monitoring system, database status

Monitoring system information is displayed in the form of system messages and graphical indication of object status.

# The information line displays the last current message from the system message window.

All monitoring system messages are divided into the following types:

| Failure | Major system disturbance |
| --- | --- |
| Unknown status | Also considered an emergency situation |
| Pre-emergency status | The monitoring object may pass to emergency condition |
| Warning | Events, after which VCCS may be operated without limitations |

There are the following types of information:

|  | Monitoring object is in the normal operating condition |
| --- | --- |
|  | Debugging messages |
|  | Other information messages |

An event record (system message) contains the following information (see Figure 159):

–	message type;

–	time;

–	device type;

–	device;

–	monitoring object;

–	message content.

Figure 161  Monitoring system, graphical indication of object status

The system object status is displayed on its icon in the form of respective indicator: normal status, failure, unknown status, pre-failure status, warning and information (description is given above). Detailed information on the status of selected system object elements may be viewed in window «Information», see Figure 161.

During operation, visibility of object indicators may be controlled, see item 10.6 «Monitoring parameters» of this Manual.

In case the status of Controller Module is unknown, all indicators and other replaceable elements from the object are hidden until CM status is normal again, see Figure 162.

Figure 162  Monitoring system, unknown CM status

10.1 Monitoring

Monitoring is intended to view system messages. The tab consists of panel «Filter», panel «Message» and List of events. Buttons «Refresh» and «Filtering» are located in the upper part of the tab, see Figure 163.

– refresh the current list of messages;

– hide/show panel «Filter».

List of events

Panel «Message»

Panel «Filter»

Figure 163  Tab «Monitoring»

The list of messages may be filtered by such parameters as time, type of message, type of device and specific device. Filtering parameters are set on «Filter» panel, see Figure 164.

– filtering of messages by selected criteria;

– reset filter parameters.

Figure 164  Tab «Monitoring», filtering messages by selected criteria

Information of the event, selected from the list, is displayed on panel «Message», see Figure 164.

– printing out of information of the event, selected from the list.

When USB flash drive appears in the system, button «Export» appears on tab «Monitoring» next to button «Filtering», see Figure 165.

– export of Monitoring in .csv format to the selected USB flash drive.

Figure 165  Tab «Monitoring», button «Export»

# In case USB flash drive is connected prior to TMCT start, the device is not detected!

To export the Monitoring log, press button «Export» and select the required USB flash drive.

Figure 166  Tab «Monitoring», selecting USB flash drive

In case the log is exported successfully, a respective information message is displayed, the file is exported in .csv format, see Figure 167.

Figure 167  Monitoring, export completed

10.2 Setting of Sound Event Alerting

Specific sound alerting can be assigned to each type of monitoring system messages.

Figure 168  Monitoring system, alerting

– all sound alerts are switched on.

– all sound alerts are off.

.wav files shall be located in folder \Sounds.

| Message type | File name |
| --- | --- |
| - Failure | error.wav |
| - Unknown status | emergency.wav |
| - Pre-emergency status | alarm.wav |
| - Warning | warning.wav |

10.3 VCDT Statistics

Tab «Statistics» is intended to view information on workstation and communication channel loading.

– refresh current list of messages;

– delete all statistics permanently.

Tab «Statistics» consists of panel «Filter», information panel «Load by channel» and List of messages.

List of messages

Information panel

«Load by channel»

Panel «Filter»

Figure 169  Tab «Statistics»

To view required information, select the required filters on panel «Filter»: time, VCDT name (one or several), type of call (one or several).

Type of call:

|  | Transmission via radio |
| --- | --- |
|  | Reception via radio |
|  | Outgoing |
|  | Outgoing indirect |
|  | Incoming |
|  | Incoming indirect |

– filtering of messages by selected criteria;

– reset filter parameters.

When USB flash drive appears in the system, button «Export» appears on tab «Statistics» next to button «Clear stats», see Figure 170.

– export of VCDT statistics in .csv format to the selected USB flash drive.

# In case USB flash drive is connected prior to TMCT start, the device is not detected!

# To export the Statistics, press button «Export» and select the USB flash drive.

Figure 170  Tab «Statistics», button «Export»

In case the log is exported successfully, a respective information message is displayed, the file is exported in .csv format.

10.4 Logins

# Tab «Logins» is intended to view authorized IME users sessions.

– refresh the current list of messages.

Tab «Logins» consists of panel «Filter» and List of authorized users. If necessary, select filter by time.

– filtering of authorized users by selected criteria.

– reset filter parameters.

Panel «Filter»

List of authorizationsof users

Figure 171  Tab «Logins»

Authorization status:

|  | Beginning of user session |
| --- | --- |
|  | Completion of user session |
|  | Attempt of authorization with wrong data |

Figure 172  Tab «Logins», button «Export»

When USB flash drive appears in the system, button «Export» appears on tab «Logins» next to button «Refresh».

– export of logins in .csv format to the selected USB flash drive.

# In case USB flash drive is connected prior to TMCT start, the device is not detected!

To export the logins, press button «Export» and select the required USB flash drive.

In case the logins is exported successfully, a respective information message is displayed, the file is exported in .csv format.

10.5 Emergency notification monitoring

«Alarm Message» page of the main interface is used to monitor emergency notification statuses system wide.

– refresh data.

– apply filter panel settings.

– reset filter panel to default values and refresh data.

«Alarm Message» page consists of the Filter panel and the notifications list, see Figure 173. For the notification schemes setup see chapter 9.1 of this manual.

Filter panel

Selected notification scheme subscribers’ statuses

Activated notification schemes list

Figure 173  «Alarm message» page

Each of the activated notification schemes contains the following information:

– scheme title;

– notification start time;

– notification initiator (a VCDT title usually).

When selecting a scheme from the list, its subscribers are displayed on the right panel, see Figure 173. Each table entry contains the following information:

– subscriber’s name;

– notification start time;

– notification receive time (when a subscriber actually received the notification call);

– notification confirm time (when a subscriber has confirmed that the notification voice message was listened to).

In addition, the notification statuses are color-coded:

| VCDT 90 | Grey – notification created, waiting for subscriber’s answer |
| --- | --- |
| VCDT 90 | Red – notification was received by subscriber and notification voice message is being listened to |
| VCDT 90 | Green – subscriber has confirmed the success of the notification |

10.6 Monitoring parameters

Visibility of some system objects indicators may be controlled using menu item «Monitoring parameters». The context menu is opened with right mouse button.

Figure 174  Monitoring parameters

When menu item «Monitoring parameters» is selected, window «Monitoring configuration of system object» with tab «Indicators» will be opened, see Figure 175. Name and current value of indicators are displayed on the tab.

Let's take system object «VCDT» as an example.

Figure 175  Parameters of system object VCDT monitoring

Leave the names of only those indicators, which shall be displayed in window «Information», in the list. All object indicators are displayed in window «Information» by default.

Figure 176  System object VCDT, selecting indicators to be displayed in window «Information»

Figure 177  Displaying selected indicators in window «Information»

11 Remote VCDT Control

IME allows to carry out remote restart of VCDT components. Open the context menu with right mouse button and select item «Reboot».

Figure 178  Remote VCDT restart

# Menu item «Graphical user interface»

A command to reboot the graphic user interface is issued to VCDT. Depending on the terminal settings, the operator makes the final decision on restart.

# Menu item «System service»

Button of VCDT server restart (completing server operation). In the standard mode, the server will be restarted automatically by the operating system.

# Menu item «Main unit»

# VCDT operating system restart attempt.

# Menu item «Change ip address»

The command allows to change ip address of certain network interface of controller module and reboot the operating system after that.

When this menu item is selected, indicate specify new IP address in the opened window:

Figure 179  Rebooting VCDT with IP address change

Menu item «Shutdown main unit»

Issuing of command to switch off VCDT.

11.1 Updating Training Configuration

An important aspect of Training operation is quick change of configuration on WS group. Open the context menu with right mouse button and select item «Update configuration (Training)».

# Figure 180  Updating (Training) configuration

WS configurations are updated in window «VCDT Training configuration». For operation in this window see item 8.1 «VCDT Training configuration» of this Manual.

# Figure 181  Window «VCDT Training configuration»

11.2 Updating Configuration

Necessity to update VCDT configuration is displayed by blinking indicator on VCDT icon. Open the context menu with right mouse button and select item «Update configuration».

# Figure 182  Updating configuration

Detailed information on required updating is displayed in window «Information»:

# Figure 183  Window «Information»

11.3 Calls Transferring

The function is required to transfer all regular ground communication incoming calls from the terminal to other address: VCDT or other network subscriber, supporting transferring.

IME allows to carry out transferring remotely. To enable the function, select menu item «Set LSC transfer», and then the required VCDT from the displayed list.

# Figure 184  Enabling transferring function

In the opened window confirm the address of VCDT, selected from the list, by pressing button «OK»:

# Figure 185  Confirming the address for transferring

If there is no required VCDT in the list, select menu item «Other address» and enter an address for transferring.

# Figure 186  Setting transferring

Transferring activation is indicated visually as a green arrow on the system object icon.

A line in the system object information window also indicates transferring activation:

# Figure 187  System object information window: transferring is on

Transferring is switched off by selecting menu item «Cancel transfer».

# Figure 188  Switching off transferring

12 Object Maintenance

In case of maintenance and/or remedial works of VCCS «Topaz», the system object maintenance may be initiated.

For this purpose select the required system object, open the context menu with right mouse button and select item «Set on maintenance», see Figure 189.

# Figure 189  Initiation of object maintenance

After initiation of maintenance, the object icon becomes light grey, see Figure 190, while the object is disconnected from the monitoring system.

# Figure 190  Object removal form maintenance

To cancel the maintenance mode, select context menu item «Deactivate maintenance mode».

13 DB Backup and Restore

# Figure 191   DB backup

To back DB data, proceed as follows:

– select menu item File Database tools Backup;

– in the opened window, press button «Start», see Figure 192.

Upon completion of data backup, a backup copy file can be moved freely (to another computer or CD).

# Figure 192  DB backup

To recover data from backup copies, proceed as follows:

– select menu item File Database tools Restore);

–select the backup file in the opened window;

–click «Restore database from selected copy» button, see Figure 193.

# Figure 193  DB restore from backup copy

13.1 Emergency recovery of a corrupted database

When a database file becomes corrupted for whatever reason (Database Server power failure, storage media physical damage, software errors, etc.), it is necessary to perform the procedure of the emergency recovery.

The recovery procedure requires exclusive access to Database Server and any connection to the database from other clients are prohibited. To achieve this, the following command must be performed:

gfix -shut –force 0 -user SYSDBA -password <your pass> \Path:\AERO.FDB

After that, to make sure that the database file is actually corrupted, the command Command 1 must be performed. Gfix, used in this command, is a tool designed to fix corrupted database, it is included in the each Firebird distribution. Gfix scans the database, checks data structures and records for corruptions and fixes them.

If, as a result of executing the above command, a message is received that the database file is corrupt and the required operation is impossible, then this is a critical case and you need to turn to the help of paid utilities to restore the damaged database.

For example, IBSurgeonFirstAid utility (http://ib-aid.com). IB Surge on First AID is the tool that can automatically diagnose and recover corrupted Firebird databases, it can repair corruptions that cannot be repaired using standard utilities. It uses its own layer for low-level database access without using the Firebird engine. It is highly recommended to stop the Firebird Server process while using this tool. In addition, after you have finished using the tool, it is recommended to close it and then start Firebird Server.

If, as a result of executing this utility (or any other special tools), it was possible to fix the corrupted database file, then after that perform the standard database regeneration procedure:

- execute Command 1;

- execute Command 2;

- execute Command 3;

- restart Firebird Server or the whole operating system;

- execute Command 4.

If these 5 steps are successfully completed, then the emergency regeneration of the database can be considered successfully completed, and the resultant database is able to continue to perform its functions in the future.

If, after executing Command 1, a list of integrity control errors is issued, then it can be assumed that the fixed errors can be eliminated in the standard way. Then follow these steps:

- execute Command 2;

- execute Command 3;

- restart Firebird Server or the whole operating system;

- execute Command 4.

Command 1. The command to control the integrity and fix damage of the database
gfix -v -full -user SYSDBA -password <your pass> \Path\AERO.FDB

Command 2. The preparation command before saving the damaged database
gfix -mend -user SYSDBA -password <your pass> \Path\AERO.FDB

Command 3. The command to save the archive (backup) of the database
gbak -b -g -v -user SYSDBA -password <your pass> \Path\AERO.FDB \Path\backup.fbk

Command 4. The command to restore the database from the archive (backup)
gbak -c -p 16384 -v -user SYSDBA –password <your pass> \Path\backup.fbk \Path\AERO.FDB

After performing the required procedure, the following command must be executed to restore other clients’ access to the database:

gfix -online -user SYSDBA -password <your pass> \Path:\AERO.FDB

13.1.1 Transaction limit exceeding

In situations when the transaction counter ends, the following message appears in the activity log files of the collector:

Implementation limit exceeded. Transactions count exceeded.

The same message is issued when you try to execute a command backup (gbak -b -v -g -user SYSDBA -pas <your pas> AERO.FDB AERO.fbk) in the following form:

gbak:readied database AERO.FDB for backup

gbak:creating file AERO.fbk

gbak:starting transaction

gbak: ERROR:invalid transaction handle (expecting explicit transaction start)

gbak:Exiting before completion due to errors

Before creating backup-а, it is necessary to take off a DB into a mode «only for reading»:

gfix -mode read_only -user SYSDBA -password <your pass> \Path\AERO.FDB

Next, you need to create backup:

gbak -b -g -v -user SYSDBA -password <your pass> \Path\AERO.FDB  \Path\backup.fbk

After backup-а creation, DB needs its recovery:

gbak -c -p 16384 -v -user SYSDBA -password <your pass> \Path\backup.fbk \Path\AERO.FDB

After carrying out the required procedures, execute the command for DB taking off into a «read/write» mode:

gfix -mode read_write -user SYSDBA -password <your pass> \Path\AERO.FDB

14 User Change

Three access levels are used to work in IME: Administrator, Engineer, and Technician (for users setting see item 3.4 «Users setting» of this Manual).

By default, IME is loaded with access level «TECHNICIAN», the only available menu item is «Change user».

# Figure 194  User change

To change the user, select menu item FileChange user. In the authorization window enter user name and password to enter the system. The password is shown on the screen in the hidden mode as symbol «*».

# Figure 195  User change, authorization

In this example, authorization is carried out with access level «Engineer». In case of successful authorization, TMCT is loaded with functions, available for authorized user.

Figure 196  Successful authorization, user «Engineer»

15 Power Supply Control

During operation, the necessity may arise to reboot the system or TMCT interface. Menu item «Power» is intended for this purpose, see Figure 197.

# Figure 197  Menu item «Power»

Menu item «Restart GUI» – a command to reboot graphical user interface is issued to IME. After rebooting IME is loaded with access level «Technician».

Menu item «Exit from Graphical User Interface» – completion of TMCT graphical user interface operation and quitting to operating system.

Menu item «System restart» – controller module hardware reboot.

Menu item «System shutdown» – hardware completion of controller module operation.

Appendix A

(reference)

Setting of Printer Epson LX-350

Switch on the printer and insert a sheet of paper. To enter the menu, when the printer is switched on press two buttons «LF/FF» and «Load/Eject» at the same time, see Figure A.1.

# Figure A.1  Location of buttons and indicators on printer Epson LX-350

When the buttons are pressed, the printer will print two lines with current menu language settings. By default, English is selected as menu language. Do not change this setting! Thus, press button «Set» only.

After that the printer will suggest to print out the current settings. Press button «Set» again, after that a sheet with settings will be printed out, see Figure A.2.

# Figure A.2  Current settings of printer Epson LX-350

In case all settings do not fit one sheet, insert a second sheet. At the bottom of the printed sheet there is an instruction how to change the settings:

- to select menu items use button «Item»,«Item»;

- to select values use button «Set»;

- to cancel changes switch off the printer;

- to save the changes, press buttons «LF/FF» and «Load/Eject» at the same time.

We shall change parameter Character table.

To change the parameter, enter the menu again by pressing buttons «LF/FF» and «Load/Eject» at the same time, then press button «Set» once, confirming selection of English.

Do not press button «Set» for the second time as an answer to the request to print out the settings. Press button «Item»instead several times until «Character table» is printed out. After that press button «Set» to search through all values of this parameter until the printer prints out value PC 1251, see Figure A.3.

To save the selected value, press buttons «LF/FF» and «Load/Eject» at the same time.

# Figure A.3  New settings of printer Epson LX-350

Setting of printer Epson LX-350 is completed.

Appendix B

(informative)

Gain Parameters

Gain parameters values are from 0 to 255. Dependence between gain parameter value and actual values is non-linear. Dependence between the parameter and actual gain value can be described using Table 1.

Table 1

| Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  | Dependence of dB from GAIN |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GAIN | dB | GAIN | dB | GAIN | dB | GAIN | dB | GAIN | dB | GAIN | dB | GAIN | dB | GAIN | dB |
| 0 | –45.0 | 32 | –14.1 | 64 | –7.7 | 96 | –3.5 | 128 | 0.0 | 160 | 3.5 | 192 | 7.7 | 224 | 14.1 |
| 1 | –38.8 | 33 | –13.8 | 65 | –7.6 | 97 | –3.4 | 129 | 0.1 | 161 | 3.7 | 193 | 7.9 | 225 | 14.3 |
| 2 | –35.2 | 34 | –13.5 | 66 | –7.4 | 98 | –3.3 | 130 | 0.2 | 162 | 3.8 | 194 | 8.0 | 226 | 14.6 |
| 3 | –32.7 | 35 | –13.3 | 67 | –7.3 | 99 | –3.2 | 131 | 0.3 | 163 | 3.9 | 195 | 8.2 | 227 | 14.9 |
| 4 | –30.7 | 36 | –13.0 | 68 | –7.1 | 100 | –3.1 | 132 | 0.4 | 164 | 4.0 | 196 | 8.3 | 228 | 15.2 |
| 5 | –29.1 | 37 | –12.8 | 69 | –7.0 | 101 | –3.0 | 133 | 0.5 | 165 | 4.1 | 197 | 8.5 | 229 | 15.5 |
| 6 | –27.8 | 38 | –12.5 | 70 | –6.8 | 102 | –2.8 | 134 | 0.6 | 166 | 4.2 | 198 | 8.7 | 230 | 15.9 |
| 7 | –26.6 | 39 | –12.3 | 71 | –6.7 | 103 | –2.7 | 135 | 0.8 | 167 | 4.4 | 199 | 8.8 | 231 | 16.2 |
| 8 | –25.6 | 40 | –12.1 | 72 | –6.6 | 104 | –2.6 | 136 | 0.9 | 168 | 4.5 | 200 | 9.0 | 232 | 16.6 |
| 9 | –24.7 | 41 | –11.9 | 73 | –6.4 | 105 | –2.5 | 137 | 1.0 | 169 | 4.6 | 201 | 9.2 | 233 | 16.9 |
| 10 | –23.8 | 42 | –11.6 | 74 | –6.3 | 106 | –2.4 | 138 | 1.1 | 170 | 4.7 | 202 | 9.3 | 234 | 17.3 |
| 11 | –23.1 | 43 | –11.4 | 75 | –6.1 | 107 | –2.3 | 139 | 1.2 | 171 | 4.9 | 203 | 9.5 | 235 | 17.7 |
| 12 | –22.4 | 44 | –11.2 | 76 | –6.0 | 108 | –2.2 | 140 | 1.3 | 172 | 5.0 | 204 | 9.7 | 236 | 18.1 |
| 13 | –21.7 | 45 | –11.0 | 77 | –5.9 | 109 | –2.1 | 141 | 1.4 | 173 | 5.1 | 205 | 9.9 | 237 | 18.5 |
| 14 | –21.1 | 46 | –10.8 | 78 | –5.7 | 110 | –2.0 | 142 | 1.5 | 174 | 5.2 | 206 | 10.0 | 238 | 19.0 |
| 15 | –20.5 | 47 | –10.6 | 79 | –5.6 | 111 | –1.8 | 143 | 1.6 | 175 | 5.4 | 207 | 10.2 | 239 | 19.5 |
| 16 | –20.0 | 48 | –10.4 | 80 | –5.5 | 112 | –1.7 | 144 | 1.7 | 176 | 5.5 | 208 | 10.4 | 240 | 20.0 |
| 17 | –19.5 | 49 | –10.2 | 81 | –5.4 | 113 | –1.6 | 145 | 1.8 | 177 | 5.6 | 209 | 10.6 | 241 | 20.5 |
| 18 | –19.0 | 50 | –10.0 | 82 | –5.2 | 114 | –1.5 | 146 | 2.0 | 178 | 5.7 | 210 | 10.8 | 242 | 21.1 |
| 19 | –18.5 | 51 | –9.9 | 83 | –5.1 | 115 | –1.4 | 147 | 2.1 | 179 | 5.9 | 211 | 11.0 | 243 | 21.7 |
| 20 | –18.1 | 52 | –9.7 | 84 | –5.0 | 116 | –1.3 | 148 | 2.2 | 180 | 6.0 | 212 | 11.2 | 244 | 22.4 |
| 21 | –17.7 | 53 | –9.5 | 85 | –4.9 | 117 | –1.2 | 149 | 2.3 | 181 | 6.1 | 213 | 11.4 | 245 | 23.1 |
| 22 | –17.3 | 54 | –9.3 | 86 | –4.7 | 118 | –1.1 | 150 | 2.4 | 182 | 6.3 | 214 | 11.6 | 246 | 23.8 |
| 23 | –16.9 | 55 | –9.2 | 87 | –4.6 | 119 | –1.0 | 151 | 2.5 | 183 | 6.4 | 215 | 11.9 | 247 | 24.7 |
| 24 | -16.6 | 56 | –9.0 | 88 | –4.5 | 120 | –0.9 | 152 | 2.6 | 184 | 6.6 | 216 | 12.1 | 248 | 25.6 |
| 25 | –16.2 | 57 | –8.8 | 89 | –4.4 | 121 | –0.8 | 153 | 2.7 | 185 | 6.7 | 217 | 12.3 | 249 | 26.6 |
| 26 | –15.9 | 58 | –8.7 | 90 | –4.2 | 122 | –0.6 | 154 | 2.8 | 186 | 6.8 | 218 | 12.5 | 250 | 27.8 |
| 27 | –15.5 | 59 | –8.5 | 91 | –4.1 | 123 | –0.5 | 155 | 3.0 | 187 | 7.0 | 219 | 12.8 | 251 | 29.1 |
| 28 | –15.2 | 60 | –8.3 | 92 | –4.0 | 124 | –0.4 | 156 | 3.1 | 188 | 7.1 | 220 | 13.0 | 252 | 30.7 |
| 29 | –14.9 | 61 | –8.2 | 93 | –3.9 | 125 | –0.3 | 157 | 3.2 | 189 | 7.3 | 221 | 13.3 | 253 | 32.7 |
| 30 | –14.6 | 62 | –8.0 | 94 | –3.8 | 126 | –0.2 | 158 | 3.3 | 190 | 7.4 | 222 | 13.5 | 254 | 35.2 |
| 31 | –14.3 | 63 | –7.9 | 95 | –3.7 | 127 | –0.1 | 159 | 3.4 | 191 | 7.6 | 223 | 13.8 | 255 | 38.8 |

Revision Record Sheet

| Rev. | Sheet (page) numbers |  |  |  | Total sheets (pages) in document | Doc. No. | Registration number of accompanying document and date | Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | revised | replaced | new | cancelled |  |  |  |  |  |